3GPP TSG SA WG 1 Meeting #112 	S1-254249r2

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	Siemens

pCR Title:	Use Case on Cooperating Mobile Robots – Update

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.8.1

Document for:	Approval

Contact:	Michael Bahr ( bahr et siemens dod com )

Abstract: Update of Use Case on Cooperating Mobile Robots for 3GPP SA1 TR 22.870 Clause 11 Further Use Cases on Industry and Verticals. KPIs with square brackets / FFS have been resolved.

1. Introduction

There have been several KPIs in Tables 11.3.6-1 and 11.3.6-2 with square brackets. These have been resolved.

2. Reason for Change

- Resolution of square brackets around several KPI values in Tables 11.3.6-1 and 11.3.6-2.

- Three broken references (internal to Clause 11.3) have been corrected.

- Several improvements for readability.

3. Conclusions

Square brackets of several KPIs in Tables 11.3.6-1 and 11.3.6-2 have been resolved.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * First Change * * * *

### 11.3.6	Potential New Requirements needed to support the use case

[PR 11.3.6-1] The 6G System shall be able to support the performance requirements in Table 11.3.6-1 for Cooperating Mobile Robots.

Table 11.3.6-1: Performance Requirements for Cooperating Mobile Robots use case [112]

| Use Cases | Max allowed end-to-end latency (ms) | Application data rate (Mb/s) | Reliability (%) | Message size (byte) | Service area (m²) | Transfer interval (ms) | Connection density [devices/m2] | UE speed (km/h) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cooperating Mobile Robots – 6G Network Services | [1 to 10] | [<10] [<250] (note 2) | [99.999 to 99.999 99] (note 4) | (note 3) | from [20 m2 (indoor) up to 1 km2 (outdoor)] (note 5) | [≤ 10] | [≤ 0.5] (note 1) | [< 20] | Robot to Network for services such as digital twin applications, AI/ML compute servers. |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: The higher data rate applies when e.g. immersive XR, AI/ML traffic, digital twin data, is exchanged. Otherwise, the lower data rate applies. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, control data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real time industrial processes requires very high service reliability. Can be lower for non-real time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario with cooperating mobile robots and on size of location/collaboration area for the scenario. Examples for service areas are the working area of collaborative carrying mobile robots (see 11.3.3.1), a factory floor or a workshop, a construction site deploying autonomous robots that work in unison to construct a building (see 11.3.3.2), or a farmer’s field in autonomous farming with self-driving, smart machines that work collaboratively in agricultural operations. |  |  |  |  |  |  |  |  |  |  |

[PR 11.3.6-2] The 6G System shall be able to support the positioning performance requirements in Table 11.3.6-2 for Cooperating Mobile Robots.

Table 11.3.6-2: Positioning Performance Requirements for Cooperating Mobile Robots use case [112]

| Use Cases | Positioning accuracy (m) | Velocity (km/h) | Remarks |
| --- | --- | --- | --- |
| Cooperating Mobile Robots | [<0.1] | slow (note) | Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. |
| Cooperating Mobile Robots – Robot localization | 1 | [< 20] | Tasks like robot localization can work with less accurate positioning. |
| Human presence detection | [0.4] | Pedestrian | Identification of human presence in areas where people are around: [40 cm] resolution within an area defined by [1 m] range around the robot. [112] |
| NOTE: Velocity of robots is usually slow and adapted to the localization performance in such situations. |  |  |  |