3GPP TSG-SA WG1 Meeting #112 	S1-254xxx

17-21 November 2025, Dallas, USA	(revision of S1-254015)

Title:	Rapporteur proposed initial consolidated PRs (Clause Y.1.11)

Agenda Item:	TBD

Source:	6G Study Rapporteurs

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Initial allocation of PRs into Clause Y.1.11

# Y	Consolidated potential functional and performance requirements

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

## Y.1	Consolidated potential Functional Requirements

### Y.1.11	Ubiquitous Connectivity

Editor’s Note: This clause contains consolidated potential requirements for 6G that include (but not limited to):s upport for non-terrestrial networks (NTN) and support for positioning services and capabilities. This includes (but not limited to) PRs from clause 8.

Editor’s NOTE: How to consolidate computing service PRs (i.e. [PR 8.9.6-1], [PR 8.15.6-2]) in the context of ubiquitous communication need to consider, e.g. whether/how compelling/relevant is spaceborne SHE.

Editor’s NOTE: For Ubiquitous, more input will be provided regarding CPRs as well as KPI values.

NEW Editor’s Note: PR 8.2.6-1 and PR 8.17.6-1 have Editor’s Notes associated with them, so they are not included for consolidation into Table Y.1.11-1 at this time.

NEW Editor’s Note: PRs that only state a requirement to meet KPIs in a “Table” will be revised to point to the appropriate consolidated tables in Y.2.

Table Y.1.11-1 – NTN /satellite accesscommunication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-1-1 | The 6G system with satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation and optimize the coverage per LEO satellite. | PR 8.3.6-1 PR 8.3.6-3 | LEO Coverage Flex configs |
| Y.1.11-1-2 | The 6G system with satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios. | PR 8.3.6-2 | LEO Service continuity |
| Y.1.11-1-3 | The 6G system with satellite access shall support service continuity within NTN, and between TN and NTN, minimising the interruption to the UE during the transition | PR 8.4.6-1 PR 8.8.6-1 | Service continuity TN-NTN |
| Y.1.11-1-4 | The 6G system with satellite access, shall support SMS delivery to a high density of UEs i.e. up to [1000] UE per km2. | PR 8.4.6-2 | KPI (Y.2) high density # of UEs SMS |
| Y.1.11-1-5 | Subject to regulatory requirements, the 6G system with satellite access, shall support text messaging services and PWS [62] for UEs in adverse propagation conditions e.g. light indoor conditions, dense forest. | PR 8.4.6-3 PR 8.3.6-3 | PWS Text Messaging Adverse propagation conditions |
| Y.1.11-1-6 | The 6G system with satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted). | PR 8.6.6-1 | eMBB |
| Y.1.11-1-7 | The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) with variant altitudes (e.g. from 0 to 3 km [99]) with satellite access and/or terrestrial access. | PR 8.9.6-2 | Communication service UEs at variant altitudes |
| Y.1.11-1-8 | The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network. | PR 8.13.6-1 | UE-UE direct communications |
| Y.1.11-1-9 | The 6G system with satellite backhaul links between a 6G base station on board of UAV and the 6G CN, shall support the selection and switching between satellite links, each having different characteristics, based on e.g. traffic load, quality of the link, satellite availability. NOTE: The following terms 6G base station/CN do not imply any architectural assumption, e.g. whether 6G CN/base station is a new or evolved CN/base station (compared to 5G). | PR 8.12.6-1 | Satellite backhaul links selection/switching |
| Y.1.11-1-10 | The 6G system with satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS). | PR 8.14.6-1 | Time synchronization |
| Y.1.11-1-13 | The 6G system using satellite access shall support mechanisms to improve communication QoS and capacity under constrained satellite link conditions, e.g. using signalling optimization, codec rate adaptation. | PR 8.18.6-1 | QoS Improvement |
| Y.1.11-1-14 | The 6G system with satellite access shall be able to provide eMBB service with KPIs from Table 8.6.6-1: KPIs of the 6G satellite access network to support the use case disaster relief [112]. | PR 8.6.6-2 | KPI eMBB |
| Y.1.11-1-15 | The KPIs in Table 8.8.6-1: KPIs for global mobile video should be met for global mobile video service. | PR 8.8.6-2 | KPI Global mobile video service |

Table Y.1.11-2 – NTN-based Positioning

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-2-1 | Subject to regulatory requirements, operator policy, and user consent, the 6G system with satellite access shall be able to determine and provide the UE location meeting the regulatory requirements for both sparse and dense satellite constellations. | PR 8.3.6-4 PR 8.5.6-2 | UE Location Regulatory Support |
| Y.1.11-2-2 | The 6G system with satellite access shall be able to provide a mechanism for energy constrained devices to determine its position using 3GPP positioning technology. | PR 8.7.6-1 | Energy constrained devices |
| Y.1.11-2-3 | The 6G system with satellite access shall be able to provide a positioning service supported by the combination of NTN and GNSS positioning technologies, including Network-based Assisted GNSS. | PR 8.11.6-1 | Support of NTN & GNSS positioning technologies |
| Y.1.11-2-4 | The 6G system with terrestrial and satellite access shall be able to provide integrity for a positioning service. | PR 8.16.6-1 | Positioning integrity |
| Y.1.11-2-5 | The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the KPIs in Table 8.5.6-1: Performance requirements for satellite-based positioning services | PR 8.5.6-1 | KPI |
| Y.1.11-2-6 | The 6G system with satellite access shall be able to provide positioning service following the KPIs in Table 8.7.6-1: Performance requirements for satellite-based positioning services for energy constrained devices from [338] | PR 8.7.6-2 | KPI Energy constrained devices |
| Y.1.11-2-7 | The 6G system with terrestrial and satellite access shall be able to provide positioning service for UE type e.g. Airplane Mounted and UAV Mounted with high availability, by using 3GPP positioning technologies and fulfilling the KPIs in Table 8.10.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies. | PR 8.10.6-1 | KPI |
| Y.1.11-2-8 | The 6G system with satellite access shall be able to provide positioning service with the KPIs from Table 8.11.6-1: Performance requirements for positioning services with satellite access | PR 8.11.6-2 | KPI |

Table Y.1.11-3 – Other aspects related to ubiquitous communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.11-3-1 | Subject to operator’s policy, the 6G network with satellite access may support onboard Service Hosting Environment(s) and mechanisms to - Minimise the necessary bandwidth of the inter-satellite and feeder links, - Modify the path for routing data traffic between a UE and the Service Hosting Environment to minimize service interruption considering the movement of UE and/or satellite, - provide a computing service via a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities, and - subject to user consent, support data sharing among multiple UEs (e.g. UAV). NOTE: The shared data is the processed results based on non-3GPP sensing data from the UEs (e.g. UAV) provided by Service Hosting Environment on aboard satellite. This implies some need to transfer computing information in/related to Service Hosting Environment (e.g. pre-processed data within the service hosting environment) between satellites over a given area. | PR 8.15.6-1 PR 8.9.6-3 PR 8.9.6-4 PR 8.15.6-2 | Onboard Service Hosting Environment Modification of data routing Computing service UE Data Sharing Non-3GPP sensing and transfer of computing service related info related to SHE |
| Y.1.11-3-2 | Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access may be able to provide to logistic companies computing service in a given area via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | PR 8.9.6-1 |  |
|  | [PR 8.9.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G system with satellite access shall be able to provide a computing service via a suitable Service Hosting Environment on board satellite to a UE (e.g. UAV) using only satellite access e.g. considering the latency and satellite capabilities. | [PR 8.9.6-1] |  |