3GPP TSG-SA WG1 Meeting #64	S1-135221

San Francisco, USA, 11-15 November 2013

Title:		eICBD use case: UE requests a Public IP Address from its MNO

Agenda Item:	FS_eICBD

Source:		Intel

Document for:	Approval

Contact:	Kerstin Johnsson [kerstin.johnsson@intel.com]

# Introduction

# This contribution details a use case where a UE asks its MNO to provide it with a public IP address.

# Proposal

It is proposed to add the following to TR 22.8xy:

x.1	UE requests a Public IP Address from its MNO

x.1.1	Description

Bill and Dave are students at USC.  USC maintains a P2P server that manages a database of media content that students want to share with each other.

Dave took videos of the Homecoming football game using his UE and wants to share them.  However, Dave’s UE does not have a public IP address, so he has his UE request one from his MNO.  Once Dave’s UE gets a public IP address, Dave logs into USC’s P2P server and updates his profile to advertise his Homecoming game videos and publish his public IP address along with them.

Later, Bill logs into the server, finds Dave’s videos, chooses one to watch, and clicks to download it.

x.1.2	Pre-conditions

- Bill and Dave are subscribers of LTE services deployed by different MNOs and are associated with different LTE base stations.

- Bill and Dave are registered with USC’s P2P server.

- Dave has Homecoming football game videos on his UE that he wants to advertise on USC’s P2P server.

- Dave’s MNO enables his UE to request and receive a  public IP address.

x.1.3	Service Flows

- Dave clicks to have his UE request a  public IP address from his MNO.

- Dave’s MNO provides his UE with a  public IP address.

- Dave logs into USC’s P2P server and updates his profile to include information about the Homecoming football game videos he wants to share along with his UE’s newly acquired public IP address.

- Bill logs into USC’s P2P server, finds Dave’s video, and clicks to download it.

- Bill’s device sends an IP connection request to Dave’s UE using the public IP address provided in Dave’s profile, establishes a P2P session, and starts to download the video from Dave’s UE.

x.1.4	Post-conditions

- Bill is downloading Dave’s video over an IP connection.

x.1.5	Potential Requirements

TBD

x.1.6	Potential Impacts or Interactions with Existing Services/Features

TBD