3GPP TSG-SA WG1 Meeting #67bis and #68	S1-144131

San Francisco, USA, 13-14 and 17-21 November 2014	(revision of S1-14xxxx)

Title:	Radio Resource Efficiency Requirements

Agenda Item:	7.1 MCPTT: Mission Critical Push To Talk over LTE

Source:	HUAWEI, TD-Tech

Contact:	Chen De (chende@huawei.com)

Abstract: It is proposed to ensure that MCPTT services can use as many radio resource as it needs in case of emergency, because currently it looks to be limited to 60% and the radio resource use between group and private calls looks not flexible if eMBMS is considered

Discussion

It looks necessary to allow the usage of a maximum of radio resources available in the network for MCPTT services in case of a critical event.

- Nevertheless, if 3GPP considers the use of current eMBMS bearers to support MCPTT services, as it is not possible to assign the entire radio resources for eMBMS: MCPTT service with eMBMS will be able to use a maximum of 60% of the radio resources in LTE FDD mode (this is due to a current MBSFN/MBMS restriction that only 6 subframes on the 10 of a radio frame could be configured for MBSFN for group communication) and a maximum of 50% of the radio resources in LTE TDD mode.

As a consequence, the number of MBMS voice sessions in parallel is limited.

Is this adapted to the MCPTT communications needs in the future?

A SA2 document (provided by UK Home Office in S2-131607) presents “UE and group numbers during a significant large planned event”.  This (SA2) table does not illustrate the number of groups communicating at the same time, but the number of groups that will cover the event. It could be up to 83 groups:

But in the future, different Public Safety organisms will probably dialog more between each others in order to share more information, due for example to all information available from more types of devices and from sensors everywhere.

When multimedia communication with higher bit rates will be used, communications will also need resources for video and/or pictures, so the number of active communication in parallel would further increase as well as the resource needs.

Also we consider that each officer on the floor will expect to gather a maximum of intelligence on the situation before starting any action and the 3GPP network should support this evolving need.

So it appears not unrealistic to consider that when up to 83 groups are in place, majority of them would need to share information at the same time in a near future. Network has long life period, the 3GPP network should be prepared for important number of groups communicating in parallel.

The capacity of VoIP over current eMBMS is lower than the potential future need for MCPTT groups.

It is proposed that SA1 clarifies that MCPTT group call service should be able to use more radio resources when needed, i.e. be not limited to 60 or 50% of radio resources.

- In some urgent public safety scenarios, for example in case of terrorist attack in a city (for example terrorist bomb event in Boston), a large number of MCPTT calls can take place and the number of MCPTT group calls compared to the number of MCPTT private  calls may vary frequently during the event, e.g.:

  - there could be 30% of group calls with 70% of private calls at certain time,

  - and then 80% of group calls with 20% of private calls at another time.

But currently, if 3GPP considers the use of eMBMS bearers to support MCPTT services, this ratio is pre-configured so has to be known in advance (to allow MBMS configuration of MBSFN areas),while this shall remain flexible to adapt to the MCPTT service needs during an urgent critical event.

It is proposed that SA1 clarifies needs on the ratio change between group and private calls.

Proposed changes

It is proposed that SA1 WG clarifies these MCPTT radio resource requirements.

For Stage1 specification, it is proposed to add a new section to clarify performance requirements:

------------------------------------First change ---------------------------------------

6.16.X	Radio Resource Efficiency Performance

The network shall be able to assign entire radio resource if needed for multicast communications to an important number of MCPTT UEs in the same cell (e.g. be able to use more than 60% of the available radio resources when it is needed by a MCPTT group communication to important number of UEs in the cell).

The network shall be able to allocate radio resource in a frequent and flexible manner between MCPTT group and private calls depending on the MCPTT service needs, e.g. the radio resource for group calls could be 30% when radio resources for private calls are 70%, or respectively 10% and 90% depending on MCPTT service need.

------------------------------------End of changes ----------------------------------