3GPP TSG-SA WG1 Meeting #70	S1-151104

Los Cabos, Mexico, 13 – 17 April 2015

Title:	Use of V2X in areas outside the network coverage

Agenda Item:	8.7 FS_V2XLTE

Source:	LG Electronics Inc.

Contact:	SungDuck Chun <sungduck.chun (at) lge.com>

Abstract: This paper proposes to capture the out-of-coverage V2X use case and the related potential requirements in Technical Report on LTE-based V2X.

- Introduction

Section 6.1 of TR on LTE-based V2X states that:

## 6.1 Consideration on coverage V2V includes applications supporting road safety, and whether part of the communication should be able to continue in areas outside network coverage should be considered.

Because assistance to drivers via infrastructures is rarely available in an area outside of network coverage, direct vehicle to vehicle communication would be beneficial and may be the only way for communication in areas outside of network coverage. In this contribution, one use case outside of network coverage is provided.

- Proposal

In this document, we propose to capture one out-of-coverage V2X use case and the related potential requirements in Technical Report on LTE-based V2X as described below.

* * * * Start of Addition * * * *

5.z	Use Case – V2X in areas outside network coverage

5.z.1	Description

This use case describes V2X communication when vehicles are located in an area not served by E-UTRAN.

Figure 5.z.1-1: Vehicles outside network coverage

5.z.2	Pre-conditions

Vehicle A and B are passenger vehicles equipped with V2X-enabled UEs.

Vehicle A and B are located in the area which is not served by E-UTRAN.

Vehicle A and B are pre-configured with parameters effective in the area when not served by E-UTRAN.

5.z.3	Service Flows

Vehicle A and Vehicle B start transmission of traffic safety-related messages through E-UTRA radio using the pre-configured parameters.

Both vehicles are getting close to each other and are within a direct communication range.

By detecting the signal transmitted by each other, Vehicle A and Vehicle B notice the existence of each other.

After having noticed other vehicle’s existence, each vehicle starts receiving each other’s traffic safety-related messages.

5.z.4	Post-conditions

Vehicle A gets aware of the location, and the moving direction and speed of Vehicle B.

Vehicle B gets aware of the location, and the moving direction and speed of Vehicle A.

5.z.5	Potential Requirements

A V2X-enabled UE shall be able to transmit and receive traffic safety-related messages using the E-UTRA radio when not served by E-UTRAN. .

A V2X-enabled UE shall be authorized to transmit traffic safety-related messages using E-UTRA radio when not served by E-UTRAN.

A V2X-enabled UE shall be authorized to receive traffic safety-related messages using E-UTRA radio when not served by E-UTRAN.

A V2X-enabled UE shall be able to be pre-configured with parameters to be used for the transmission and reception of the traffic safety-related messages using E-UTRA radio when not served by E-UTRAN.

When not served by E-UTRAN, a V2X-enabled UE shall be able to detect the existence of other V2X-enabled UE within [500] milliseconds at the relative speed of up to [400] km/h after being located within a range of [500] m from each other

When not served by E-UTRAN, the V2X-enabled UEs shall be to transmit and receive the traffic safety-related messages within a range of [500] m from each other, up to relative speed of [400] km/h, at a given packet error rate [5%].

* * * * End of Addition * * * *