3GPP TSG-SA WG1 Meeting #76Bis	S1-170179

# Spokane, US, 14-20 Jan, 2017 												(revision of S1-170016)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.886 | CR | 0002 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify use case associated to platooning size in section 7 2 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eV2X |  |  |  |  |  |  | Date: |  | 2016-01-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 2 different platooning sizes are given with no explanation of why there is a difference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | propose to consolidate the two requirements into one the platooning size in platooning use case.Platoon size is assumed from 5 to 20 vehicles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The platooning size requirement is not clear and will confuse the downstream GWs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

***************************** Start of Change  ***************************************

7.2.2	Requirements for Platooning

[CPR.P-00x]	The 3GPP system shall be able to support 5-20 UEs for a group of UEs supporting V2X application.

[CPR.P-002]	For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications.

[CPR.P-003]	The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

# Table 7.2.2-1 Performance requirements for Platooning

| Communication Scenario |  |  |  | Payload (Bytes) | Tx Rate (Message/ Sec) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Commu-nication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section # | Description |  | CPR # |  |  |  |  |  |  |
| 5.1 | Among a group of UEs (or two UEs) supporting V2X application |  | [CPR.P-004] | 50-1200 (NOTE 1) | 30 | 10 (NOTE 2) |  |  |  |
|  |  |  | [CPR.P-005] | 300-400 | 30 | 25 | 90 |  |  |
| 5.2 | Between UE supporting V2X application and RSU via another UE supporting V2X application |  | [CPR.P-006] | [50-1200] | 2 | 500 |  |  |  |
| 5.5 | Between UEs supporting V2X application | Driver control | [CPR.P-007] | 300-400 (NOTE 3) |  | 25 | 90 |  |  |
|  |  | Fully automated driving | [CPR.P-008] | 1200 |  | 10 | 99.99 |  | 80 |
| 5.12, 5.13 | Between UEs supporting V2X application | Driver control/ Limited automated driving | [CPR.P-009] | [6500] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-010] |  |  | [20ms] |  | [65] | [5] sec * (max. relative speed) [m/s] |
| 5.12, 5.13 | Between UE supporting V2X application and RSU | Driver control/ Limited automated driving | [CPR.7.P-011] | [6000] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  |  | Fully automated driving | [CPR.7.P-012] |  |  | [20ms] |  | [50] | [5] sec * (max. relative speed) [m/s] |

NOTE 1:	This value does not including security related messages component.

# NOTE 2:	This value is end-to-end latency.

NOTE 3:	This value is applicable for both triggered and periodic transmission of data packets.

***************************** End of Change  ***************************************