3GPP TSG-SA WG1 Meeting #77	S1-171022

Jeju, KOREA, 13-17 FEB 2017	Revision of S1-171xxxx

Title:	eV2X Normative Requirements for Extended Sensors

Agenda Item:	8.1

Source:	LG Electronics

Contact:	SungDuck Chun (sungduck.chun@lge.com)

- Background

At SA1 #76bis, it was agreed that normative requirements for eV2X be incorporated into new TS.

During Study phase of eV2X, potential requirements of 25 use cases were consolidated into 5 categories.

- General Aspect

- Platooning

- Extended sensor

- Advanced driving

- Remote driving

This contribution proposed to adopt requirements related to Extended Sensors into new TS.

Following is latest CPRs considering 22.886V15.0.0 and S1-170319, which were agreed at SA1 #76bis

Table 7.2.4-1 Performance requirements for extended sensors Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Communication range (meters). Section #; Description; CPR #. 5.3; Between UEs supporting V2X application Fully automated driving; [CPR.E-001]; 10; 95; Peak data rate [25]. 5.6; Between UEs supporting V2X application; Driver control; [CPR.E-002]; [1600]; 100; 99; 1000. Fully automated driving; [CPR.E-003]; 3; 99.999; 200. [CPR.E-004]; 10; 99.99; 500. [CPR.E-005]; 50; 99; 1000. [CPR.E-006]; 1000; 50. 5.16; Between UEs supporting V2X application; Driver control/ Automated driving; [CPR.E-007]; [50]; 90; [10]; [100]. Fully automated driving; [CPR.E-008]; [10]; 99.99; [700]; [500].

- Discussion

Overall Comments:

- Because this is contents for TS, the direct link between use case and requirement cannot be pursued.

- The description of each requiremen should be general to avoid the impression that the KPIs is for specifics scenario.

- Avoid using use case name in the description as much as possible.

- ‘fully autonomated driving’ and ‘driver control, limited automated driving’ is not clearly defined.

- According to NHTSA/SAE, there are 6 level of automation for driving. Level 0, 1 and 2 is for ‘Human Driver” and level 3, 4 and 5 is for “automated driving system”.

- Use ‘Human Driver’ and “Automated Driving System’ to describe scenario.

- However, background information for each requirement will benefit readers for both in 3GPP and outside of 3GPP.

- Annex can be used to include iformation that allows tracing back to the use case.

- (Payload size * TX rate) = Data rate. If this value does not match, there will be a confusion.

- Either fill “Payload and Tx Rate” or “Data rate”. Not both.

CPR specific comments

Following table summarized discussion/proposed changes for each CPR.

| CPR Of TR 22. 886 | Discussion (empty cell means no change) | Req # for new TS | Suggested changes for new TS (empty cell means CPR is mirror into Req without any change) |
| --- | --- | --- | --- |
| [CPR.E-001] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.3; Between UEs supporting V2X application Fully automated driving; [CPR.E-001]; 10; 95; Peak data rate [25]. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. | [R.5.4-003] |  |
| [CPR.E-002] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Driver control; [CPR.E-002]; [1600]; 100; 99; 1000. | In section 5.6, repetition rate is mentioned as 5-10HZ | [R.5.4-001] | Clarify that max tx rate is 10hz |
| [CPR.E-003] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-003]; 3; 99.999; 200. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. | [R.5.4-004] | Data rate is 50 mbps up to 200 m |
| [CPR.E-004] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-004]; 10; 99.99; 500. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. Then, for 1000 m distance, data rate should be smaller than that. | [R.5.4-005] | Data rate is 25 mbps up to 500 m |
| [CPR.E-005] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-005]; 50; 99; 1000. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. Then, for 1000 m distance, data rate should be smaller than that. | [R.5.4-006] | Data rate is 10 mbps up to 1000 m |
| [CPR.E-006] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-006]; 1000; 50. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. | [R.5.4-007] |  |
| [CPR.E-007] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.16; Between UEs supporting V2X application; Driver control/ Automated driving; [CPR.E-007]; [50]; 90; [10]; [100]. |  | [R.5.4-002] |  |
| [CPR.E-008] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.16; Between UEs supporting V2X application; Fully automated driving; [CPR.E-008]; [10]; 99.99; [700]; [500]. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Section 5.16 mentions [27] as reference for ‘700 Mbps’. But, there is no reason why communication range is [500]. CPR.E.006 which requires 1Gbps only states 50m. Merge 008 to 006 and distance is set to 50 m | [R.5.4-008] | Merged into R.5.4-007 |

- Proposal

It is proposed to agree on the text proposal below.

------------------------------------------------------------------------------------------------

------------------------------ START of first text Proposal----------------------------

------------------------------------------------------------------------------------------------

5.4	Requirements to support Extended Sensors

Table 5.4-1 Performance requirements for extended sensors

| Communication scenario |  |  | Payload (Bytes) | Tx rate (Message/ Sec) | Max latency (ms) | Reliability (%) | Data rate (Mbps) | Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  | Req # |  |  |  |  |  |  |
| Between UEs supporting V2X application | Human Driver | [R.5.4-001] | [1600] | 10 | 100 | 99 |  | 1000 |
|  |  | [R.5.4-002] |  |  | [50] | 90 | [10] | [100] |
|  | Automated Driving System | [R.5.4-003] |  |  | 10 | 95 | [25] (NOTE 1) |  |
|  |  | [R.5.4-004] |  |  | 3 | 99.999 | [50] | 200 |
|  |  | [R.5.4-005] |  |  | 10 | 99.99 | [25] | 500 |
|  |  | [R.5.4-006] |  |  | 50 | 99 | [10] | 1000 |
|  |  | [R.5.4-007] (NOTE 2) |  |  | 10 | 99.99 | 1000 | 50 |
| NOTE 1: This is peak data rate. NOTE 2: This is for imminent collision scenario. |  |  |  |  |  |  |  |  |

------------------------------------------------------------------------------------------------

------------------------------ START of 2nd text Proposal----------------------------

------------------------------------------------------------------------------------------------

Annex <X> (informative):
<Background information on service Requirements>

For each requirement in this specification, Table X-1 indicates reference to the consolidated potential requirements in TR 22.886:

Table X-1 Reference information for requirements

| Requirement # in this specification | CPR # in TR 22.886 |
| --- | --- |
| [R.5.4-001] | [CPR.E-002] |
| [R.5.4-002] | [CPR.E-007] |
| [R.5.4-003] | [CPR.E-001] |
| [R.5.4-004] | [CPR.E-003] |
| [R.5.4-005] | [CPR.E-004] |
| [R.5.4-006] | [CPR.E-005] |
| [R.5.4-007] | [CPR.E-006] [CPR.E-008] |