3GPP TSG-SA WG1#94 e-Meeting	S1-211079

10th. May 2020

Title:	Update potential new requirements for use case 5.2 on roaming service

Agenda Item:	7.13.1

Source:	Futurewei,

Contact:	Amanda Xiang (amanda.xiang@futurewei.com)

Abstract: This document proposes modifications on the potential new requirements.

Discussion

As indicated in the use case, the main differences between this use case and existing local breakout roaming solution are related to the subscription and credential: UE has a temporary subscription with the local host network while still use home credential for the local service access in this new use case, while UE only has one home subscription and home credential if using the existing roaming solution. Based on existing 3GPP specification, when UE has home subscription, the home network can update user’s roaming agreement for accessing the local service.

Therefore, dual-subscriptions with the same home credential is a key foundation of this new use case and need to be reflected in the requirements.

## --------------------------------------------------------Change 1 begin ---------------------------

## 5.2	New Use case for roaming service

### 5.2.1	Description

When the user visits a foreign country for a few days, the user may want to have access to temporary mobile data service and prefer to have an arrangement directly with a local provider for the temporary mobile data service, while keeping an arrangement with its home provider for other services.

The use case in this section assumes that the UE uses the credential of the home provider also for the temporary mobile data service, though the subscription for the temporary mobile data service is bound to the local provider. The case where the local provider uses a separate independent credential for the temporary mobile data service is not in the scope of this use case.

### 5.2.2	Pre-conditions

Lily is traveling around the world. She has no strict schedule and always wanders off on her own. Sometimes she stays in a new place for several days if she likes there. In some place where Lily decides to buy the roaming data service provided by a local network e.g. via an app that she has obtained from a 3rd party data service provider.

### 5.2.3	Service Flows

1. Lily turns on her smartphone to see what networks and what data service plans are available.

2. the step 1 leads to a mobile data service shop which is displayed on the mobile screen, wherein there are several data combo services, each service is labelled with (network identifier, price, valid duration, valid location, data volume, type of service [e.g. data, voice etc] and optionally the mobile technology, e.g. 4G or 5G) and if there are anything that needs to be satisfied for that plan e.g. watching some advertisement may be required. Or, only foreigner is eligible for the service.

3. according the time that Lily plans to stay at current location, Lily chooses to buy the most cost-effective data combo service. For example, if Lily plans to stay there for one day, she buys the one-day data combo service; if Lily plans to stay there for one week, she buys the seven-days data combo service; if Lily just stay at the transfer airport for several hours and the WLAN connection is under bad condition, she buys the 1G data combo service.

4. Lily pays the bills for the data service she chooses at step 3. After a few seconds, Lily’s mobile phone has download necessary information and is ready to provide temporary mobile data service to Lily. Since then, Lily becomes a temporary subscriber of the VPLMN for that temporary mobile data service.

5. Lily may activate the mobile data service immediately or Lily may decide to activate the mobile data service when she lost WLAN connection. While this temporary subscription is valid, the UE selects and camps on the selected network providing the temporary local service.

### 5.2.4	Post-conditions

Lily can access to the internet and use apps via the mobile data service of the network where she is temporary subscriber now or can temporarily use the service during the validity period of the service within the valid location using the subscribed RAT, until the maximum data quota is reached.

Lily uses her normal phone number for making and receiving calls or SMS over the VPLMN by means of traditional roaming and charging information per the roaming agreement is reported to Lily home operator.

### 5.2.5	Existing features partly or fully covering the use case functionality

Existing specifications support the home-routed traffic service and local-breakout traffic service for roaming cases. For the local-breakout service, even if the traffic does not traverse the home network, the service is associated to the subscribed service toward the home network. In this use case, the temporary data service is different from the local-breakout service, because it is not the subscribed service toward the home network.

TS 22.261 has following requirements:

-	The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis. For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

Thus, following requirement is covered by above requirements:

-

-	The 3rd party service provider shall be able to agree the details of a temporary subscription for local services (such as validity period, location, QoS, services etc.) with a local hosting network.

Editor’s Note: It is FFS whether this requirement is supported by the existing requirement or not.

-	The 5G system shall allow a UE to present available local hosting networks to the user, which the user is allowed to use.

### 5.2.6	Potential New Requirements needed to support the use case

[PR.5.2.6-1]	5G system shall be able to support UE to use the credential from its home network to simultaneously access to the local services in the local hosting network with a separated temporary subscription and the services offered by the home network via the local hosting network.

[PR.5.2.6-1] The 5G system shall allow a user to manually select and access a specific local hosting network with the credential from his/her UE’s home operator.

[PR.5.2.6-2] The 5G system shall be able to authorize a UE to use the home network credential to make use of a specific local hosting network with a temporary hosting network subscription.