3GPP TSG-SA WG1 Meeting #94e	S1-211442

Electronic Meeting, 10 May – 20 May 2021	(revision of S1-211054)

Title:	FS_Resident: Resolving the Editor’s note on the use case of seamless switching to a service hosting environment via an evolved residential gateway

Agenda Item:	7.11.1

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Discussion

This document proposes an update to an existing use case for FS_Resident, “Use case: seamless switching to a service hosting environment via an evolved residential gateway” to resolve the Editor’s Note in clause 5.11.6.

Proposal

Due to the Editor’s Note

Editor’s Note:	The term “Service Hosting Environment” will be reconsidered to prevent overlap with existing architectural concepts.

The term Service Hosting Requirement is replaced by Application Server.

The following change to 3GPP TR 22.858 version 1.0.0 is proposed.

* * * *   First Change   * * * *

## 5.11	Use case: seamless switching from a service hosting environment to an application server via an evolved residential gateway

### 5.11.1	Description

This use case describes a scenario of service continuity for a UE consuming a low latency service provided by a service hosting environment. The low latency service (e.g. gaming) is maintained with the same QoS settings as the UE moves from the service hosting environment to an Application Server (AS) connected to an evolved Residential Gateway (eRG). While the AS can be a standalone platform, it can also be an on-site extension of an in-network Service Hosting Environment, e.g. an edge computing platform. In this case, the operator of the AS (e.g., a MEC server operator), can influence discovering/authorizing/orchestrating a residential application/service on the AS. The AS can be under the control of a 5G network operator, a customer or a third party. The service hosting environment and AS are served by the same 3GPP 5G core network (5G-CN).

Figure 5.11.1-1. Seamless switching to an Application Server via an evolved residential gateway.

### 5.11.2	Pre-conditions

A service hosting environment is attached to the 3GPP 5G-CN. An AS connects to the same 3GPP 5G-CN via an eRG. An indoor base station is deployed in the residential environment and is connected to the 3GPP 5G-CN via the eRG. The AS can be either in the MNO domain (i.e. a trusted application) or external to the MNO domain (i.e. an authorized third-party application).

### 5.11.3	Service Flows

We consider a scenario where a given UE, UE1, is receiving a service from a service hosting environment via a Premises Radio Access Station (PRAS 1) connected to the 3GPP 5G-CN. While the UE is enjoying its service, an event occurs that triggers the move of UE1 to an AS (e.g. UE1 arrives home). This AS is connected to the same 5G-CN via an eRG. As this event occurs, it is the expectation of UE1 to continue its service seamlessly with the same QoS albeit through the eRG in the AS. Below we present the service flows:

1.	UE consumes a low latency service (e.g. gaming) provided by a service hosting environment through the 3GPP 5G-CN (Path #1). This low latency service has a specific QoS.

2.	UE moves to an AS connected to the same 5G-CN via an eRG. The UE connects automatically to the PRAS 2 which in turn is connected to the eRG.

3.	The UE continues to receive its service seamlessly and with the same QoS through the eRG in the AS.

### 5.11.4	Post-conditions

The UE service is continued seamlessly through the eRG in the AS and with the same QoS as it was in the service hosting environment. The different paths going through the eRG in Figure 5.11.1-1 (Path #2, Path #3 and Path #4) are examples on the path the 5GS can choose to ensure seamless switching.

### 5.11.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] clause 6.5.2 Efficient user plane: Requirements

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network.

The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to:

-	another location served by a different Service Hosting Environment; or

-	another location served by an application server located outside the operator’s network, and vice versa.

The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows:

-	within a Service Hosting Environment; or

-	from a Service Hosting Environment to another Service Hosting Environment; or

-	from a Service Hosting Environment to an application server located place outside the operator’s network, and vice versa.

The 5G network shall support configurations of the Service Hosting Environment in the network (e.g. access network, core network), that provide application access close to the UE's point of attachment to the access network.

The 5G system shall support mechanisms to enable a UE to access the closest Service Hosting Environment for a specific hosted application or service.

The 5G network shall enable instantiation of applications for a UE in a Service Hosting Environment close to the UE's point of attachment to the access network.

3GPP TS 22.261 [2] clause 6.7.2: Priority, QoS, and policy control: Requirements

The 5G system shall be able to support E2E (e.g. UE to UE) QoS for a service.

NOTE:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

### 5.11.6	Potential New Requirements needed to support the use case

[PR 5.11.6-001] The 5G system shall be able to provide support for an eRG to interconnect with an Application Server.

[PR 5.11.6-002] The 5G system shall be able to maintain QoS for a UE, when it is moving between a Service Hosting Environment and an Application Server in the CPN that is connected to an eRG.

* * * *   End of Changes   * * * *