3GPP SA WG1 Meeting #98e 	S1-221020r3

Electronic Meeting, 09 May - 19 May 2022

# Source:	ZTE Corporation, CEPRI, China Telecom, China Unicom

# Title:	New WID on Measurement Data Collection

# Document for:	Agreement

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Measurement Data Collection

Acronym: MeasureData

Unique identifier:

Potential target Release:	{Rel-19}

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | New Services and Markets Technology Enablers (SMARTER) | Previous normative work including QoS monitoring requirements |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | Previous normative work in SA1, specifying industrial service requirements. |
| 840041 | Enhancements for cyber-physical control applications in vertical domains | Previous normative work in SA1, specifying industrial service requirements. |
| 790004 | QoS Monitoring | Previous normative work in SA1, specifying QoS monitoring service requirements. |

3	Justification

As indicated in TS 22.261-Annex D, the 5G system is required to provide specific communication services for different vertical domains e.g. critical infrastructure control, safe and efficient transport, integrated systems, environmental control etc. Considering the communication QoS will be the key factor to impact vertical applications, it is very important to support QoS monitoring.

However, the vertical industry applications have stringent communication service performance requirements which have been specified in TS22.104. The the existing QoS monitoring enabled by the 5G system can’t support sufficient monitoring granularity for network operators to accurately assess communication service quality.

As example in below table, the 5G based motion control is a typical use case in factory automation. And it has stringent requirements in latency, service availability and reliability as specified in TS22.104. This kind of periodic motion control application with transfer interval (500μs), will deliver 2000 messages per second.

TS22.104 Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user experienced data rate (note 12a) | Message size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) |  |
| 99.999 % to 99.999 99 % | ~ 10 years | < transfer interval value | – | 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |

In clause 6.23.2 of TS 22.261, it has specified that the 5G system shall support QoS monitoring with one update per second. It can be observed that existed requirement about refresh rate for QoS monitoring is too rough to be used for accurate assessment of communication service quality. Therefore, clear requirements for high refresh rate network measurement data collection and high refresh rate reporting are needed.

And it is also observed that the network measurement data collection refresh rate and reporting refresh rate are always same as the QoS monitoring refresh rate. To avoid network burden and process cost, it is suggested to clarify that the refresh rate for network measurement data collection and the refresh rate for reporting can be different, e.g. higher refresh rate network measurement data collection and lower refresh rate network measurement data reporting.

So, the following gaps are identified:

- The 5G system hasn’t supported accurate QoS monitoring with higher refresh rate especially for periodic deterministic communication service required by vertical industries, which means that the network measurement data collection and reporting with different refresh rate higher than one measurement per second or above is needed.

Considering the above analysis, it is proposed to add new requirements for network measurement data collection with high refresh rate to improve QoS monitoring.

4	Objective

This work item proposes new requirements for TS22.261 which will augment existing TS-22.261 QoS monitoring requirements. The following related requirements will be included:

- High refresh rate network measurement data collection and efficient reporting.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Add new requirements for QoS monitoring | SA#97 (Sept 2022) |  |

6	Work item Rapporteur(s)

ZhouYingjun, ZTE Corporation, zhou.yingjun@zte.com.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

SA2, RAN

9	Supporting Individual Members

Supporting IM name

ZTE Corporation

China Electric Power Research Institute

China Unicom

China Telecom

OPPO

CALTTA

China Mobile

Spreadtrum

Novamint