3GPP TSG-SA WG1 Meeting #99e 	S1-222167

Electronic Meeting, 22 Aug – 1 Sept 2022	(revision of S1-22xxxx)

Source:	China Telecom

pCR Title:	Update of Use Case of Rainfall Monitoring

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Yuying Zhang, zhangyy45@chinatelecom.cn

Abstract: This contribution updates the use case of rainfall monitoring for the FS_Sensing.

* * * First Change * * * *

5.3	Use case of rainfall monitoring

5.3.1	Description

Rainfall monitoring is a topic of great importance for several application contexts: hydraulic structure design, agriculture, weather forecasting, climate modelling, etc. At present, the most widely used measurement method is rain gauge.

Traditional rainfall monitoring use rain gauges, which are located at a particular location. Wide-area rainfall monitoring using traditional rain gauges would be costly. The base stations are deployed by the operators with radio cell planning that could cover a wider area. With base stations monitoring the rainfall, for example rain rate (mm/h), it could obtain a horizontally wider-area measurement.

Radio signals, as they propagate through the atmosphere, are reduced in intensity by constituents of the atmosphere. Oxygen and water vapor are the two major components which are responsible for the signal absorption. If it is a rainy day, an additional attenuation caused by rain further increases the propagation path loss. [7] The rain attenuation depends on the size and distribution of the water droplets, hence, by quantifying and modelling the base station signal measurements, we are able to know the rain rate.

The mmWave bands, such as 28GHz and 38GHz have been used to assess coverage, large-scale path loss, and fading and multipath effects [6]. Since the 28 GHz and 38 GHz bands are also licensed for wireless backhaul communications, these frequencies can used for rainfall monitoring [7].

The granularity of the rainfall monitoring could be smaller than the traditional measurements.

5.3.2	Pre-conditions

Peter is a farmer who takes care of a big farm that grows different crops. Peter needs to monitor the rainfall of his farm to manage reasonable irrigation, drainage and fertilizer. When there is less rainfall, Peter can select reasonable irrigation plans to improve the farmland water content condition. When there is high rainfall, Peter should improve the drainage system and fertilize the crops to avoid crop losses.

5.3.3	Service Flows

- Peter has a subscription for the premium service of rainfall monitoring for a more granular location.

- Peter is at daily working routine and wants to check the timely rainfall information from the weather application on his phone.

- The base station obtains the NR based sensing measurement data every hour and the 5G system processes the sensing measurement data to obtain sensing results and exposes the NR based sensing results to the weather application via the core network.

- Based on the sensing results above, the application server obtains the rainfall information associated with location information.

- Peter obtains timely rainfall information from weather application on his phone.

5.3.4	Post-conditions

Peter could check the rainfall information at any time on his phone. Based on the timely rainfall information, Peter could plan the irrigation, drainage and fertilizer for the crops in his farm.

5.3.5	Existing feature partly or fully covering use case functionality

None

5.3.6	Potential New Requirements needed to support the use case

[PR. 5.3.6 - 001] The 5G system shall support collection of the NR based sensing measurement data from the base station.

[PR. 5.3.6 - 002] Based on operator’s policy, the 5G system shall support mechanisms to process the sensing measurement data to derive the sensing results.

[PR. 5.3.6 - 003] Based on operator’s policy, the 5G system shall provide mechanisms to expose NR based sensing results with assisted information, e.g. location, to a trusted 3rd party application via the core network.

[PR. 5.3.6 – 004] The 5G system shall support sensing services with KPIs as given Table 5.3.6-1.

Table 5.3.6-1: Performance requirements for the use case of rainfall monitoring

| Scenario | Sensing Range | Sensing Frequency |
| --- | --- | --- |
| Rainfall monitoring | 200m (Note 1) | [1/1min-1/15min] (Note 2) |
| Note 1: The sensing range for rainfall monitoring refers to the reference [5]. Note 2: The sensing frequency should be adjustable and depends on the requirement of the weather application. |  |  |

Editor’s Note: The KPI table should be aligned with the agreed common KPI table format.

* * * End of Change * * * *