3GPP TSG SA WG 1 Meeting #99e 	S1-222034r2-Intel

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-222zzz)

Source:	Samsung, Intel

pCR Title:	Pseudo-CR on Spatial Mapping and Localization Service Enabler Use Case

Draft Spec:	3GPP TR 22.856-010

Agenda item:	7.4

Document for:	Approval

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This P-CR proposes a use case for a new 'spatial localization' service enabler.

1. Introduction

Please consider the use case below.

2. Reason for Change

This is a use case for the FS_Metaverse study, applying concepts of environment mapping which is in some ways analogous to location based services

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.856 version 0.1.0.

r1		changes

- corrected the reference

- corrected the definition of Localization

- added definition of Spatial Mapping Service, and Spatial Localization Service

- added a NOTE and EN to requirement 1 to clarify

- added clarification to the section describing mapping

- changed wording of the requirements to align with the new terminology

r2		changes

- a word was added in 5.A.1 'spatial mapping considerations' for clarity

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[G]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[H]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[I]			           https://remero6.wixsite.com/wslhanyang/5g-slam

[J] 				Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based       	          SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular 				     Technology

[K] 	Petrov, V., Gapeyenko, M., Paris, S., Marcano, A. and Pedersen, K.I., 2022. Extended Reality (XR) over 5G and 5G-Advanced New Radio: Standardization, Applications, and Trends. arXiv preprint arXiv:2203.02242.

[L] 	3GPP TR 26.928 Release 17 - Extended reality in 5G -https://www.3gpp.org/ftp/Specs/archive/26_series/26.928/26928-h00.zip

[M] 			https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[N] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[O] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[P] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[Q] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next Change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Spatial Map: A collection of information that corresponds to space, including information gathered from sensors concerning characteristics of the forms in that space, especially apperance information.

Localization: A known location in 3 dimensional space, including an orientation, e.g. defined as pitch, yaw and roll.

Spatial Mapping Service: A service offered by a mobile network operator that gathers sensor data in order to create and maintain a Spatial Map that can be used to offer customers Spatial Localization Service.

Spatial Localization Service: A service offered by a mobile network operator that can provide customers with Localization.

Next Change

## 5.A	Spatial Mapping and Localization Service Enabler Use Case

### 5.A.1	Description

Spatial mapping and localization are constructing or updating a map of an unknown location and tracking an object simultaneously. For the localized mobile metaverse use case 5.1, the service provider or operator needs to provide and use spatial map information, i.e. a 3D map of indoor or outdoor environment. This use case consider show a spatial map can be created and employed, both as service enablers. The creation and maintenance of the spatial map is referred to as Spatial Mapping Service and the employment of the map to identify the customer's Localization is termed Spatial Localization Service. Base Station (BS) along with UE can construct spatial mapping of any unknown location, which can be used for autonomous vehicles, Extended reality (XR), drone navigation, robots navigation etc.,. spatial mapping and localization obtain model and tracking of an environment at the same time unlike camera, which needs a reference model for modelling and tracking. Spatial mapping will classify objects into modelling and tracking of stationary and moving objects. For stationary object, spatial mapping has to estimate the number of objects, type of object and position. Whereas for moving objects, spatial mapping has to determine the position, type of object, direction, speed. Once spatial mapping and localization service has all the object information, it has to map all the stationary and moving objects related to UE’s environment and relay it to UE and surrounding subscribed users as well [J, H, I]

Figure 5.A.1-1: Spatial Location Services Enabler

Specifically, this use case proposes two spatial localization service enablers.

1)	Spatial Mapping

Sensor data gathered transparently is processed in order to identify the static and transient forms. For example, in Figure 5.A.1-1 (A), the Rubens room in the Louvre, sensor data captures information. (B) This information is processed to identify the static and transient information, to establish an up to date spatial map. (C) In combination with location information (for the sensors) and other information (e.g. architectural specifications of the Louvre), a spatial map can be achieved in which not only the forms but also their locations in 3D space are known.

2)	Localization

Given that a spatial mapping exists, (A) sensor data for a user (from devices communicating by means of a UE) can be captured, (B) compared with the spatial map, (C) used to identify in 3D space the position, viewing direction, angle, etc. of the user.

Spatial Mapping Considerations

Examples when spatial mapping could be useful:

- 	A government conducting a digital city project can build a 3D spatial map of outdoor environment of an entire city or public spaces such as outdoor parks or indoor offices of their government building.

- 	A navigation (or Spatial Localization Service) service provider can build a 3D spatial map of outdoor environment of entire roads or public spaces. An operator’s partner or operator also can build a 3D spatial map of outdoor environment.

-	A customer wanting mapping of their indoor environment, e.g. the interior of a commercial space such as the cheese shop as described in 5.X.

-    Autonomous vehicle (AV) driving on a freeway and it is enabled with spatial mapping and localization service.  Spatial mapping and localization services on the AV can map all the vehicles- along with speed and distance- around it, including road signs. This information enables AV to anticipate the kind of vehicles around the lanes, which aid in navigation and avoid accidents.

It is a hard task to perform the mapping of the entire city using a vehicle by traversing various roads and spaces. It also requires lots of time and effort (for data conversion, etc.) if the work is performed in offline. If the multiple capturing devices are used in parallel, the spatial map data in the same location should be consistent over the different cameras and input devices to generate the spatial map.

In extended XR, the virtual reality super imposes an Augmented objects onto the 3D real world as shown in Figure-5.A.1-2 a & b. AR has to align the coordinating systems between the real world and the virtual world known as registration. The conventional tracking system combines images from the camera with a reference model to super impose the AR objects to the real world. In spatial mapping and localization services, model and tracking are objected at the same time, which is faster and less computational. Spatial mapping will continuously sense and update the base station with the current state of sensing parameters. Base station will also continuously sense the surroundings of the UE (AR deice) and combine both the sensing data from the UE and BS to form spatial map, which is offloaded to the VR device. Based on the spatial map the AR will super impose AR objects to the real world [K, L, M].

The mobile capturing device, vehicle or robot equipped with multiple stereo/mono RGB cameras and multiple LiDAR to capture various qualities of images and depth information of the environments. As an example in [G], a mobile indoor robot is equipped with two LiDARs, 6 industrial cameras and 4 smartphone cameras. Based on the example, we can derive the uplink bandwidth requirements for one mobile indoor mapping robot. There is a corresponding use case for 'transparent sensing' in TR 22.837.

Editor's Note: Provide a complete reference to 22.837 if / once S1-222030 is implemented.

Localization Considerations

The environment mapping can be used for providing a visual positioning service, for enhancing the accuracy of location service or for helping the metaverse contents management system for spatial internet.

In this use case, the UE provides uplink sensor information that can be interpreted, along with a spatial map, to identify Localization. This is analogous to the process used by the UE to provide sensor data that the 5G system can use to determine UE location for Location Services.

In 3GPP-R16, bistatic localization is performed on both downlink and uplink using time-difference-of-arrival (TDoA), angle-of-arrival (AoA), and angle-of-departure (AoD) measurements both at the base station. Both bistatic and monostatic have limitation with their accuracies, with the advent of high BW mmWave brings high resolution in both time-delay and angle domains. The spatial mapping problem can be divided into front and a back-end problem. In front-end, the problem is to determine the data association - between landmarks and measurement directions. The back-end problem is to find the probabilistic SLAM density for a given data association- determined in the front end. Most of the back-end algorithms are Kalman filter (EKF), Fast-SLAM and Graph-SLAM, which are based on Bayesian method [N, O, P].

Figure 5.A.1-2: a) Mapping of stationary objects using 5G SLAM, b) Super imposing Augmented reality to 3D real-world using 5G SLAM (Source: [M], theinnerdetail.com)

### 5.A.2	Pre-conditions

For Spatial Mapping

Mobile operator or service provider can determine the target area for the spatial mapping. The target area can be divided into multiple areas where each can be mapped with multiple capturing devices (e.g. capturing indoor robot, vehicles).

Each capturing device is equipped with multiple sensors, e.g. mono or stereo RGB camera or one or more multiple LiDAR cameras. The intrinsic parameter of cameras equipped with the device are pre-configured in the device.

Each capturing device has a capability with high resolution positioning system, the positioning technique also can derive the altitude information.

An indoor robot can be equipped with non-3GPP based high resolution positioning system such as UWB. To derive the exact the position of the capturing device, each capturing device also can utilize the structure-from-motion technologies.

The capturing devices communicate via a UE that can access the mobile network of the MNO that supports spatial mapping. The capturing devices can either sense the spatial area to be mapped, or move about sufficiently that the capturing devices can be used to acquire sensor information corresponding to the area.

For Localization

A set of sensors are accessible by a UE. These could be built into the terminal equipment or communicate with it in some way that is out of scope of this use case, e.g. using a cable, a personal area network, etc.

The UE can access a mobile network of MNO M that offers localization services.

5.A.3	Service Flows

For Spatial Mapping

1.	A capturing mobile device attaches to the mobile network and becomes authorized to deliver sensor information for the purposes of spatial mapping.

2.	The capturing mobile device starts the mapping operation.

a.	The capturing mobile device arranges for its sensors to provide information as needed by the 5G system. This may require some configuration of the capturing mobile device, e.g. the sensors or to control the uplink communication of sensor information.

3.	The capturing mobile device navigates, e.g. with a pre-defined route, within the selected target region in order to provide a sufficiently complete set of sensor information for the space to be mapped.

4.	The capturing mobile device uploads the captured RGB camera images and LIDAR depth images to the 5G system with the positioning information of the mobile device.

5.	The mapping server collects and analyzes the information provided to cumulative create or update a spatial map.

For Localization

1.	A mobile device requiring localization attaches to a mobile network and becomes authorized to obtain Localization services.

2.	The mobile device uses sensors to capture information corresponding to the point and direction that must be localized.

3.	The mobile device sends via uplink communication the sensor information to the 5G system.

4.	The 5G system uses the sensor information and a spatial map to determine the localization, that is, the corresponding positioning information and sensor pose.

5.A.4	Post-conditions

For Spatial Mapping

The spatial mapping enabler re-structures the data provided by mobile capture devices to create a spatial map. This information will be combined with other information available, e.g. floor plans of buildings, survey data, satellite images, etc. Over time sufficient information will be captured to allow the spatial mapping enabler to distinguish between static and dynamic objects in the environment.

For Localization

The result of the localization service is available as a service enabler. This could be provided to the UE for use by applications or exposed to a third party by means of an API by the 5GS. This information can be used for many purposes, especially for media based applications that require localization to control the rendering of AR or MR content. Localization can be done over time, e.g. to track a user's movement.

The localization enabler can enhance location services as it can identify with some degree of precision a location by means of diverse sensors compared with known location data in the spatial map.

### 5.A.5	Existing feature partially or fully covering use case functionality

Location Services provide location information which can be used to support services that are triggered or informed by the user's location.  This information is not at the degree of accuracy needed for some applications, e.g. AR. Location Services also do not work in all environments, e.g. indoor.

The 5GS supports uplink media and uplink sensor data communication.

In clause 4.1.4 of TR 26.928 [L] - XR spatial mapping and localization - has been proposed. This use case specifies the key areas of XR and AR are spatial mapping, which is creating a map of the surrounding areas, and localization - positioning the user in that map. This use case depends on multiple external sensors such as monocular/stereo/depth cameras, radio beacons, GPS, inertial sensors, etc., There is no requirement on how 5G sensing can be used for spatial mapping and localization services.

Positioning in 5G Networks been proposed in 3GPP release-16, it specifies positioning signals and measurements for the 5G NR. In release-16, 5G Positioning architecture extends 4G positioning architecture by adding Location Management Function (LMF) and Transmission reception points (TRP).  5G- along with enables- provides new positioning methods based on multi-cell round-trip time measurements, multiple antenna beam measurements, multiple to enable downlink angle of departure (DL-AoD) and uplink angle of arrival (UL-AoA) [M, Q, P]. Current 5G system supports positioning of the device-based but not device-free – Objects that do not radiate EM signals. We propose to add new requirement in section- 5.A.6 to extend this architecture to support spatial mapping and localization service enablers.

### 5.A.6	Potential New Requirements needed to support the use case

#### 5.A.6.1	Requirements for Spatial Mapping

[PR5.A.6.1-1]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to authorize Spatial Mapping Service.

NOTE: 	Some locations may not be appropriate to map, e.g. for regulatory reasons, or where they require user consent.

Editor's Note: It is FFS whether and how an MNO can identify whether Spatial Mapping Service authorization is required, and how this can be obtained.

[PR5.A.6.2-2]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms for an authorized UE to provide sensor data that can be used to produce or modify a spatial map.

[PR5.A.6.1-3]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to receive and process sensor data to produce or modify a spatial map.

[PR5.A.6.1-4]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to expose a spatial map or derived localization information from that map to authorized third parties.

Editor's Note:	The requirements for the precision of spatial positioning of the sensors that provide sensor data and the spatial map are FFS.

#### 5.A.6.2	Requirements for Localization

[PR5.A.6.2-1]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to authorize Spatial Localization Service.

[PR5.A.6.2-2]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms for an authorized UE to provide sensor data that can be used to for Spatial Localization Service.

[PR5.A.6.2-3]	Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to expose Spatial Localization Service information to authorized third parties.

Editor's Note:	The requirements for the precision of spatial positioning of the sensors that provide sensor data and the spatial map are FFS.

End of Changes