Agenda Item:

Title: 	SAP offered to RANAP

Date	15 Mar 1999

Action	For Discussion and inclusion in S3.12

Source	Lucent Technologies

# Introduction

This contribution highlights the requirements placed by RANAP on its signalling bearer. These are primarily based on the requirements stated in [1].

The driving principle behind the specification of RANAP is the independence from the signalling bearer.

It is however reasonable to assume that RANAP will require similar services to the signalling bearer which are similar to ones that required by the current GSM BSSMAP.

Therefore it seems straightforward to assume that an SCCP like interface will exist between RANAP and its service provider.

This is reflected in the service requirement described in this contribution.

# Services Requirements

The services offered to RANAP should include the following:

- Provide reliable transfer of control plane signalling messages in both connectionless mode and connection-oriented mode;

- Provide flow control and sequence control for connection-oriented services.

- Provide separate independent connections for distinguishing transactions with individual UE’s;

- Supervise the ‘UE connections’ and provide connection status information to the Upper Layers for individual UE’s;

- Provide networking and routing functions

- Provide redundancy in the signalling network

- Provide load sharing

# SAP to the signalling bearer

As outlined above, the SAP offered to RANAP by its service provider should be independent of the nature of the signalling bearer itself. Furthermore given the general consensus on SS7 as the signalling bearer on the Iu interface for the PSTN domain, it is reasonable to assume that the SAP offered to RANAP by its service provider be an SCCP SAP.

This should be taken into consideration when the choice of the signalling bearer for the IP domain is made.

Figure 1 depicts this concept by showing a single service access points which is used to access the services offered to RANAP both by an SS7 stack (through SCCP) and by TCP/IP (through the RANAP Adaptation Layer).

Figure 1 RANAP SAP:

The following paragraphs summarise the primitives offered trough the SCCP SAP.

- Connection-Oriented Primitives

| Primitives |  |  |
| --- | --- | --- |
| Generic name | Specific name | Parameters |
| N-CONNECT | Request | Called address |
|  | Indication | Calling address |
|  | Response | Quality of service parameter set |
|  | Confirm | User data |
|  |  | Importance |
|  |  | Connection identification |
| N-DATA | Request | Importance |
|  | Indication | User data |
|  |  | Connection identification |
| N-DISCONNECT | Request | Originator |
|  | Indication | Reason |
|  |  | User data |
|  |  | Connection identification |
|  |  | Importance |

- Connectionless Primitives

| Primitives |  |  |
| --- | --- | --- |
| Generic name | Specific name | Parameters |
| N-UNITDATA | Request | Called address |
|  | Indication | Calling address |
|  |  | Sequence control |
|  |  | User data |
|  |  | Importance |
| N-NOTICE | Indication | Called address |
|  |  | Calling address |
|  |  | Reason for return |
|  |  | User data |
|  |  | Importance |

- Management Primitives

| Primitives |  |  |
| --- | --- | --- |
| Generic name | Specific name | Parameters |
| N-COORD | Request | Affected subsystem |
|  | Indication | Subsystem multiplicity indicator |
|  | Response |  |
|  | Confirm |  |
| N-STATE | Request | Affected subsystem |
|  | Indication | User status |
|  |  | Subsystem multiplicity indicator |
| N-PCSTATE | Indication | Affected signalling point Signalling point status Restricted importance level Remote SCCP status |

# Conclusion

It proposed that figure 1 in this contribution is included in section 5.2 of [1] together with the following text:

Figure 1 RANAP SAP

Figure 1 shows the point at which the service primitives are invoked. A single SAP is defined independently of the signalling bearer. The SAP provides the SCCP primitives. The figure is not intended to constrain the architecture.

[1]	Toc R3-99-128 “S3.12 ‘Iu interface Signalling Transport’”, Source: editor