Source:	Nortel Networks

Subject:	Proposed LS on Agreed changes to 23.20 on flexible use of Iu

Following the agreement on Tdoc S2-99-134 at the last S2 meeting it is proposed that S2 sends this document to RAN2&3 asking these groups to take account of this decision in their work

3GPP TSG SA Wg 2	S2-99-     134      .

Stockholm   15 - 19 March 99

Source:	Nortel Networks

Subject:	Agreed changes to 23.20 on flexible use of Iu

## 7.1 General

The phase 1 UMTS/Release ’99 GSM standards should provide the capability to support:

- a core network based on an evolved 2G MSC and an evolved SGSN

- an optionally evolved Gs interface

- class A GSM’ mobiles.

- Transcoder location shall be according to 23.30

- UMTS/IMT2000 Phase1 (Release 99) network architecture and standards shall allow the operator to choose between Integrated and Separated core networks for transmission (including L2)

- The UMTS standard shall allow for both separated and combined MSC/VLR and SGSN configurations.

- The UE shall be able to handle separated or combined MSCs and SGSNs.

- There can be several user planes to these CN nodes.

- The following general concepts should be followed :

- Separate the layer 3 control signalling from the layer 2 transport discussion (do not optimise layer 3 for one layer 2 technology).

- MSC-MSC layer 3 call control is out of scope of standardization in SMG.

- As future evolution may lead to the migration of some services from the CS-domain to the PS-domain without changes to the associated higher-layer protocols or functions. UMTS release 99 shall provide the flexibility to do this in a way that is backwards compatible with release 99 UEs provided this does not introduce significant new complexity or requirements in the system.

## 7.2 Iu Interface

- Transport protocol across the Iu interface for UTRAN shall be according to 23.30

- The UTRAN shall support two logically separate signalling flows via Iu to combined or separate network nodes of different types (MSC and SGSN).

- The UTRAN shall contain a "domain distribution function" to route transparent application-level control signalling from the UE to the correct core network domain. The UE shall indicate the type of application being addressed (eg via a protocol discriminator). The UTRAN shall map this on to the correct Iu instance to forward the signalling.

- UTRAN-services (including radio access bearers) shall be independent from the core network domain used to access them. Either core network domain can access any appropriate UTRAN-service (eg it should be possible to access a "speech" radio access bearer from the PS-domain).