Agenda Item: 	23.20

Title: 	Clarification of removal of an acknowledgement layer for UMTS phase 1

Date	24-May-1999

Source	Nokia

Related To 	Clarification of chapter 7.2.2.1 in 23.20

# DISCUSSION

Chapter 7 in 23.20 contains important working assumptions for the interested groups. It has been found that statement currently in 23.20 in chapter 7.2.2.1 about the removal of an acknowledgement layer may lead to confusion in those groups. This is due to fact that it does not clear state whether an acknowledgement layer is intended to be removed from UMTS phase 1 or from GSM/GPRS R99 or from both. Intention of this contribution is to clarify that issue.

# PROPOSAL

Following change in section 7.2.2.1 in 23.20 is proposed:

“Hence the acknowledgement layer LLC that is needed in GSM/GPRS is not needed between CN and UE in UMTS because data reliability is provided between UE and SRNC. In case of  SRNS relocation or GSM <-> UMTS Hand-Over, transmission of not yet acknowledged downstream packets between (old and new) SRNC or between SRNC and 2G-SGSN provide the necessary reliability.”