SA WG2 Meeting #100	S2-134059

11 - 15 November 2013, San Francisco, USA	(revision of S2-13xxxx)

Source:	CATT, CATR

Title:	Update of Solution I3

Document for:	Discussion/Approval

Agenda Item:	6.4.6

Work Item / Release:	ProSe/Rel 12

Abstract of the contribution: this contribution updates solution I3 to support retrieval of temporary ProSe ID and to include some evaluations. It is proposed to include them into TR 23.703.

1. Introduction

This paper updates solution I3 to clarify, what the ProSe ID is, how to login to application server with ProSe ID, and what kind of ProSe ID should be used in EPC level discovery and direct discovery respectively.

In addition, it is said that a permanent ProSe ID exposed on the air may cause security problem during direct discovery procedure, therefore, this paper proposes to allocate a temporary ProSe ID, e.g. expression code, for direct discovery procedure.

Detail description refers to the updates of the solution.

2. Proposed Change in TR 23.703

>>>Start Changes<<<<

6.4.3 	Solution I3: Retrieval of ProSe Identity

6.4.3.1	Functional description

This solution assumes that there is a unique ProSe identity which has one to one mapping with IMSI for each user subscribing to ProSe service, and unique application layer user identity per ProSe-enabled application, each ProSe-enabled UE can support multiple ProSe-enabled applications operating simultaneously.

ProSe ID is assigned/allocated by ProSe Server when user subscribes to the ProSe service. It is a permanent identity for identifying a ProSe user, it can also be used to indicate the subscription of the user, the entity storing the subscription, and ProSe function subscribed in home PLMN, for example, EPSID.

When a ProSe-enabled application performs registration procedure, it provides both of application layer user ID and ProSe ID in the registration request to application server. Alternatively, it can provide both of application layer user ID and ProSe ID in the registration request to ProSe function first, then ProSe function perform registration to application server with application layer user ID and ProSe ID. The Application Server stores the mapping of application layer user ID and ProSe ID.

Before performing discovery procedure, a discovering/discoverable UE has to get ProSe IDs used in the discovery procedure, different ProSe IDs are needed in different ProSe discovery procedure:

For EPC level discovery, if the discovering UE has got the ProSe IDs of target users, it can send EPC level ProSe discovery request to ProSe function. Otherwise, it has to get the ProSe IDs of target users first.

For direct discovery, if a user using ProSe enabled UE would like to announce itself, the user has to get his temporary ProSe ID (e.g. expression code) beside ProSe ID; On the other hand, a discovering UE gets the user’s temporary ProSe ID first, and monitors this temporary ProSe ID in the discovery procedure.

After the successful discovery, the discovery results should be stored in the UE with ProSe ID of target users for a lifetime. Then, if another ProSe-enabled application on the UE also requests ProSe discovery, and after getting ProSe ID of target user indicated by application layer user ID, it finds that the ProSe ID has been stored in the UE together with the valid discovery result, the application can re-use the stored discovery result.

6.4.3.2	Signaling Flow

6.4.3.2.1	Login procedure of ProSe enabled application

There are two options for ProSe-enabled application to login to ProSe application server, i.e. login over PC1 interface and login over PC3 interface, as shown in figure 6.4.3.2.1-1.

# Figure 6.4.3.2.1-1: Login procedure of ProSe enabled application

0. 	ProSe ID is assigned/allocated for a user by ProSe Server when the user subscribes to ProSe service, it has one to one mapping to IMSI. This procedure is out of 3GPP scope. ProSe ID is a permanent identity for identifying a ProSe user, it can also be used to indicate the subscription of the user, the entity storing the subscription, and ProSe function subscribed in home PLMN, for example, MSISDN or a new defined identity like user@proseserver.operator.3gpp.

Option A. login over PC1 interface:

1-2. A ProSe enabled application running on a ProSe enabled UE gets ProSe ID of the user who subscribed ProSe service by using this UE.

3. The application sends application layer login request carrying ProSe ID to application server over PC1 interface.

4. The application server stores the mapping of application layer user ID and ProSe ID, in addition, the discovery setting related information if possible, e.g. whether the user represented by the ProSe ID is discoverable for this application.

5. The application sends back a response over PC1 interface.

Option B. login over PC3 interface:

1-2. A ProSe enabled application running on a ProSe enabled UE triggers the UE to send login request with application ID, application layer user ID and ProSe ID to ProSe function over PC3 interface. The ProSe function is identified by ProSe ID, such as constructing a FQDN containing ProSe ID to use DNS service

3. The ProSe function authenticates that this UE is allowed to use ProSe service, then sends registration request with application layer user ID and ProSe ID to Application server identified by application ID.

4. The application server stores the mapping of application layer user ID and ProSe ID, in addition, the discovery setting related information if possible, e.g. whether the user represented by the ProSe ID is discoverable for this application.

5-7. The Application server sends back a response to the ProSe enabled application.

Editor's Note: In the option B, it is FFS that the login request from UE are sent on 3GPP control plane or 3GPP user plane.

6.4.3.2.2	Retrieval of ProSe ID for discoverable procedure

# Figure 6.4.3.2.2-1: Retrieval of temporary ProSe ID for discoverable procedure

1. After a user has logged in a ProSe-enabled application, the user is allowed to be discoverable through direct ProSe discovery procedure as long as the user has such requirement. In this case, the user can send discoverable request to ProSe enabled UE.

2. In order to perform direct discovery without disclosing user’s permanent ProSe ID on the air, the UE needs get user’s temporary ProSe ID from ProSe function if there is no valid temporary ProSe ID, then the UE sends Get temporary ProSe ID request including application ID and user’s ProSe ID to ProSe function. The ProSe function is the subscribed ProSe function identified by user’s ProSe ID.

3. The ProSe function retrieves user’s subscription by using user’s ProSe ID, and then authenticates that the UE and the application are allowed to use ProSe service. Further, the ProSe function allocates a temporary ProSe ID corresponding to the provided ProSe ID.

4. The ProSe function sends temporary ProSe ID back to the UE and authorizes the UE to perform discovery procedure.

5-6. The ProSe-enable UE (3GPP layer) sends response to upper layer (application layer) and performs direct discovery procedure by using received temporary ProSe ID.

Temporary ProSe ID may be re-allocated periodically or as requested by user or ProSe function.

6.4.3.2.2	Retrieval of ProSe ID for discovery procedure

# Figure 6.4.3.2.2-2: Retrieval of ProSe ID for discovery procedure

1-2. A ProSe-enabled application is requested by user to discover some target users, this application layer requests will trigger the Prose-enabled UE(3GPP layer) to retrieve the Proe IDs of users by using indicated application layer user identities. The UE (3GPP layer) sends Get ProSe ID Request including application layer user ID list of target users, user’s ProSe ID and application ID to the ProSe function, the ProSe function is the subscribed ProSe function identified by user’s ProSe ID.

3. The ProSe function retrieves user’s subscription by using user’s ProSe ID, and then authenticates that the UE and the application are allowed to use ProSe service. After that, the ProSe function sends the Get ProSe ID Request to Application server identified by application ID to ask for searching the ProSe IDs of target users.

4. The Application server finds the ProSe ID and the discovery setting of each target user by indexing its application layer user ID, then sends Get ProSe ID Response including the list of application layer user ID and ProSe ID pair for each discoverable user to ProSe server. For supporting restrictive discovery, the application server needs to check the discovery setting of target user to determine how to response, e.g. if the discovery setting of target user is not discoverable, the application server may not provide ProSe ID or return a negative response.

5-6. When the ProSe function receives target user’s ProSe IDs, it finds the temporary ProSe ID corresponding to these ProSe IDs if necessary, then sends the list of <application layer user ID, ProSe ID and/or temporary ProSe ID> pairs to the UE and authorizes the UE to perform discovery procedure.

7. The UE stores the mapping of application layer user ID and ProSe ID/temporary ProSe ID which are included in the response message, which allows the UE to re-use the stored ProSe IDs to perform discovery procedure.

8. The ProSe-enabled UE performs ProSe direct discovery based on received temporary ProSe ID of the target user, or EPC level discovery procedure based on received ProSe ID of the target user, e.g. direct discovery procedure defined in solution D8 can be used, and Proximity Request procedure defined in solution D4 with modification that discovering UE provides ProSe ID of "discoveree".

9-10. The ProSe-enable UE (3GPP layer) forwards the discovery result to upper layer (application layer), and stores the discovery result for preconfigured limited lifetime for future use, e.g. If other ProSe-enabled applications request the UE to perform discovery/communication, after performing step 2~6 to retrieve the ProSe ID/temporary ProSe ID of a target user identified by different application layer user ID, the UE finds target user’s ProSe ID/temporary ProSe ID has been stored together with valid discovery result, then the UE doesn’t need to perform duplicated discovery procedure as shown in step 8 for this target user.

The application on public safety UEs may use ProSe ID as application layer user ID directly, which allows them to discover each other without network assistance when they are out of network coverage.

6.4.3.3	Impact on existing entities and interfaces

# Editor's Note: To be completed.

6.4.3.4	Solution evaluation

Editor’s Note: The fulfilmentfulfilment of requirements in section 4.2 needs will be evaluated.

>>>End of Changes<<<<