SA WG2 Meeting #101	S2-140447

20 - 24 January 2014, Taipei, Taiwan	(revision of S2-140245)

Source:	NSN

Title:	Generic Pre-established MBMS bearer for GCSE

Document for:	 Approval

Agenda Item:	6.3

Work Item / Release:	GCSE_LTE / Rel-12

Abstract of the contribution: This paper suggests an improvement to Solution 1 to allow more effective use of pre-established bearer for group communication.

1. Introduction

This paper can be considered as part of: (H) Resource Efficiency of the study:

- Overall Analysis and evaluation with the selected solution(s)

A “pre-established eMBMS bearers” has been suggested as a solution for Group Communication (ref 6.1in TR 23.768). One salient feature is “fast GC setup”.

Obviously, a pre-established eMBMS bearer can avoid the eMBMS setup time which can be anywhere from one to two MCCH modification periods (~5 to 10sec). However, it is not practical to implement pre-established eMBMS bearer for many gcse group communications as the radio resources is not infinite.

Also as indicated in evaluation of solution 1, the use of per-established bearer may not be always useful (e.g, no designated gcse group users in the broadcast area.)

To maximize the usage of pre-established eMBMS bearer, one possibility is to define such pre-established eMBMS bearer as a generic pre-established eMBMS bearer which will allow temporary media establishment toward a GCSE group until its own eMBMS bearer (with own TMGI) is established.

3. Discussion

The essential requirement for using pre-established MBMS bearer is basically a re-use of the following 2 capabilities that have been stated already in this TR.

- Multiplexing of many GCSE group with one TMGI is already a requirement under conclusion (see section 8.1).

- UE capable to listening to 2 TMGIs that is related to the same group communication session is already suggested as one possible way to handle MBMS bearer with new QoS (see 6.7.1.3).

i.e,., a gcse communication session can be break away from the multiplex TMGI to its own dedicated TMGI.

2. Propose

The capability to allow the use of generic pre-established eMBMS bearer is added as a possible variation for option 1

*** begin change ***

# 6.x	Using pre-established eMBMS bearers as generic multicast delivery bearer

When the eMBMS bearer is pre-established for the purpose of serving multiple gcse groups, the following procedures are used:

- terminals interested in different group communication and waiting for possible broadcast session to start would all monitor this same generic pre-established eMBMS bearer. The identity for this bearer (e.g., TMGI) and identification of the relevant group communication (e.g., UDP port#) and associated decryption keys are done based on the application layer signalling between GCSE AS and UE.

- once a session for a given service starts:

-	its data is broadcast for some time on the generic bearer (and hence received by all terminals monitoring this bearer); the precise identity of the service in question is signalled in-band as part of the scheduled data. It is assumed that the terminals not authorized to receive the service content will not have the keys to decipher the content

-  eventually, for the service with session starting a separate broadcast bearer of its own is established, identified with its own identifiers, at which point the broadcasting of the data moves from the generic bearer to the service-specific bearer. (The receiving terminals will notice this from the appearance of the service in question on the MCCH channel.) This is to stop draining the batteries of terminals not addressed by the service in question.

- This way, only one such generic bearer per MCH channel (characterized by the same set of cells, same set of reserved subframes, and same modulation and coding used in transmission) needs to be kept pre-established and group-specific broadcast bearers can still be set up only on need basis.

*** NExt change ***

### 6.x.1	Solution evaluation

Editor's Note: The fulfilment of requirements in clause 4.2 needs will be evaluated.

Same as solution 6.1 with the following additions:

a) the pre-established area may be required to be in a much bigger area in order to account for multiple gcse groups with different geo-graphic requirements

b) GCSE AS needs to send data to both multicast delivery sessions (pre-established TMGI and dedicated TMGI) for transitioning period (i.e., allow UE to switch the DL reception with the dedicated TMGI).

c) UE needs to monitor both pre-established TMGI and dedicated TMGI and be able to switch the DL reception coming from the dedicated TMGI when available.

d) more UE battery consumption when the pre-established bearer is deployed with gcse groups being multiplexing into single TMGI

*** NExt change ***

##### 7.1.3.1.2 	Pre-established eMBMS bearers

In this scenario, the GCSE AS pre-establishes eMBMS session in certain pre-configured areas before the start of the group communication. When a UE originates a request for group communication from one of these areas, the pre-established eMBMS session is used for the DL traffic.

See sections 6.1.2.1, 6.x, and 6.5.2 for example (flows) using pre-established eMBMS bearers.

*** End of change ***