SA WG2 Meeting #103	S2-141997

19 - 23 May 2014, Phoenix, Arizona, USA	(revision of S2-141859)

Source:	NSN

Title:	Solution-y (RAN based) for “PS Attach after CS mobility”

Document for:	Approval

Agenda Item:	7.3

Work Item / Release:	CSPS_Coord/Rel-13

Abstract of the contribution: This PCR describes a RAN based proposal for solving mis-match for use case for PS Attach in GERAN after CS mobility.

# 1	Introduction

This PCR describes a solution for solving the use case -  PS Attach after CS mobility .

# 2	Proposal (all new texts)

The following text is proposed to be included in TR23.704.

>>>>>>>>>>>>>> Start of change <<<<<<<<<<<<<<<<<<<

## 7.x	Solution-x: Storing IMSI/PLMN info in RAN

### 7.x.1	Description of Solution

The starting point is that the UE has a valid NRI for one of the domain due to previous registration (e.g, the user has selected the operator manually earlier at 3G network for CS and disabled the PS function before moving to the 2G MOCN shared area).

The principle is that the BSC will query the registered domain for the IMSI/PLMN pair and this information is stored internally for CS/PS coordination purpose with the other domain.

The following figure shows a high level call flows with this solution:

Figure xx-1: CS/PS coordination using stored IMSI/PLMN info

- UE has valid NRI to point to the CS core network (CS CN 1).

- UE performs a LAU as it enters into a new LA.

- RAN checks the NRI and routes to the corresponding MSC (CS CN 1). It requests CS CN1 for IMSI/PLMN pair info with a new flag – IMSI/PLMN Request. As an option, RAN can only send this flag if it can determine that the UE is new in this area (i.e, checking the  “old LAI” against its internal LA/RA configuration).

- When LAU with the new flag and valid NRI is received by CS CN 1, CS CN 1 would send existing RANAP/BSSAP message Common ID with the IMSI of the subscriber and Selected PLMN ID (both IEs exist already in the standardized message). RAN then store this IMSI/PLMN info locally.

- CS CN 1 continues to process the LAU as normal and LAU accept is return to UE via RAN.

- UE starts the PS attach or RAU with no valid NRI.

- PS CN node selection is done e.g. with round robin as normally since there is no valid NRI in P-TMSI, and RAN node indicates need for rerouting as normally (RAF-Redirect Attempt Flag included).

- PS CN –X will based on Redirect Attempt Flag and missing IMSI in the initial message require CS/PS coordination as normally, and gives the IMSI to RAN node at the same time.

- When IMSI is received from PS CN-X as part of CS/PS coordination, then RAN node would based on the stored IMSI – Selected PLMN ID route the subscriber to the same PS operator (PS CN 1) as where the CS was routed

- Normal PS Attach or RAU procedure continues.

This solution works the same way if the PS is registered previously and CS domain is being registered later.

### 7.x.2	Evaluation of Solution

The solution impacts MSC, SGSN, BSC, and RNC.

For BSC/RNC:

- Sends IMSI/PLMN request flag to PS/CS Core (or optionally based on old LAI/RAI checking).

- Store the IMSI/PLMN info from CS/PS Core.

- Query the stored info to determine the proper CS/PS domain for registration.

- Remove the IMSI/PLMN info when it has determined that the UE has registered to both PS/CS domain, or based of implementation (e.g, pre-define period).

For MSC/SGSN:

- When receives a IMSI/PLMN Request flag, it needs to respond back to RAN via a Common ID message with IMSI/PLMN pair info.

Other considerations:

- This solution requires the RAN to store the IMSI/PLMN info when UE is in IDLE state.

- How long should this data be stored and what if the UE never performs the other domain registration via this same RAN is FFS.

- How this solution will work in conjunction with other solution(s) to solve other uses cases listed in this TR is FFS (e.g., handover, DTM, etc).

- How does this solution handle the case when RAU and LAU is being done at the same time is FFS.

>>>>>>>>>>>>>> End of change <<<<<<<<<<<<<<<<<<<