SA WG2 Meeting #106	S2-144534

17-21 November 2014, San Francisco, USA	rev of S2-144100

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.246 | CR | 0387 | rev | 4 | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Long Group Calls and New Cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA 2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GCSE_LTE |  |  |  |  |  |  | Date: |  | 2014-11-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Some Public Safety groups could be established for very long periods – easily spanning the introduction of new cells (especially incident related temporary cells) and the restart/reconfiguration of existing cells. Section 15.1.1 of TS 36.300 permits multiple physical/logical MCE/eNB configurations. With some of these (e.g. the “distributed MCE” architecture), the addition of a new eNB will result in the creation of a new MCE. This new MCE may be part of an MBMS Service Area (and possibly part of an MBSFN Area) where there is an ongoing MBMS session. To ensure that the (Public Safety) content is distributed in the commanded MBMS Service Area, and/or to ensure that all MCEs within the MBSFN Area receive the same MBMS sessions, it is necessary that something (e.g. the MME) informs the new MCE of the ongoing MBMS sessions in that MCE’s MBMS Service Areas. Note: Mobile TV channels would also be established for long periods. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MME behaviour to handle new MCEs in the service area is described. MCE set up and restart is referenced specifically to the M3 Setup Request/ Reset procedure over M3AP. MCE modification of its list of service areas is referenced specifically to the MCE Configuration Update procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Public Safety users would not receive mission critical broadcast media when camped on/connected to temporary cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR .... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The MBMS Restoration procedures for the MCE, which were introduced by CT 4 in Release 12 in section 15A.3 of TS 23.007, appear to provide the capability to deliver this functionality for the cases of Reset and M3 Setup. |  |  |  |  |  |  |  |  |

### *************** start of changes **************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service; Stage 1".

[3]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[4]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[5]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service".

[6]	3GPP TS 22.246: "Multimedia Broadcast/Multicast Service (MBMS) user services".

[7]	3GPP TS 26.346: "MBMS: Protocols and Codecs".

[8]	void.

[9]	void.

[10]	3GPP TS 25.346: "Introduction of the Multimedia Broadcast Multicast Service (MBMS) in the Radio Access Network".

[11]	3GPP TS 43.246: "Technical Specification Group GSM/EDGE Radio Access Network; Multimedia Broadcast Multicast Service (MBMS) in the GERAN".

[12]	3GPP TS 23.125: "Overall high level functionality and architecture impacts of flow based charging; Stage 2".

[13]	3GPP TS 23.003: "Numbering, addressing and identification".

[14]	3GPP TS 32.422: "Subscriber and equipment trace; Trace control and Configuration Management (CM)".

[15]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[16]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN Access".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[18]	3GPP TS 25.446: "MBMS synchronisation protocol (SYNC)".

[19]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[20]	IETF RFC 3810: "Multicast Listener Discovery Version 2 (MLDv2) for IPv6".

[21]	IETF RFC 4607: "Source-Specific Multicast for IP".

[22]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source specific Multicast".

[23]	3GPP TS 23.203: "Policy and charging control architecture".

[24]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[25]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[ab]	3GPP TS 23.007: “Restoration procedures”.

### *************** start of 2nd changes **************************

### 5.9.2	MBMS Control plane function

The MBMS control plane function is supported by MME for E-UTRAN access and by SGSN for UTRAN access. For SGSN functions, see clause 5.4.

One or more MBMS control plane functional entities are used in a PLMN.

MME supports the following functions in order to enable MBMS support for E-UTRAN:

-	Session control of MBMS bearers to the E-UTRAN access (including reliable delivery of Session Start/Session Stop to E-UTRAN);

-	Transmit Session control messages towards multiple E-UTRAN nodes;

-	When connected to multiple MCEs (Multi-cell/multicast Coordination Entity, see TS 36.300 [17]), the MME should filter the distribution of Session Control message to the MCEs based on the MBMS service area;

-	Provision of the list of MBMS Service Areas served by the MCE to the MME using M3AP Setup signalling;

-	Transmit Session Control messages towards the necessary E-UTRAN nodes to ensure the distribution of content from ongoing MBMS sessions:

-	if an MCE sets up (or restarts) an M3AP connection with the MME, by applying the MCE Failure procedure specified in TS 23.007 [ab]; or

-	if the MCE modifies the list of MBMS Service Areas it serves, by triggering the MCE Configuration Update procedure.

-	It provides an Sm interface to the MBMS GW function: it receives MBMS service control messages and the IP Multicast address for MBMS data reception from MBMS GW function over the Sm interface.

NOTE:	When a UE leaves or enters MBMS bearer coverage, the service continuity is handled by the Service Layer (in UE and network).

### ******** end of changes ****************