SA WG2 Meeting #110	S2-152658

06 - 10 July 2015, Dubrovnik, Croatia 	merge of S2-152310, S2-152384 and S2-152338

Source:	NTT DOCOMO, ZTE, China Mobile, Deutsche Telekom AG

Title:	Key Issue: Determination of ID of the Visited PLMN at IMS entities in HPLMN

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_V8/Rel-14

Abstract of the contribution: Proposes the key issues of providing VPLMN-ID to various IMS entities in the HPLMN during IMS registration and IMS session establishment and proposes a solution based on enhancement of Rx interface.

Discussion

There are several key issues where the VPLMN-ID or change of VPLMN-ID needs to be made available at IMS entities in the HPLMN:

-	During the registration, the S-CSCF and TAS need to be made aware about the VPLMN identity e.g. for charging purposes and to enable the TAS to subsequently perform communication barring supplementary services. In the LBO roaming solution, the PLMN information is statically configured in the P-CSCF and communicated in the P-Visited-Network-ID header to the HPLMN. However, this is not possible in the S8HR case where the P-CSCF is located in the HPLMN.

-	To handle UE undetectable IMS session Establishment the P-CSCF needs to be aware of the MCC of the VPLMN from where the UE initiates IMS session (see S2-152383). This information is needed at the P-CSCF at every IMS session establishment.

-	When an active IMS session is on going, the UE may perform inter-VPLMN handover (if this is supported by the HPLMN with the PGW connected to SGW of source-VPLMN and SGW of target-PLMN). The new VPLMN-ID should be made available to IMS entities in the HPLMN the next time IMS signalling occurs.

For solution we propose that the Rx interface is enhanced to provide PLMN ID change to P-CSCF. Gx interface already supports PCRF subscribing to PLMN change from the PGW (see TS 23.203, event triggers table).

Proposal

It is proposed to add the following text to the scope clause of TR 23.xxx:

***************** Start of changes **********************

5.x	Key Issue x  Determination of the ID of the visited PLMN at IMS Entities in HPLMN

5.x.1	Description

The ID of the visited PLMN needs to be made available at IMS entities in the HPLMN at different instances:

-	During the registration, the S-CSCF and TAS need to be made aware about the ID of the visited PLMN e.g. for charging purposes and to enable the HPLMN to subsequently perform roaming registration restriction or communication barring supplementary services.

-	To handle non UE detectable IMS emergency session establishment the P-CSCF needs to be aware of the MCC of the VPLMN from where the UE initiates IMS session. This information is needed at the P-CSCF at every IMS session establishment.

-  The HPLMN IMS entities need to be aware of the ID of the visited PLMN at session set up in order to populate the charging records.

***************** Next change **********************