SA WG2 Meeting #S2-111	S2-153016

19 - 23 October 2015, Chengdu, P.R. China

3GPP TSG-CT WG1 Meeting #93 	C1-152714

# Vancouver (Canada), 17-21 August 2015

Title:	Reply LS on Handover procedures without the dynamic PGW ID

Response to:	LS (C4-151055) on Handover procedures without the dynamic PGW ID from CT4

Release:	Rel-13

Work Item:	TEI13

Source:	3GPP TSG CT WG1

To:	CT4

Cc:	SA2

Contact Person:

#### Name:	Youyang Yu

Tel. Number:

E-mail Address:	yuyouyang@huawei.com

Attachments:

1. Overall Description:

CT1 thanks CT4 for the LS on Handover procedures without the dynamic PGW ID. CT1 would like to provide the following feedback on the questions asked by CT4.

“If NBM is used and if the UE receives from the ePDG an IKE_AUTH response message containing a Notify Payload with a Private Notify Message Type PDN_CONNECTION_REJECTION as specified in subclause 8.1.2 that includes an IP address information in the Notification Data field, the UE shall not attempt to re-establish this PDN connection while connected to the current ePDG and the UE shall close the related IKEv2 security association states.

…

If NBM is used for IP mobility management and the UE performs an initial attach, the UE configures a home address based on the address information from the CFG_REPLY Configuration Payload. Otherwise, if NBM is used and the UE performs a handover attach, the UE continues to use its IP address configured before the handover, if the address information provided in the CFG_REPLY Configuration Payload does match with the UE's IP address configured before the handover. If the UE's IP address does not match with the address information of the CFG_REPLY Configuration Payload, the UE shall configure a new home address based on the IP address information contained in the INTERNAL_IP4_ADDRESS or INTERNAL_IP6_SUBNET attribute of the CFG_REPLY Configuration Payload. In the latter case, the IP address preservation is not possible.”

Regarding the above two paragraphs from TS 23.402 subclause 7.2.2, questions to CT1:

Q1:  For the first paragraph, CT4 kindly asks CT1 to indicate whether the UE is allowed to establish another PDN connection without the IP address information in the Notification Data field to the same APN?

CT1 Answer to Q1: Yes, the UE is allowed to establish another PDN connection without the IP address information in the Configuration Payload field to the same APN. This is also aligned with the requirements stated in stage 2 in TS 23.402. Please find the attached agreed C1-15xx.

Q2:  For the second paragraph, CT4 is of the understanding that it is allowed for the network to continue the handover procedure without IP address preservation. CT4 kindly asks CT1 to confirm if this understanding is correct.

CT1 Answer to Q2: If PMIP is used on S2b, the network is allowed to continue the handover procedure without IP address preservation as defined in stage 2 (TS 23.402 subclause 4.1.3.2.3. bullet c), f) and NOTE1). The UE hehavior defined in the second paragraph is to satisfy this stage 2 specified requirement.

For the GTP case, the UE should receive the PDN_CONNECTION_REJECTION with an IP address information in the Notification Data field as defined in the first paragraph. The UE should not continue the Handover procedure.

Q3. CT4 also asks CT1 to clarify, what the UE does with the IP address allocated on the 3GPP access when

- The non-3GPP access network rejects the handover request

- The non-3gpp access network accepts the handover attach but without IP address preservation by providing a different IP address

CT1 Answer to Q3: In case a, the UE shall not attempt to re-establish the PDN connection with the same IP address allocated on 3GPP side while connected to the current ePDG and the UE shall close the related IKEv2 security association states.

In case b, the UE shall use the new IP address allocated on non-3GPP access to replace the IP address allocated on the 3GPP access and keep the related IKEv2 security association states.

2. Actions:

To SA2  group.

ACTION: 	CT1 kindly asks CT4 to take the above answers into account.

3. Date of Next TSG-CT WG1 Meetings:

TSG CT WG1 Meeting 94	12-16 October 2015	Belgrade (Serbia)

TSG CT WG1 Meeting 95	16-20 November 2015	Anaheim, CA (USA)