SA WG2 Meeting #S2-111	S2-153020

19 - 23 October 2015, Chengdu, P.R. China

3GPP TSG-CT WG1 Meeting #93 	C1-153277

# Vancouver (Canada), 17-21 August 2015

Title:	Reply LS on handover policy for a terminal reaches the handover collision between from PS to CS and from LTE to WiFi at the same time

Response to:	LS (RILTE 45 Doc 111) on handover policy for a terminal reaches the handover collision between from PS to CS and from LTE to WiFi at the same time

Release:	Rel-13

Source:	3GPP TSG CT WG1

To:	GSMA NG RILTE, 3GPP TSG SA WG2

Cc:

Contact Person:

#### Name:	Yue Song

E-mail Address:	songyue (at) chinamobile (dot) com

Attachments:

1. Overall Description:

CT1 thanks GSMA NG RILTE for the LS on handover policy for a terminal reaches the handover collision between from PS to CS and from LTE to WiFi at the same time. CT1 would like to provide the following feedback:

Q1: Is the terminal required to support the above-described handling of a handover collision if supporting both abilities of proceeding the handover to CS and proceeding the handover to WiFi?

CT1 Answer to Q1: CT1 cannot identify the requirements, however the procedures for the UE on handling handover collision are described in 3GPP TS 24.237, subclause 12.2.4.3, as:

If the SC UE engaged in one or more IMS sessions receives a handover command from the eNodeB (as described in 3GPP TS 36.331 [62]) or from the NodeB (as described in 3GPP TS 25.331 [61]) when the UE is engaged in moving the PDN connection from an 3GPP access to a non-3GPP access as described in 24.302 [84], the UE shall either:

1)    accept the handover command and abort the move to the non-3GPP access; or

2)    continue with the move to the non-3GPP access and ignore the handover command.

It’s up to the terminal implementation to choose which option to proceed with.

Q2: Is it possible for the network to instruct the terminal with which handover to proceed? Is there any related terminal configuration or signalling during the handover?

Q3: Is it possible for the network to instruct the terminal to proceed the handover for different types of calls according to operator’s policy? For example, to handover a voice+video call or high quality voice-only call to WiFi while to handover a standard quality voice call to CS.

CT1 Answer to Q2 and Q3: 3GPP CT1 has not defined relevant terminal configuration or signaling indication, CT1 is not aware of any requirements to do so.

Q4: For the roaming scenario, should the HPLMN or VPLMN instruct the terminal with which handover to proceed?

CT1 Answer to Q4: CT1 considers Q4 to be outside the remit of CT1.

2. Actions:

To GSMA NG RILTE group.

ACTION: 	CT1 kindly asks GSMA NG RILTE to take the above answers into account.

To 3GPP TSG SA WG2

ACTION: 	CT1 kindly asks 3GPP TSG SA WG2 to evaluate the problem and to provide guidance on possible solutions.

3. Date of Next TSG-CT WG1 Meetings:

TSG CT WG1 Meeting 94	12-16 October 2015	Belgrade (Serbia)

TSG CT WG1 Meeting 95	16-20 November 2015	Anaheim, CA (USA)