3GPP TSG-SA WG2 Meeting #111			S2-153678

Chengdu, P.R. China, 19-23 October 2015

Title:	[DRAFT] LS on failure to setup the resources for VoLTE MO call

Response to:	-

Release:

Work Item:	SAES

Source:	SA WG2

To:	3GPP CT WG1

Cc:	-

Contact Person:

#### Name:	Antoine Mouquet

Tel. Number:

E-mail Address:	Antoine point Mouquet at Orange point com

Attachments:	None

1. Overall Description:

SA2 discussed how to avoid VoLTE call setup failure when under weak LTE coverage (esp. indoor).

SA2 agreed that for a "Voice centric" UE registered in IMS for voice, the failure to setup the resources for an IMS voice session (e.g. QCI-1 bearer rejected by the eNB) should trigger the UE to perform a new call attempt using the CS domain.

SA2 further discussed the specific SIP error codes that are sent by IMS in this case and whether they can be used to trigger the UE using the CS domain.

Furthermore SA2 also agreed that under this circumstance, the UE should not be required to attempt combined TA/LA Update.

SA2 concluded that this should be regarded as an error case and therefore handled by stage 3.

2. Actions:

To CT1 group.

ACTION: 	 SA2 would like CT1 to address this issue by considering the above and performing necessary updates to their specification(s).

3. Date of Next SA WG2 Meetings:

TSG-SA WG2 Meeting #112	17-21 November, 2015		Anaheim, US

TSG-SA WG2 Meeting #113	25-29 January, 2016		TBD, North America