SA WG2 Meeting #112	S2-153954

16-20 November 2015, Anaheim, USA

Source:	Deutsche Telekom

Title:	Key issue: 3GPP architecture impacts to support network slicing roaming

Document for:	Approval

Agenda Item:	NextGen/6.10

Work Item / Release:	NextGen/ Rel-14

Abstract of the contribution: This contribution propose to add a new key issue on Network Slicing roaming

# Introduction

It is anticipated that Network Slicing will be an important component of the Next Generation network. Therefore Network Slicing roaming scenarios need to be studied. The following is an extract from the SA1 SMARTER TR 22891 on Network Slicing roaming:

“Network slicing allows the operator to provide dedicated logical networks with customer specific functionality, without losing the economies of scale of a common infrastructure. As such a big variety of use cases with diverging requirements can be fulfilled (see Use Case 5.2).

To guarantee a consistent user experience and support of services in case of roaming, slices composed of the same network functions should be available for the user in the VPLMN. To keep the operational effort for the operator low a few network slices that cover most of the use cases could be agreed among operators.

Other network slices can be composed by operators as needed. They will consist of mainly 3GPP defined functions but could also include proprietary functions that are provided by different operators or 3rd parties. Configuration of network slices and provision of proprietary functions will be based on agreements between operators.

The roaming users may need to be assigned to a network slice in the VPLMN that provides the same or similar functionality as the HPLMN slice. Different criteria can be used to associate UEs including pre-5G UEs with a particular slice. ”

# Proposal

It is proposed to add the following Key Issue to the TR 23.xyz “Study on Architecture for Next Generation System”.

* * * Start of changes * * * *

# 5	Key Issues and Solutions

## 5.x	Key issue on 3GPP architecture impacts to support network slicing roaming

Editor's Note: This clause will identify key architectural issues and the corresponding candidate solutions during the design of the next generation system architecture.

### 5.x.1	Description

Network slicing enables the operator to provide optimized solutions for different market scenarios which demands diverse requirements. A consistent user experience and support of services in case of roaming needs to be enabled.

Solutions for this key issue will study:

-	Network Slicing Roaming as per requirements defined in TR 22.891 and in normative stage 1 specifications (when available).”.

-  Network Slicing Roaming scenarios where slices composed of the same network functions are available for the user in the VPLMN.

-  how to enable operators to define and identify network slices with common functionality available for both , home and roaming users.

-  how to compose network slices from 3GPP defined functions as well as from proprietary 3rd party or other    operator provided functions.

-  how to associate the user in a VPLMN to the network slice that provides the required functionality for this user , e.g. the same functionality as the associated network slice in the HPLMN.

-  how to associate the user in a VPLMN to a default network slice if no corresponding slice has been defined by the VPLMN.

* * * End of Changes * * * *