SA WG2 Meeting #113	S2-160444

25 - 29 January 2016, Saint Kitts, KN	(revision of S2-16xxxx)

Source:	Deutsche Telekom AG

Title:	Solution for Key Issue 2 (Emergency Calls Routed via IMS Service Domain)

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This document proposes potential solution to SeDoC key Issue 2 (Emergency Calls Routed via IMS Service Domain).

# Discussion

Current ICS specification assumes emergency calls originated in 2/3G CS RAN will be established via legacy means (MSC Call Control only, ISUP/TDM rsp. BICC/RTP links towards PSAP) w/o usage of IMS Service Domain.

This is stated in 3GPP TS23.292 as follows:

IMS control of all other types of calls, including emergency calls that utilise TS12, are outside the scope of the present release of this specification

It is the opinion of the authors of this paper that the reason for keeping emergency calls in the CS domain is because at the time when ICS was specified IMS emergency calls were still not specified (this is a Release 9 feature): there are no technical reasons why in an ICS-enabled network emergency calls placed while the UE is in the CS domain cannot be routed to the E-CSCF.

The proposal is to remove this restriction within TS23.292.

# Proposal

It is proposed to add the following solution to TR 23.719

Add Text

## 6.X Solution X: Emergency Calls Routed via IMS Service Domain

Current ICS specification assumes emergency calls originated in 2/3G CS RAN will be established via legacy means (MSC Call Control only, ISUP/TDM rsp. BICC/RTP links towards PSAP) w/o usage of IMS Service Domain.

This is stated in 3GPP TS23.292 as follows:

IMS control of all other types of calls, including emergency calls that utilise TS12, are outside the scope of the present release of this specification

It is the opinion of the authors of this paper that the reason for keeping emergency calls in the CS domain is because at the time when ICS was specified IMS emergency calls were still not specified (this is a Release 9 feature): there are no technical reasons why in an ICS-enabled network emergency calls placed while the UE is in the CS domain cannot be routed to the E-CSCF.

The proposal is to remove this restriction within TS23.292.

Editor’s note: Solution architecture that supports SeDoC emergency calls will be provided in a separate contribution.

### 6.X.3 Solution Evaluation

Editor’s Note: Evaluation of this solution is FFS.

End of added Text