SA WG2 Meeting #113	S2-160712

25 - 29 January 2016, Saint Kitts, KN	(revision of S2-160461)

Source:	Deutsche Telekom AG

Title:	Key issue 1 update proposal on CS data support by SeDoC architecture

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_SeDoc / Release 14

Abstract of the contribution: This document proposes to add to key issue 1  CS data support by SeDoC architecture.

# Discussion

There is still a vast amount of CS data only devices e.g. for certain M2M applications deployed and cannot be replaced any time soon or with reasonable cost.

Unless there is a solution for CS data handled via the IMS domain, simplification of the network implementation (and hence reduction in operational costs) will not be possible.

Therefore it is proposed to add CS data support by SeDoC architecture to key issue 1.

# Proposal

It is proposed to add the following key issue to TR 23.719

Add Text

## 5.1	Key Issue 1- Transition to Single Service Profile in IMS

### 5.1.1	Description

Current solutions for IP based voice services require the network to support two types of service profiles for different users, an IMS based profile for services supported in an MMtel TAS (for IMS enhanced users) and a HLR based profile for legacy services (for non-IMS enhanced users).   Even with ICS enhanced networks there is still need to support both user profiles (for different segments of users); although ICS enhanced subscribers use the IMS profile for the MSC services simulated in the IMS domain, there are still some circumstances that require the use of the HLR profile.  Careful configuration and management is needed to ensure customer service profiles remain consistent between the two domains even if both profiles are not expected to be used simultaneously.

Unless a single service profile is used for all users (in the IMS domain), simplification of the network implementation (and hence reduction in operational costs) will not be possible.

### 5.1.2	Architectural Requirements

The following architectural requirements are defined:

- Service profile for all users shall be in the IMS domain only.

- Service logic execution for all users shall be in the IMS domain only.

- CS data shall be supported by the SeDoC architecture.

End of added Text