SA WG2 Meeting #114	S2-161543

11 - 15 April 2016, Sophia Antipolis, France	(revision of S2-16xxxx)

Source:	LG Electronics

Title:	Solution on Key issue #18: UE Functionality to support migration scenarios

Document for:	Approval

Agenda Item:	6.10.18

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: It considers migration scenarios and the functionality of the UE which are required to support them.

Discussion

Figure 1 represents architectures that are used to describe the migration scenarios.

- EPC connected via eLTE	(b) NextGen Core connected via New RAT

Figure 1. The baseline architectures

The following terminology is used for this solution.

- R-15 E-UTRAN: E-UTRAN upgraded to interface with the NextGen Core.

- NS1: Reference point between the New RAN and the NextGen Core.

- NUu: Reference point between the New RAN and the New RAT capable UE. It depends on RAN WG’s definition.

- N-RAN: New RAN. It depends on RAN WG’s definition.

- L-NAS: LTE NAS

- L-AS: LTE AS

- NG-NAS: NextGen NAS

- NR-AS: New RAT AS. It depends on RAN WG’s definition.

The followings are assumed for this solution.

- Pre-R15 LTE UE connects to EPC via R-15 E-UTRAN

- R-15 eLTE UE connects to EPC via R-15 E-UTRAN

- R-15 New RAT (NR) capable UE connects to NextGen Core via R-15 New RAN (N-RAN)

Figure 2 shows the migration scenario which R-15 eLTE UE connects to NextGen Core via R-15 E-UTRAN.

From the perspective of UE,

- It has only LTE AS while having both LTE NAS and NG-NAS.

- In the baseline architecture as depicted in Figure 1, it can connect to EPC via E-UTRAN.

- It accesses to E-UTRAN in order to connect to NextGen core.

  - It basically needs R-15 NG-NAS, and has some additional NAS functionality, f(L), in compared to R-15 NR capable UE as depicted in Figure 1. It supports the interaction with LTE AS.

  - It basically needs R-15 L-AS, and has some additional AS functionality, f(N), in compared to R-15 eLTE UE as depicted in Figure 1. It supports the interaction with NG-NAS.

Figure 2. Migration scenario with eLTE UE

Figure 3 shows the migration scenario which R-15 eLTE/NR capable UE connects to NextGen Core via R-15 E-UTRAN.

From the perspective of UE,

- It has both LTE AS and NR-AS, also has both LTE NAS and NG-NAS.

- In the baseline architecture as depicted in Figure 1, it can connect to EPC via E-UTRAN. Also, it can connect to NextGen core via N-RAN.

- In addition, it accesses to E-UTRAN in order to connect to NextGen core.

  - It basically needs R-15 NG-NAS, and has some additional NAS functionality, f(L), in compared to R-15 NR capable UE as depicted in Figure 1. It supports the interaction with LTE AS.

  - It basically needs R-15 L-AS, and has some additional AS functionality, f(N), in compared to R-15 eLTE UE as depicted in Figure 1. It supports the interaction with NG-NAS.

Figure 3. Migration scenario with eLTE/NR capable UE

Finally, Figure 4 shows one of migration path which an eLTE/NR capable UE connects to EPC via N-RAN, then it is out of FS_NextGen scope.

From the perspective of UE,

- It has both LTE AS and NR-AS, also has both LTE NAS and NG-NAS.

- In the baseline architecture as depicted in Figure 1, it can connect to EPC via E-UTRAN. Also, it can connect to NextGen core via N-RAN.

- In addition, it accesses to N-RAN in order to connect to EPC.

  - It basically needs R-15 L-NAS, and has some additional NAS functionality, f(N), in compared to R-15 eLTE UE as depicted in Figure 1. It supports the interaction with NR-AS.

  - It basically needs R-15 NR-AS, and has some additional AS functionality, f(L), in compared to R-15 NR capable UE as depicted in Figure 1. It supports the interaction with L-NAS.

Figure 4. Migration scenario with eLTE/NR capable UE connecting to EPC

Proposal

It is proposed to add the following changes to TR 23.799.

* * * * Start of 1st Change * * * *

All new text to TR 23.799

6.18	Solutions for Key Issue 18: Interworking and Migration

6.18.X	Solution 18.X: UE Functionality to support migration scenarios

This is a solution for key issue 18 on Interworking and Migration, and in particular it addresses the migration scenarios and the functionality of the UE which are required to support them.

6.18.X.1	Architecture description

Figure 6.18.X.1-1 represents architectures that are used to describe the migration scenarios.

- EPC connected via eLTE	(b) NextGen Core connected via New RAT

Figure 6.18.X.1-1. The baseline architectures

The following terminology is used for this solution.

- R-15 E-UTRAN: E-UTRAN upgraded to interface with the NextGen Core.

- NS1: Reference point between the New RAN and the NextGen Core.

- NUu: Reference point between the New RAN and the New RAT capable UE. It depends on RAN WG’s definition.

- N-RAN: New RAN. It depends on RAN WG’s definition.

- L-NAS: LTE NAS

- L-AS: LTE AS

- NG-NAS: NextGen NAS

- NR-AS: New RAT AS. It depends on RAN WG’s definition.

The followings are assumed for this solution.

- Pre-R15 LTE UE connects to EPC via R-15 E-UTRAN

- R-15 eLTE UE connects to EPC via R-15 E-UTRAN

- R-15 New RAT (NR) capable UE connects to NextGen Core via R-15 New RAN (N-RAN)

6.18.x.2	Function description

Figure 6.18.X.2-1 shows the migration scenario which R-15 eLTE UE connects to NextGen Core via R-15 E-UTRAN.

From the perspective of UE,

- It has only LTE AS while having both LTE NAS and NG-NAS.

- In the baseline architecture as depicted in Figure 6.18.X.1-1, it can connect to EPC via E-UTRAN.

- It accesses to E-UTRAN in order to connect to NextGen core.

  - It basically needs R-15 NG-NAS, and has some additional NAS functionality, f(L), in compared to R-15 NR capable UE as depicted in Figure 6.18.X.1-1. It supports the interaction with LTE AS.

  - It basically needs R-15 L-AS, and has some additional AS functionality, f(N), in compared to R-15 eLTE UE as depicted in Figure 6.18.X.1-1. It supports the interaction with NG-NAS.

Figure 6.18.X.2-1. Migration scenario with eLTE UE

Figure 6.18.X.2-2 shows the migration scenario which R-15 eLTE/NR capable UE connects to NextGen Core via R-15 E-UTRAN.

From the perspective of UE,

- It has both LTE AS and NR-AS, also has both LTE NAS and NG-NAS.

- In the baseline architecture as depicted in Figure 6.18.X.1-1, it can connect to EPC via E-UTRAN. Also, it can connect to NextGen core via N-RAN.

- In addition, it accesses to E-UTRAN in order to connect to NextGen core.

  - It basically needs R-15 NG-NAS, and has some additional NAS functionality, f(L), in compared to R-15 NR capable UE as depicted in Figure 6.18.X.1-1. It supports the interaction with LTE AS.

  - It basically needs R-15 L-AS, and has some additional AS functionality, f(N), in compared to R-15 eLTE UE as depicted in Figure 6.18.X.1-1. It supports the interaction with NG-NAS.

Figure 6.18.X.2-2. Migration scenario with eLTE/NR capable UE

Editor's Note: Additional migration scenarios are FFS. It may depend on the work of RAN WG.

6.18.x.3	Solution evaluation

Editor's Note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * End of Changes * * * *