SA WG2 Meeting #115	S2-163005

23 - 27 May 2016, Nanjing, P.R. China	(revision of S2-16xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei

Title:	Solution for Session Continuity

Document for:	Approval / Discussion

Agenda Item:	6.10.6

Work Item / Release:	FS_NextGen / 14

Abstract of the contribution: This contribution proposes solutions for session continuity when efficient user plane path (re)selection needs to be supported.

# Introduction

Key issue #5 Session and Service continuity states the following requirements:

“In order to address the specific needs of different applications and services, the next generation system architecture for mobile networks should support different levels of data session continuity or service continuity based on the Mobility on demand concept of the Mobility framework defined in Key Issue 3. For example, the next generation system may do one of the following on per session basis for the same UE:

Editor's Note: The first sentence in the paragraph above may need to be further refined due to the simultaneous change made by the pCRs in S2-161278 and S2-161297.

-	support session continuity.

-	not support session continuity.

-	support service continuity when session continuity is not provided.”

The definitions for session, session continuity states the following:

“PDU Session: Association between the UE and a data network.

PDU Session of IP Type: Association between the UE and an IP data network.

Editor’s Note: PDU session is comparable to a single-stack PDN connection in EPS. A dual-stack PDN connection in EPS corresponds to two PDU sessions. This is a working definition and FFS.

Session Continuity: The continuity of a PDU session. For PDU session of IP type “session continuity” implies that the IP address is preserved for the lifetime of the PDU session.”

Enabling (re)selection of efficient user plane paths, especially the following issues:

-	Reselection of user-plane path between a UE attached to the mobile network and communication peers outside of the mobile network (e.g. Internet hosts) when the previous path becomes inefficient.

-	Minimising impact to the user experience (e.g. minimisation of interruption time and loss of packets) when changing the anchoring point for some or all packet data connections of a UE.

In order to specify the solution for session continuity and efficient user plane path (re)selection, it is first essential to understand the scenarios for which session continuity should be supported. So, in this paper, we have identified some possible scenarios (in other words, network topology based on services offered) that should be considered and propose solutions for scenarios for which session continuity should be supported.

# Analysis

NextGen UE can be expected to support and receive diverse services - both services offered centrally and services offered distributed. In some cases, the same UE may receive diverse services simultaneously. This plays a role in determining the need to specify a solution for session continuity. Based on the services supported at a given time, we have identified the following scenarios that should be considered in the context of session continuity:

# Scenario 1

# Description:

- UE is accessing centralized services (e.g. internet browsing or access to an application server in a central location). It supports a local mobility anchor and centralized IP anchor. In case of services provided centrally (e.g. internet services), the central IP anchor need not be relocated.  Thus, support for session continuity (i.e. IP address preservation) when the UE moves from one IP anchor to another IP anchor is not seen necessary for such a scenario. Session continuity should be supported when the UE moves from one mobility anchor to another mobility anchor (i.e. similar to S-GW relocation in LTE network).

Figure 1: Scenario 1 - UE accessing centralized services

# Scenario 2

# Description:

- UE has access to two services – centralized services (access to internet) via IP anchor, distributed services (e.g. cached content of a CDN, content distribution network) via UPGW with the local mobility anchor. The IP anchor is assumed to be located in a centralized location. Note, that in this scenario only one host IP address that is centrally anchored has been assigned to the UE. The IP address is allocated and advertised to outside world by the central IP anchor. So, the UPGW at the Mobility Anchor 1 examines the IP packets (i.e. destination IP address) sent by the UE and decides whether to encapsulate the IP packet within a tunnel and transmitted towards the central IP anchor or it should be transmitted (e.g. within a tunnel) towards the local application server (e.g. CDN).

- The IP anchor is assumed to be located in a centralized location thus support for session continuity (i.e. IP address preservation) when the UE moves from one IP anchor to another IP anchor is not seen necessary for such a scenario. Session continuity should be supported when the UE moves from one mobility anchor to another mobility anchor (i.e. similar to S-GW relocation in LTE network).

- Network decides how the traffic is routed, whether it is routed towards the Application Server (AS) in the local network or towards the IP anchor targeting Data Network. Thus, the UE is unaware of the traffic redirection in this scenario.

- Charging, LI enforcement for traffic directed towards AS in the local network should happen in the user plane processing function collocated with the mobility anchor.

Figure 2: Scenario 2 - UE accessing both local and centralized services

# Scenario 3

# Description:

- UE has access to a locally distributed service via local mobility anchor and local IP anchor. It is assumed that the application server is running in an edge cloud (e.g. V2X services that requires lowest possible latency). So, when the UE moves, mobility anchor, IP anchor and application server instance are all relocated (i.e. application instance moves along with the UE).

Figure 3: Scenario 3 - UE accessing local services

Note 1: In case, the UE supports IPv6 multi-homing, the UE could receive 2 IPv6 prefixes - one from a local IP anchor and another from a centralized IP anchor (IETF RFC 4177). Thus combination of scenarios 1 and 3 can be envisioned for the same PDU session. The main difference between scenario and multi-homing scenario for a single PDU session is that the UE decides when the traffic is intended for central IP anchor or local IP anchor (e.g. by choosing the appropriate source IP address).

Note 2: the figures 1, 2 and 3 show only the initial (before HO) and final states (after HO). Intermediate states (User plane path during the HO) are not shown here.

Based on this analysis, we believe that session continuity solutions should be specified for scenario 2, scenario 3 and potential variants.

# Proposal

It is proposed to add the following solution to the TR 23.799 “Study on Architecture for Next Generation System” V0.3.0.

* * * Start of changes * * * * (all new text)

# 6	Solutions

## 6.x	Solutions for Key Issue 5, 6: Enabling (re)selection of efficient user plane paths, Support for session and service continuity

### 6.x.y	Solution x.y: Solution to support Session continuity to enable (re)selection of efficient user plane paths

The Solution applies to Key issue 5: Enabling (re)selection of efficient user plane paths, especially the following issues:

-	Reselection of user-plane path between a UE attached to the mobile network and communication peers outside of the mobile network (e.g. Internet hosts) when the previous path becomes inefficient.

-	Minimising impact to the user experience (e.g. minimisation of interruption time and loss of packets) when changing the anchoring point for some or all packet data connections of a UE.

The Solution applies to Key issue 6: Support for session and service continuity

-	The types of sessions to be considered in the context of session continuity;

-	The level of session continuity to be supported depending on e.g., the type of service such as broadband, group communications, mission critical communications, etc.;

Editor’s note: This solution assumes support for IP services. Extending this for supporting non-IP (e.g. Ethernet) services is FFS.

#### 6.x.y.1	Architecture description

This solution assumes that the Next Generation network user plane will include two functions.

-	One UP function providing the following functionality: uplink classifier functionality (UL CL) that allows steering local traffic to local services and/or central UP function towards central services, i.e. the ability to examine the destination IP address of IP packets sent by the UE and determine how the packet should be routed. In addition, packet routing, packet forwarding, traffic offloading to a local network as well as charging, LI and bitrate enforcement for locally offloaded traffic.

-	Another UP function providing IP anchoring functionality.

Figure 6.x.y.1-1 User plane Architecture for the solution

The operator uses configuration or policies to determine which flows are to be routed to/from the local network. Such configuration or policies are applied in the “UP function UL-CL” using uplink classification based on filtering rules.  For downlink, the operator configures the network routing such that only legitimate traffic from the local server may pass via the UP function local IP point of presence.

The operator may use a dedicated IP address range for the set of UEs that need to be reachable locally. The local service network is configured such that this address range can be used for local downlink traffic.

Editor’s note: How the dedicated IP address range work for roaming UEs is FFS.

In the case of mobility, session continuity is provided using some form of tunnelling, both for the central IP anchor point, and for the local IP point of presence.

#### 6.x.y.2	Function description

##### 6.x.y.2.1 	General

UE has access to centralized services (access to internet) via the IP anchoring UP function, distributed services (e.g. cached content of a CDN, content distribution network) via the UL-CL UP function. The IP anchoring UP function providing access to the centralized services is assumed to be located in a more centralized location than the UL-CL UP function providing access to the distributed services. Note, that in this scenario only one host IP address that is centrally anchored has been assigned to the UE.

The IP anchor is assumed to be located in a centralized location. Note, that in this scenario only one host IP address that is centrally anchored has been assigned to the UE.. So, the UL-CL UP function with the local IP point of presence examines the IP packets (i.e. destination IP address) sent by the UE and decides whether to transmit the PDU within a tunnel to the central IP/PDU session anchor or it should be transmitted to the local application server (e.g. CDN) (e.g. within a tunnel).

Another option for IPv6 is that IPv6 multi-homing is used with one IPv6 address assigned to the UE for the local services and one IPv6 address assigned for the more central services.   In case, the UE supports IPv6 multi-homing, the UE could receive 2 IPv6 prefixes - one from a local IP anchor and another from a centralized IP anchor (IETF RFC 4177 or RFC 4191). The main difference between the scenario described above and multi-homing scenario for a single PDU session is that in case of multi-homing, rules configured into the UE decides when the traffic is intended for central IP anchor or local IP anchor (e.g. by choosing the appropriate source IP address). RFC 6724 specifies basic algorithms how IPv6 host should select IP addresses and RFC 5014 is an example of the IPv6 socket API for source address selection. RFC 4191 includes mechanisms by which an operator can configure rules into the UE to influence how the selection of the address.

Editor’s note: How IPv4 and IPv6 traffic can be supported in parallel for the same UE if IPv6 multihoming is used is FFS

Following are the main characteristics of the solution:

- The possibility of having both a local IP point of presence (via the UL-CL UP function) and a central IP point of presence for a given session using the IP address allocated for by the central IP anchoring UP function. The local IP point of presence is applicable only for a well-defined set of traffic flows under operator control, while the central IP point of presence is used by default. The operator configures the local networks such that only the well-defined set of traffic flows can access the local IP point of presence.

- An operator controlled uplink classification functionality within the UL-CL UP function determines which uplink flows to direct towards the local IP point of presence when local IP point of presence is applied by the operator.

- The network decides how the traffic is routed i.e. whether it is routed towards the local network or towards the more centrally located UP function and Data Network. Thus, the UE is not aware of the traffic redirection in this scenario.

- The core network control plane is responsible for the setup, configuration and release of the user plane functions mentioned above, based on operator policies, UE preferences and subscription information, on a per session basis.

.

##### 6.x.y.2.2	Mobility aspects

Figure 6.x.y.2-1: UL-CL UP function relocation for the same Local Network and existing PDU session.

When the UE is relocated from one UL-CL UP function 1 to UL-CL UP function 2, the existing session continues to be supported by the existing PDU session via the original local network-1 until termination. Session continuity is maintained from the UE’s perspective for the PDU session as the central UP function acting as central IP anchor and the Local Network-1 remains the same when the UE is relocated from UL-CL UP function 1 to UL-CL UP function 2.

Alternatively, the UE’s session towards the local network may be relocated from local network 1 to local network 2 in addition to UP function 1 relocation. Application session relocation may be necessary which is out of scope for 3GPP. This is shown in the figure below.

Note: 	During this relocation, if the existing session moves from local network 1 to local network 2, then application context information or application instance may need to be transferred from Local network 1 to Local network 2 in such a way that there is no interruption for the application running in the UE. If this is not possible for a certain application session, then the existing session continues to be supported via Local network-1 until termination.

Editor’s note: Impacts to on-going TCP sessions when changing from one UP function to another are FFS.

Figure 6.x.y.2-2: UL-CL UP function relocation along with relocation from Local Network 1 to Local Network 2

##### The IP anchoring UP function is maintained for the PDU session as the UE moves, i.e. session continuity is provided for the PDU session as the IP anchoring  UP function remains the same. Thus, this solution applies also if the UP function is relocated.

#### 6.5.x.3	Solution evaluation

Editor's Note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.