SA WG2 Meeting #116-BIS	S2-164976

29 Aug – 2 Sep 2016, Sanya, CN	(revision of S2-164455)

Source:	ZTE

Title:	SGW CP Partitioning

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes to add SGW CP partitioning function into TS 23.214.

1	Proposal

It is proposed to agree the following text in the TS 23.214.

/******************** Start of Changes ***************/

4.3.x	SGW-C Partitioning

If the SGW-U service area is smaller than the SGW-C service area, the SGW-C can be partitioned into multiple SGW-C partitions. Each of the SGW-C partition is aligned with the corresponding SGW-U service area. The MME treats the SGW-C partition as legacy SGW. The Sxa reference is not partitioned, i.e. there is only one Sxa reference point between one SGW-C and one SGW-U. This ensures that the TAI List allocated by the MME will only contain TAs that have connectivity to the SGW-U function serving the UE in case the MME uses the service area of the SGW-C partition when constructing the TAI List (and it cannot be assured that there is “full mesh” IP connectivity between the eNBs and the SGW-U function outside the SGW-U Service Area)

/******************** End of Changes ***************/