SA WG2 Meeting #116-BIS	S2-165397

29 Aug – 2 Sep 2016, Sanya, CN	(revision of S2-164903,4448)

Source:	ZTE, CATR

Title:	Update to solution 2.8

Document for:	Approval

Agenda Item:	6.10.2

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes to adding media type in the application information. The application information provision procedure is also added.

1	Proposal

It is proposed to agree the following text in the TR23.799.

/******************** Start of Changes ***************/

6.2.8	Solution 2.8: Application-based uplink traffic identification

6.2.8.1	Solution Description

This solution addresses the working task#3 and #4 of key issue 2 on a QoS framework and is specific for the uplink traffic identification.

In this solution, the network may provide the UE the QoS policy together with application information when the PDU session is setup. The application information at least includes pair of OSId and OSAppId which can uniquely identify the application installed in the UE. For uplink traffic the UE identifies the application which generates the UL traffic and enforces the QoS Policy associated with the application information.

The User Plane Function in core network performs UL traffic verification by using application detection filters.

6.2.8.2	Function description

Editor's note: This clause will contain function descriptions and the interactions among the network functions.

Figure 6.2.x.2-1 Application information provisioning

1.	The CN-CP may provide the UE with the pre-authroized QoS rules. The pre-authorized rules include the application information and associated preauthorized QoS rules. The application information includes pairs of OSId and OSAppId which can uniquely identify the application installed in the UE. The UE uses this application information for UL traffic classification

2.	The CN-CP may update the pre-authorized QoS rules in the UE.

3.	 The CN-CP may provide additional dedicated QoS rules including application information to the UE when the CN-UP detects an application which is subject with dedicated QoS control or the CN-CP receives dedicated QoS control request from application.

Editor note x: It is FFS whether the option in step 3 is needed or not.

Editor note x:  How the CN-CP conveys the pre-authroized QoS rules to the UE is FFS. It could include e.g OMA-DM, or dedicated NAS signalling.

6.2.8.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

/******************** End of Changes ***************/