SA WG2 Meeting #S2-116BIS	S2-165448

29 August - 2 September 2016, Sanya, P.R China	(e-mail revision 5 of S2-165173)

Source:	Huawei, HiSilicon

Title:	Addition of charging control to the Policy Framework

Document for:	Approval

Agenda Item:	6.10.10

Work Item / Release:	NextGen / Rel-14

Abstract of the contribution: This contribution proposes to add charging control part to the policy framework.

####################### START TEXT FOR TR 23.799  ##########################

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.203: "Policy and charging control architecture".

[4]	3GPP TR 22.861: "Feasibility Study on New Services and Markets Technology Enablers for Massive Internet of Things; Stage 1 (Release 14)".

[5]	3GPP TR 22.862: "Feasibility Study on New Markets and Technology Enablers - Critical Communications; Stage 1(Release 14)".

[6]	3GPP TR 22.863: "Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1 (Release 14)".

[7]	3GPP TR 22.864: "Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1 (Release 14)".

[8]	NGMN Alliance: "Description of Network Slicing Concept", Version 1.0, January 13, 2016 (http://www.ngmn.org/publications/technical.html).

[9]	3GPP TR 22.891: "Feasibility Study on New Services and Markets Technology Enablers; Stage 1 (Release 14)".

[10]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[11]	SP-160464/RP-161266, "5G Architecture Options- Full Set". Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

[12]	SP-160460/RP-161269, "Tasks", Joint RAN/SA Meeting, Busan, S. Korea, June 2016.

[13]	3GPP TR 38.801: "Study on New Radio Access Technology; Radio Access Architecture and Interfaces"

[14]	IETF RFC 7296, "Internet Key Exchange Protocol Version 2 (IKEv2)".

[15]	IETF RFC 5191, "Protocol for Carrying Authentication for Network Access (PANA)".

[16]	3GPP TR 33.899: "Study on the security aspects of the next generation system"

[17]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses" Stage 2 (Release 14)

[18]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity" Stage 2

[xx]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[xx]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

####################### NEXT TEXT FOR TR 23.799 ##########################

### 6.10.3	Solution 10.3: Policy function and framework for non-roaming case

This solution addresses Key issue 10, WT#1, i.e. an overall non-roaming policy framework for NextGen is described.

#### 6.10.3.1	Architecture description

The figure 6.10.3.1-1 shows the NextGen policy architecture for non-roaming case.

Figure 6.10.3.1-1: Non-roaming NextGen Policy Architecture

NOTE：	Policy Function is regarded as part of control plane. The CP function in this figure includes other control plane functions, e.g. SM function, MM function. Only policy related interfaces are shown with corresponding names in this figure.

The following functions are involved in the NextGen Policy framework:

Policy Function: The Policy Function is the policy decision point to provide different control rules to NextGen CP Function based on the application session information received from the AF, operator's configuration, user subscription from the USD (User Subscription Database) and spending information from the Online Charging System. The Policy Function also supports the access type selection.

Editor’s note: How to support other policy functionalities, such as mobility policy, roaming policy and the third party policy are FFS.

User Subscription Database: The User Subscription Database stores the subscriber/subscription related information needed to support policy functionality.

Editor’s note: Whether or not the user subscription database for policy is collocated with the user subscription database for other CP functions is FFS.

NextGen UP Function: NextGen UP Function provides the packet core related user plane functionalities, including:

- 	QoS enforcement

- 	Measurement enforcement

- 	Gating

- 	SAF (Service Awareness Function): provides application detection and enforcement function.

- 	TSF (Traffic Steering Function): enforces the user plan traffic steering related policy received from the Policy Function via NextGen CP Function, e.g. moving individual IP flows from one access network to another, and or from one service function to another service function (e.g. for video analytics, optimized local content distribution and data caching, Firewall, Parental Control).

NOTE: The functionalities related to traffic steering for the services are the same as in EPC.

The NextGen UP Function may provide all above functionalities or part of these functionalities based on the operator's configuration/deployment.

NextGen CP Function: The NextGen CP Function provides the packet core control plane function.

-	When receiving the control rules from the Policy Function, the NextGen CP Function instructs the NextGen UP Function to enforce the policy (e.g. QoS policy, charging policy, gating policy, SAF policy, TSF policy).

-	When receiving access type selection related control rules from the Policy Function, the NextGen CP Function will transfer them to the UE.

-	When enforcing charging control from the Policy Function:

-	For online charging, the NextGen CP Function sends the credit request for a charging key to the Online Charging System, and reports the usage when the threshold is reached, as defined for credit management in TS 23.203.

-	For offline charging, the NextGen CP Function collects usage information per charging key /and service identifier combination and reports to the Offline Charging System as defined for reporting in TS 23.203.

Online Charging System: Provides support to credit management with the NextGen CP Function and support for policy decision based on spending limits as defined in TS 23.203.

Offline Charging System: Provides support for receiving usage information per charging key /and service identifier combination from the NextGen CP Function.

The following reference points are defined for the NextGen Policy framework:

NP1:	Reference point between the Policy Function and the USD. Similar to Ud/Sp interface in EPC.

NP2:	Reference point between the Policy Function and NextGen CP Function.

NG5:	Reference point between the Policy Function and the AF, the AF provides the application session information to the Policy Function. Similar to Rx interface in EPC.

NG4: 	Reference point between NextGen CP Function with NextGen UP Function.

NP3:	Reference point between the Policy Function and the Online Charging System, same functionality as Sy defined in TS 23.203.

NC1:	Reference point between the NextGen CP Function and the Online Charging System, same functionality as Gy defined in TS 32.251.

NC2:	Reference point between the NextGen CP Function and the Offline Charging System, same functionality as Gz defined in TS 32.240.

NOTE:	The Online Charging System and the Offline Charging System, and the protocols of NP3, NC1 and NC2 reference points are defined by SA5.

#### 6.10.3.2	Function description

Editor's note: This clause will contain function descriptions and the interactions among the network functions.

#### 6.10.3.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.

####################### END TEXT FOR TR 23.799 ##########################