SA WG2 Meeting #116	S2-163370

11 - 15 Jul 2016, Vienna, AT	(revision of S2-16xxxx)

Source:	LG Electronics

Title:	Solution - Session Management on the PDU session(s) via different accesses

Document for:	Approval

Agenda Item:	6.10.4

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This paper proposes the solution for the Session Management on the PDU session(s) via different accesses

Introduction

This contribution considers

-	SM_WT_#5: PDU session(s) via different accesses

| SM_WT_#5 | PDU session(s) via different accesses | UE connected via multiple accesses, including non-3GPP - Multiple PDU sessions via different accesses - A single PDU session via different accesses (a.k.a. multi-access PDU session) Identify the architecture options with the combination of access networks, and studying the procedure to set up them and manage their SM context. Define « a multi-access PDU session » with the combination of access networks (Note: the way to move IP flows between AN1 and AN2 depends on Key issue#20.) |
| --- | --- | --- |

The proposed solution identifies the architecture options with identifying the combination of access networks Also it proposes the initial definition of “a multi-access PDU session”

Proposal

It is proposed to add the following changes to TR 23.799.

* * * * Start of 1st Change * * * *

All NEW text to TR 23.799

6.4.X	Solution 4.X: Session Management on the PDU session(s) via different accesses

This solution applies to key issue 4 on session management. Specifically, the solution addresses the following work tasks:

-	SM_WT_#5: PDU session(s) via different accesses

6.4.X.1	Architecture description

This clause describes a session management architecture focusing on the following three cases:

- Multiple PDU sessions via different accesses to the different data networks

- Multiple PDU sessions via different accesses to  the same data network

- A PDU session via different accesses to the same data network (a.k.a. multi-access PDU session)

Editor's note: The high level architecture and interfaces for the non-3GPP access are not fixed yet, so the architectures of this solution may be needed to be updated with agreed architecture for the non-3GPP access.

The proposed solution assumes the following session management model principles:

- 	A UE can have multiple PDU sessions via simultaneous multiple accesses to the different data networks or same data networks (as showed in the figure 6.4.X.1-1 and figure 6.4.X.1-2)

-	A UE can have multiple PDU sessions via NG RAN and non-3GPP access network

-	The establishment and release for the PDU Session via NG RAN (NR or Evolved E-UTRA) is performed using NG1 signalling.

- 	The establishment and release for the PDU Session via non-3GPP access may be performed using NG1 signalling.

NOTE 1:	A UE may have multiple PDU sessions via Evolved E-UTRAN with non-standalone NR option. For example, one PDU session via Evolved E-UTRA and another PDU session via non-standalone NR. However, NR is not visible in the NextGen Core Network. It is seen as multiple PDU sessions via one access network.

- The access network type should be managed in the CP function when handing SM context of multiple PDU sessions via multiple accesses.

- A UE can have a PDU session via different accesses to the same data network (a.k.a. multi-access PDU session) (as showed in the figure 6.4.X.1-3)

- This solution defines « a multi-access PDU session » as follows;

A PDU session whose traffic can be routed over either a NG RAN or a non-3GPP access or both.

Editor's note: The definition of « a multi-access PDU session » is needed to be refined. The non-3GPP access for « a multi-access PDU session » may be limited to WLAN access in the Rel-15.

- The access network type should be managed in the CP function when handing SM context of a multi-access PDU session

Figure 6.4.X.1-1: Multiple PDU sessions via different accesses to the different data networks

Figure 6.4.X.1-2: Multiple PDU sessions via different accesses to the same network

Figure 6.4.X.1-3: Multi-access PDU session to the same network

6.4.X.2	Function description

Editor's note:	This clause will contain function descriptions and the interactions among the network functions.

6.4. X.3	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * * End of Changes * * * *