SA WG2 Meeting #116	S2-163387

11 - 15 Jul 2016, Vienna, Austria	(revision of S2-16xxyy)

Source:	Qualcomm Incorporated

Title:	Architecture requirement for support of location services

Document for:	Approval

Agenda Item:	6.10.0

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: There is no architecture requirement for support of location services in TR 23.799.

# 1	Discussion

In RAN requirements document (TR 38.913) there is already a requirement that NR needs to support location and positioning techniques:

"The NR should enable, and improve if suitable, state-of-art positioning techniques, such as RAN-embedded (Cell-Id, OTDOA, etc.) and RAN-external (GNSS, Bluetooth, WiFi, terrestrial beacons, etc…)."

NextGen is required to support an architecture in order for the operator to be able to provide location based services e.g. for use in emergency calls. In TR 23.799 there is no architecture requirement at the moment for the support of location services.

# 2	Proposal

It is proposed to add an architecture requirement in clause 4.1 of TR 23.799 for the support of location based services.

>>> First Change<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.203: "Policy and charging control architecture".

[4]	3GPP TR 22.861: "Feasibility Study on New Services and Markets Technology Enablers for Massive Internet of Things; Stage 1 (Release 14)".

[5]	3GPP TR 22.862: "Feasibility Study on New Markets and Technology Enablers - Critical Communications; Stage 1(Release 14)".

[6]	3GPP TR 22.863: "Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1 (Release 14)".

[7]	3GPP TR 22.864: "Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1 (Release 14)".

[8]	NGMN Alliance: "Description of Network Slicing Concept", Version 1.0, January 13, 2016 (http://www.ngmn.org/publications/technical.html).

[9]	3GPP TR 22.891: "Feasibility Study on New Services and Markets Technology Enablers; Stage 1 (Release 14)".

[x]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

>>> Next Change<<<

## 4.1	High level Architectural Requirements

Editor's note:	This clause will document high-level architectural requirements that guide the architecture study.

The architecture of the NextGen System shall:

1	Support the new RAT(s), the Evolved E-UTRA, and non-3GPP access types. GERAN and UTRAN are not supported:

a)	As part of non- 3GPP access types, WLAN access and Fixed access shall be supported. Support for satellite access is FFS.

2	Support unified authentication framework for different access systems.

3	Support multiple simultaneous connections of an UE via multiple access technologies.

4	Allow independent evolutions of core network and RAN, and minimize access dependencies.

5	Support a separation of Control plane and User plane functions.

6	Support transmission of IP packets, non-IP PDUs and Ethernet frames.

NOTE 1:	This requirement assumes a point-to-point link between the UE and the data network.

7	Leverage techniques (e.g. Network Function Virtualization and Software Defined Networking) to reduce total cost of ownership, improve operational efficiency, energy efficiency, and simplicity and flexibility for offering new services.

8	Efficiently support different levels of UE mobility (including stationary UE(s)) / service continuity.

9	Support different levels of resilience for the services provided by the network.

10	Support different means for reducing UE power consumption while UE is in periods with data traffic as well as in periods without data traffic.

11	Support services that have different latency requirements between the UE and theData Network.

12	Minimize the signalling (and delay) required to start the traffic exchange between the UE and theData Network, i.e. signalling overhead and latency at transition from a period where UE has no data traffic to a period with data traffic.

13	Support access to applications (including 3rd party applications) with low latency requirements hosted close to the access network within the operator trust domain.

NOTE 2:	The details of applications are beyond the scope of 3GPP.

NOTE 3:	Details of hosting of applications is considered beyond scope in this release.

14	Support optimized mechanisms to control (includes avoiding) signalling congestion.

15	Efficient network support for a large number of UEs in periods without data traffic.

16	Support network sharing.

17	Support roaming:

a)	As part of roaming, the architecture shall support both routing of user traffic entirely via the VPLMN and routing of the user traffic back to the HPLMN.

18	Support broadcast services.

19	Support network slicing.

20	Support Architecture enhancements for vertical applications.

21	Support dynamic scale-in /scale-out.

22	Minimize energy consumption in the overall network operation.

NOTE 4:	Specific architecture work resulting from the previous requirement may have to be addressed by SA WG2, SA WG5, or both.

23	Support critical communications, including mission-critical communications.

24	Support network capability exposure

25	Support efficient transmission of user data with characteristics required by Stage 1 requirements (e.g. low throughput short data bursts) including support of SMS.

26	Support regulatory requirements for Lawful Intercept.

27	Support a flexible information model with relationships between user related managed data, and with a level of abstraction sufficient to be independent of any specific protocols.

NOTE 5:	The previous requirement may have to be addressed by SA WG2, SA WG5, orstage 3 working groups.

28	Support optimized distribution of the data and the location of the data repositories in the network (access and core network) for efficient management of user related data by network entities.

29 Support for location services as defined for EPC in TS 23.271 [x] and in alignment with NR RAN TR 38.913 [10].

>>> End of Changes<<<