SA WG2 Meeting #116	S2-164093

11 - 15 Jul 2016, Vienna, AT	(was 3742 was 3671)

Source:	Intel (Drafting)

Title:	Interim agreements on QoS framework

Document for:	Approval

Agenda Item:	6.10.2

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: Based on the email discussion summary in S2-163427 a couple of interim agreements (2 – 5) are proposed on Key Issue #2 (QoS framework).

# 1	Proposal

It is proposed to agree the following text in TR 23.799:

####################### START TEXT FOR TR 23.799 ##########################

## 8.3	Interim Agreements on Key Issue #2 QoS Framework

Interim agreements for Key issue #2 QoS framework are as follows:

1)	Support Reflective QoS over RAN under control of the network. The network decides on the QoS to apply, reflects the DL traffic and the UE reflects the DL QoS for associated UL traffic.

Editor’s note: How reflective QoS is supported will be discussed as part of the solutions.

2.	U-plane marking for QoS is carried in encapsulation header on NG3 i.e. without any changes to the e2e packet header.

3a.	 A default QoS rule shall and pre-authorised QoS rules may be provided at PDU Session establishment to UE. using NG1 signalling.

NOTE: In some cases part of the QoS information can be provided as AS information even at PDU Session establishment.

Editor’s Note: QoS related signalling to the UE for non-3GPP access is FFS.

3b. QoS rules can be (e.g. depending on access capabilities) provided at PDU Session establishment to the RAN using NG2 signalling.

4.	Flow-specific QoS signalling via the C-plane is needed for GBR SDF.

Editor’s Note: Definition of Flow is for FFS.

####################### END TEXT FOR TR 23.799 ##########################