SA WG2 Meeting #117	S2-165583

17 Oct – 22 Oct 2016, Taiwan

Source:	China Telecom

Title:	Update to solution 18.4

Document for:	Approval

Agenda Item:	FS_NexGen /6.18.4

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: the contribution updates solution 18.4.

- Introduction

The paper proposed to update the text in the figure 6.18.4.1-1 in solution18.4. Figure 6.18.4.1-1 depicted a simplified system architecture that allows for tight interworking between evolved NG RAN anchored on E-UTRA (options 5, 7) and NG RAN anchored on NR (options 2, 4). The indications of the options in the figure seemed confusing.

The paper proposed a minor modification, in which the text "Upgraded E-UTRAN : NG RAN (2, 5) and legacy LTE" in the left ellipse was replaced by "Upgraded E-UTRAN : NG RAN (5, 7) and legacy LTE ", while the text "NG RAN (4, 7)" in the right ellipse was replaced by "NG RAN (2, 4)".

- Proposal

It is proposed to update the following text to the TR 23.799 “Study on Architecture for Next Generation System”.

>>>Start of Changes<<<<

6.18.4	Solution 18.4: Solution for interworking and migration

6.18.4.1	Architecture description

This solution addresses Key Issue #18 (in particular WT#1, WT#2 and WT#3) based on the following principles:

-	Where possible the installed E-UTRAN access base is upgraded to support the new CN-RAN interface towards the NG Core (NG2/NG3). This approach allows for tight interworking between upgraded E-UTRAN access and NG RAN access.

-	Loose interworking based on "dual radio" operation in areas where the installed E-UTRAN access base has not been upgraded. The same type of interworking is supported to/from GERAN and UTRAN access.

Depicted in Figure 6.18.4.1-1 is a simplified system architecture that allows for tight interworking between evolved NG RAN anchored on E-UTRA (options 5, 7) and NG RAN anchored on NR (options 2, 4).

Figure 6.18.4.1-1: Architecture for tight interworking between upgraded E-UTRAN base (supporting options 5, 7 and legacy LTE) and NG RAN anchored on NR (options 2, 4)

It is assumed that the installed E-UTRAN base has been upgraded to support the new CN-RAN interface ("NG2/NG3" in Figure 6.18.4.1-1) in addition to supporting the S1 interface towards the EPC. On the radio side it is assumed that the upgraded E-UTRAN base supports the legacy LTE-Uu interface, in addition to the E-UTRA anchored NR (options 5, 7).

>>>End of Changes<<<<