SA WG2 Meeting #117	S2-165809

17-21 October 2016, Kaohsiung city, Taiwan(R.O.C)	(was S2-16)

Source:	ZTE Corporation

Title:	Clarification on the SSC mode

Document for:	Approval

Agenda Item:	6.10.6

Work Item / Release:	FS_NextGen / Rel14

Abstract of the contribution: It is proposed that same SSC mode is applied to all PDU sessions of one UE towards same Data Network.

1	Proposal

It is proposed to modify TR 23.799 as following.

* * * * First Change * * * *

6.6.1.1.4.1	Mode selection and network support

The following principles apply:

-	When requesting a PDU session, the UE may indicate the requested session and service continuity (SSC) mode as part of the PDU session setup signaling to the network. How the UE can determine the requested SSC mode is specified in clause 6.6.1.1.4.

Editor's note:	It is FFS whether the requested SSC mode is indicated as part of the DN name or as a separate indication.

-	The serving network receives the list of supported SSC modes and the default SSC mode per data network per subscriber as part of the subscription information from the subscriber database.

-	The serving network selects the SSC mode by either accepting the requested SSC mode or modifying the requested SSC mode based on subscription and/or local configuration.

NOTE 1:	A serving network may be configured  to modify  the SSC mode requested by the UE for different reasons, e.g. the serving network may prefer to avoid SSC mode 1 for a specific data network to avoid concentrating traffic on centralized peering points with the related data network.

-	If a UE does not provide an SSC mode when requesting a new PDU session, then the network selects the default SSC mode listed in the subscription (for the data network to connect to) or applies local configuration to select the SSC mode.

-	After selecting the SSC mode, the network either (a) accepts the PDU session request from the UE and indicates the selected SSC mode to the UE, or (b) the network rejects the PDU session request and sends the selected SSC mode and a cause value to the UE indicating that the selected SSC mode is already used by another PDU session in the UE.

-	SSC modes apply per PDU session. A UE may request different SSC modes for different PDU sessions towards different Data Networks, i.e. different PDU sessions towards different Data Networks which are active in parallel for the same UE may have different SSC modes. Same SSC mode is used for all PDU sessions of one UE towards same Data Network.

-	The SSC mode is not changed during the lifetime of a PDU session.

-	TUPF selection: When selecting the TUPF for a PDU session, the network takes the UE's current point of attachment and the requested SSC mode into account.

NOTE 2:	It is not precluded that other information is also taken into account for TUPF selection (e.g. as determined by other key issues).

* * * * End of Change * * * *