SA WG2 Meeting #118Bis	S2-170209

16- 20 January 2017, Spokane, America	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	Selection of SMF

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution focus on the Session Management selection function in 5G.

# Introduction

This proposal discusses the factors that need to be considered when select a SMF.

# Proposal

It is proposed to add the following descriptions to the TS 23.501 “System Architecture for 5G System”.

* * * Start of changes* * * *

## 6.2	Network function selection

Editor’s notes: This should include NF selection procedure for various NF selection.

## 6.2.x	SMF selection function

The SMF is selected by the AMF during the PDU session establishment procedure. For home-routed roaming case, both the V-SMF and H-SMF are selected by the AMF located in the VPLMN. The SMF can be selected on per-session granularity. The following factors may be considered during the SMF selection:

-  Data Network Name (DNN)

-  Network slice instance identifier

-  Subscription information from UDM, e.g. Default DNN, whether local breakout may apply to the session

-  UE location information

-  Local operator policies

-  Load conditions and weight factors of the candidate SMFs

* * * End of Changes * * *