SA WG2 Meeting #118bis	S2-170243

16-20 January 2017, Spokane, WA,USA	(was S2-17xxxx)

Source:	Sony

Title:	Pairing – Fast connection setup between Relay and Remote UE

Document for:	Approval

Agenda Item:	6.10

Work Item / Release:	FS-REAR/Rel-15

Abstract of the contribution: Discuss the requirement for fast connection setup and the relation to pairing of Relay UE and Remote UE and propose change to Key Issue 3

# Discussion

The term “pairing” was briefly discussed in Reno, and it was concluded that we need a definition of what this term means in the scope of REAR. The term “pairing” was introduced in SA1 in TS 22.278 clause 7C.1

“The 3GPP system shall support pairing of an Evolved ProSe Remote UE and a specific Evolved ProSe UE-to-Network Relay to allow for fast remote connectivity setup with UE consent given, and/or MNO control applied, at the time of pairing.”

And further included in the objective of this feature FS_REAR

“Depending on the work in RAN2 (see TR 36.746), investigate and study impacts on the EPC to aid pairing between an Evolved ProSe Remote UE and an Evolved ProSe UE-to-Network Relay to allow for fast remote connectivity setup.”

RAN2 is also looking for a definition of paring TR36.746 0.3.0

“Editor’s Note: A definition of when the evolved ProSe Remote UE and evolved ProSe UE-to-Network Relay UE are “paired” or “connected” is needed.”

Observation 1: SA1 is looking for a way to allow fast connectivity setup and use the term “pairing” for the method. SA2 is going to investigate how the EPC can aid the method “Pairing”. However, the method has not been proposed and agreed.

Proposal 1: Initially focus on identifying what enhancements to the D2D connectivity setup procedures that are needed in REAR. What are the inefficiencies, and how can those inefficiencies be addressed.

The term pairing has a strong legacy meaning in the local wireless connectivity community. Pairing is used in Bluetooth for the exchange of security keys before link establishment, and often this requires user consent via e.g. a pin-code, or a user acceptance dialogue. SA1 mention that UE gives consent. RAN2 mention that the Remote UE and Relay UE may be paired before connection establishment.

Observation 2: Pairing could mean some kind of association between the Remote UE and Relay UE that aids the connection setup speed, but it is unclear.

In ProSe there is no explicit need for a direct association between the Relay and the Remote UE, both devices have been authorized and provisioned with enough information (codes and discovery masks) to aid the discovery/selection of relays and to set-up the communication. The Network, ProSe function, provides the association between Relay-UEs and Remote-UEs implicitly.

feD2D and REAR is based on the concept of Layer2 relay solution which indicates that the security relation/association is between the Network and the remote UE, rather than between Relay UE and Remote UE.

In a mass deployment of IoT devices, multiple UEs can act as Relays and the neighbouring relay constellation is likely to change over time. In such scenario, the relationship between Relay and Remote UE can be rather short-lived.

Observation 3: In a mass deployment with rather short-lived connections/associations, it can actually be counterproductive to require prior device2device pairing.

The Relays are ordinary UEs with added Relay capabilities, and network authorization to act as a Relay. These Relays cannot be judged as part of the network trusted domain. Requiring Remote UEs to pair with any/multiple Relays before communication might reduce the integrity protection of the Remote UE. To protect data integrity and user integrity for both Relay and Remote UE, it might be better not to require any association/pairing between the relay and remote UE.

Observation 4: We must consider UE integrity issues when investigation any pairing concept.

Proposal 2: Mass deployment scenario and User integrity protection must be considered when enhancing the D2D connection set-up.

# Conclusion

Before defining what pairing is, SA2 should first agree that the purpose/Key Issue is to speed up the connection set-up between a Relay and Remote UE. We propose:

- Initially focus on identifying what enhancements to the D2D connectivity setup procedures that are needed in REAR. What are the inefficiencies, and how can those inefficiencies be addressed.

- Mass deployment scenario and User integrity must be considered when enhancing the D2D connection set-up.

# Proposal

It is proposed to update TR 23.733 to reflect above conclusions.

* * * Start of changes * * * *

## 5.3	Key issue #3: Fast Connection setup between an eRemote-UE and an eRelay-UE

### 5.3.1	General description

Fast connection setup between an eRelay-UE and an eRemote-UE is part of the service requirements, and pairing has been suggested as a mean to achieve fast connection setup. When developing solution for fast connection setup the following should be considered:

- Identify inefficiencies, in ProSe Relay discovery, selection and connection setup, that should be enhanced for eRelay-UE and eRemote-UE

- How can pairing enhance the discovery, selection and connection procedures, to enable fast connection setup

Editor’s note: It is FFS whether pairing is only used for private Relay Networks.

- How will short-lived associations between eRelay-UE and eRemote-UE be managed

- How to protect and maintain user integrity in a mass deployment of eRelay-UEs and eRemote-UEs.

- Whether a UE is willing to serve as an eRelay-UE.

-	How and whether the network assists the eRemote-UE to select an appropriate eRelay-UE.

* * * End of changes * * * *