# 3GPP TSG SA WG 2 Meeting 118bis	TDoc S2-170332

Spokane, WA (USA), 16-20 January 2017

Title:	[Draft] Response to LS on SA2 involvement for the light connection

Response to:	C1-165447

Release:	Rel-14

Work Item:	LTE_LIGHT_CON-Core

Source:	SA2

To:	CT1, RAN3

Cc:	RAN2, CT4

Contact person:	Ericsson TBD

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA2 thanks CT1 for the LS concerning the work related to Light Connection in RAN WGs and in CT WG1.

It is SA2 understanding that the work leading WGs will communicate any potential issues resulting from this work that require SA2 actions.

Furthermore, SA2 discussed the provided information on the content of the new functionality and would like to bring the following to the attention:

- SA2 assumes that paging functionality, when moved to eNB, needs to provide the same system level reliability as when managed in the MME. RAN WGs need to ensure that paging success rate is maintained taking the network configuration limitations and dynamics into account.

- SA2 assumes that RAN will conduct reachability monitoring ensuring that UEs that are no longer reachable (e.g. powered off while out of coverage) will be discovered and the allocated network resources can be released.

- Furthermore, SA2 discussed the side-effects of the UE being in ECM-Connected while in RRC Light Connected state.  For a UE in ECM-Connected, some CN functions, e.g. LI, assume a very precise knowledge of the UE location, i.e. on the cell level. On the other hand, that precise UE location information is not available while a UE is in RRC Light Connected state. That mismatch can cause systematic errors in certain CN functions and is considered as not acceptable. To avoid errors originating from incorrect assumption about UE RRC state mismatch RAN can notify the CN about a state transition.

- SA2 has concerns that neither Power Save Mode (only applicable for UEs in ECM-Idle state) nor extended DRX for Idle mode where data is expected to be buffered in the CN, are interoperable with the RRC Light Connected state.

# 2	Actions

To RAN3 and CT1:

ACTION: SA2 kindly asks CT1 and RAN3 to take the issues raised in the LS into consideration during their normative work and kindly indicate if the chosen solutions require SA2 actions.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 119	13-17 February 2017	Dubrovnik, Croatia

TSG SA WG2 Meeting 120	27-31 March 2017	Busan, Korea