SA WG2 Meeting #118 Bis	S2-170500

16 – 20 January 2017, Spokane, Washington, USA     	(revision of S2-170212)

Source:	Huawei, HiSilicon

Title:	Session Connection State Management

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes session connection state management procedures to support indualvidual session activation and deactivation.

1. Discussion

This paper proposes session connection state management procedures to Clause 4.2 of TS 23.501, “Session Management states model”

2. Proposal

It is proposed to add the following text to the TS 23.502.

* * * Begin of Changes * * * all new texts

4.2.x	PDU Session Connection State Transition

4.2.x.1	Session-IDLE State Transition triggered by UE under CM_Connected mode

Figure 4.2.x.1-1: Session-IDLE state transition procedure triggered by UE, AN or SMF

1.	One of the following triggers for Session Connection State Transition procedure takes place.

1a. The UE decides to initiate a session-IDLE state transition procedure: the UE sends to the SMF a Session-IDLE request. The message includes the UE Temp ID, Session ID(s), and Session-IDLE state.

1b. The AN monitors PDU session activities of all PDU sessions. If the AN detects no UL and DL packets  within Session Inactive Timer set by SMF during Session Establishment procedure in Clause 4.2.1, the AN sends to SMF the Session-IDLE State Transition request, which requests to set Session-IDLE state for a single or multiple PDU sessions. The message includes the UE Temp ID, and Session ID(s), AN ID.

Editor’s Note: Whether a special value of Session ID(e.g. wildcard) can be used to denote all of the PDU sessions in the requests is FFS.

Editor’s Note: Whether SMF may initiate the session_idle state transition is FFS.

2. Session update on the UPF:

2a.	The SMF requests the UPF to release NG3 information of AN from the UE’s PDU session context in UPF, including the tunnel endpoint ID of AN and the AN IP address.

2b. The UPF responds to SMF to confirm the completion of UE Context Update request.

3.	The SMF sends N2 session update request to the AN  via the AMF. The message includes UE Temp ID, Session ID(s) and Session-IDLE state.

4.	The AN and UE perform RRC Connection Reconfiguration procedure to release allocated resouces for the session(s). The AN informs the UE that the session state transition via the procedure.

Note: It is up to RAN groups to specify further details of this step.

5. 	The AN releases contexts of the PDU session(s) and then sends N2 Session update response to the SMF Svia the AMF to confirms the completion of Session-IDLE state transition in the UE. The message includes the UE Temp ID and Session ID.

6. 	The SMF sets the session connection state of the UE context to the Session-IDLE.

Editor’s Note: In steps 3 and 5, which services should be used to transfer the N2 messages are to be decided later.

Editor’s Note: It is FFS whether the PCF should be involved during the session connection state transition procedures.

4.2.x.2	Session-ACTIVE State Transition Triggered by UE under CM_Connected mode

Figure 4.2.x.2-1: Session-ACTIVE State transition procedure triggered by UE

1.	The UE sends to the SMF the Session-ACTIVE request. The message includes the UE Temp ID, Session ID(s), and Session-ACTIVE state.

Editor’s Note: Whether a special value of Session ID (e.g. wildcard) can be used to denote all of the PDU sessions in the requests is FFS.

2.	The SMF sends N2 Session Update Request to the AN via the AMF. The message includes UE Temp ID, Session ID, Session-ACTIVE state, and NG3 connection information.

3.	The AN and UE perform RRC Connection Reconfiguration procedure.

The AN assigns radio resources to serve the PDU session according to the QoS profile. If the QoS profile requires admission control, the AN performs admission control. In case the radio resources are not enough to support the PDU session, the AN informs the UE to release the PDU session.

If the AN can support the PDU session, the RRC Connection Reconfiguration message sent from the AN to the UE indicates radio resource allocation. The UE sets the session connection state to Session-ACTIVE.

Editor’s Note: It is up to RAN groups to specify further details of this step.

4. 	The AN sends to the SMF the message N2 Session Update Response. The message includes the UE Temp ID and Session ID. The AN notifies either the successful completion of Session-ACTIVE state transition or a cause code, e.g. lack of radio resources for to support the PDU session.

5.	If in Step 4, the AN notifies successful completion of Session-Active state transition, the SMF requests the UPF to update the UE session context. Otherwise, Steps 5a and 5b are skipped.

5a.	The SMF requests the UPF to update NG3 information of AN from the UE’s PDU session context in UPF, The message include UE Temp ID, Session ID, and NG3 information, including the tunnel endpoint ID of AN and the AN IP address.

5b. The UPF responds to the SMF to confirm the completion of Session Update request.

6. 	Based on the message received from the AN in Step 3, the SMF sets the session connection state in the UE context accordingly.

-	If the session connection state transition is successful, the SMF sets the session connection state to the Session-ACTIVE.

-	If the session connection state transition is unsuccessful (the AN cannot support the PDU Session), the SMF initiate Session Release procedure as specified in Clause 4.2.3.

Editor’s Note: It is FFS whether the PCF should be involved during the session connection state transition procedures.

Editor’s Note: In steps 2 and 4, which services should be used to transfer the N2 messages are to be decided later.

* * * End of Changes * * *