SA WG2 Meeting #118Bis	S2-170210

16- 20 January 2017, Spokane, America	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	Selection of SMF

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes the UP selection function.

# Discussion

This contribution proposes the UP selection function based on TR conclusion.

# 2. Proposal

It is proposed to add the following text to the TS 23.501.

* * * Start of changes * * * *

## 6.2	Network function selection

Editor’s notes: This should include NF selection procedure for various NF selection.

### 6.2.x	UPF selection function

The UP function selection and reselection are performed by the SMF. For home routed roaming case, the UPF in home PLMN is selected by H-PLMN, and the UPF(s) in visited PLMN is selected by V-PLMN. The following factors may be considered during the UPF selection:

-  UE location

-  Data Network Name (DNN)

-  PDU session type (i.e. IPv4, IPv6, Ethernet Type or non-IP Type)

-  SSC mode selected for the PDU session

-  UE subscription profile in UDM

-  Information on where to route the traffic

-  Location or potential locations of the application to where the traffic routing should apply

-  UE mobility information (mobility pattern)

-  load conditions and weight factors of the candidate UPFs

-  Local operator policies

* * * Next change * * * *