SA WG2 Meeting #118	S2-167071

14 – 18 November 2016, Reno, Nevada, USA	(revision of S2-166718)

Source:	Huawei, HiSilicon

Title:	Evaluation for Solution 6.5.5 and additional interim agreements for Key Issue #6

Document for:	Approval

Agenda Item:	6.10.6

Work Item / Release:	FS_NexGen/Rel-14

Abstract of the contribution: Propose evaluation for Solution 6.5.5 and additional interim agreements for Key Issue #6.

# Proposal

It is proposed to update TR 23.799 as follows.

* * * Start of 1st Change * * *

#### 6.5.5.3	Solution evaluation

Solution 6.5.5 provides a solution for mobile network operators to establish an efficient end-to-end user plane (UP) connection between UE and external application server (AS), where AS can be operated by the same mobile network operator or a third party service provider. Application Servers can provide different types of services, including content distribution network (CDN) for video streaming and online shopping, and IoT applications, etc. The UP path selection can be initiated by CP SM function for e.g. UP-GW load balancing or user mobility, taking the AS location and maybe other information received from the AS controller into account.

-	The solution 6.5.5 allows the external AS controller to provide additional information to CP functions, e.g. application location or potential application locations, SSC indicator, etc., to ensure efficient end-to-end UP path.

-	The solution 6.5.5 allows the CP to perform UP path (re)selection with taking the information provided by the external AS controller into account to ensure timely UP path (re)selection.

-	The solution 6.5.5 allows the CP to notify the external AS controller about UP path (re)selection to facilitate AS (re)location and ensure end-to-end UP path efficiency.

* * * End of 1st Change * * *