SA WG2 Meeting #119	S2-170899

16 - 20 Jan 2017   Dubrovnik, Croatia

Source:	Ericsson

Title:	Principles for Change of AMF

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Update and clarification of S2-170142, proposes a solution to change the AMF instance serving a UE without activating the UE and also address the how other functions connected to AMF and also serving the UE becomes aware of the change of AMF instance.

# Discussion

Background and derived requirements & principles

In 23.799 v 14.0.0 the KI #19 was concluded with the single statement:

8.14	Agreements on Key Issue #19: Architectural impacts when using virtual environments

The architecture should support mechanisms to avoid issues caused by the persistence ("stickiness") of UE-specific associations on at least NG2.

To give some more guideline for a solution to this agreement we also note the following statements in 23.799:

5.19.2.2	Scaling in/out of the AMF, and, handling of long lived NG1/2 connections

…

In a virtualised environment, this AMF could be instantiated multiple times in one or multiple data centres.

…, the Next Generation system should permit complete NAS procedures (c.f. EPC Attach procedure) to be executed on the same instance of the AMF.

… the Next Generation system should permit time-separated NAS procedures from one UE to use different instances of the AMF.

and

8.12.4	Reference points and Functionality description

…

NOTE 1:	Regardless of the number of Network functions, there is only one NAS interface instance between the UE and the CN, terminated at one of the Network functions that implements at least NAS security and mobility management.

…

From these statements we derive the following requirements for a solution:

- During a NAS or NG2 procedure a UE shall be served by the same AMF instance

- From one NAS or NG2 procedure to another it shall be possible to have a different AMF instance that serves a UE

- It shall be possible to change the serving AMF at any time without activating a UE that is inactive and without any additional UE interaction

The principle of keeping the same AMF instance during the lifetime of a NAS or NG2 procedure is important in order to ensure proper coordination and consistent handling of messages received by the AMF during the procedure, including potential unsolicited interfering messages from other entities in the network. This is assured by handling all ongoing signaling for one UE in the same instance of the AMF.

For the network to be able to route all messages to the same AMF instance during the life time of any procedure for a specific UE, there needs to be a mechanism to steer to the “serving AMF” instance for the specific UE. In EPC this mechanism is based on information in the temporary UE identifier (GUTI) for signaling from the terminal, but the mechanism/identifier differs depending on which interface the signaling enters in the MME function in EPC. In 5G CN it would be beneficial to have a similar mechanism/identifier for signaling from the UE towards the AMF. Whether to extend to use the same identifier on all AMF interfaces is FFS.

Another principle is to have a reasonable scaling for the 5G RAN entities. The scaling up or down of the core can not only be done per AMF instance which could result in excessive changes for the individual 5G RAN instance on the number of AMF instances to connect to. Instead it is expected that a single AMF instance itself can scale in/out within the instance without RAN coordination by adding VMs or other implementation choices.

Further, it is generally expected that core network functionality will be provided as virtual functions running on an execution environment at one or more physical locations in an operator’s network, e.g. in data centers. The core network functionality (or parts of it) may be instantiated at multiple physical locations throughout the operator’s network. The network architecture must include redundant functionality and robust mechanisms to handle failures of complete physical locations, e.g. due to earthquakes, or other unplanned major events.

The network functionality must not make assumptions about the functionality provided by the execution environment and rely on that it can provide scaling and reliability mechanisms over multiple physical locations, as the cloud infrastructure and its execution environment may vary from operator to operator.

Based on the above an AMF instance at one physical location must be able to handle a UE that was previously handled by another AMF instance at a different physical location. The limitation on which physical location, handling of the UE, can be moved to is governed by the available connectivity to the base station that is connected to the UE. This is also what limits the physical location of a MME pool in EPC, i.e. where there is direct connectivity from the pool-members to the eNBs included in the pool area. Thus, handling of a UE can be moved between different AMF instances at different physical locations, if there is direct connectivity from the base station that is connected to the UE.

One can see this as there is a pool of AMF instances, between which the handling of the UE can be moved. What characterizes this AMF pool is that the AMF instances have the same/similar functionality and that all base stations connected to one AMF instance also are connected to all other AMF instances in the pool.

Note the difference from a “resource pool” which can be used to scale up functionality e.g. of an AMF instance within e.g. a data center which typically is limited to a physical location. The pool of AMF instances can scale over several physical locations.

Solution

To meet the above derived requirements a possible solution is outlined in this paragraph. The solution is based on the following key principles:

- Enable moving a UE context between AMF instances between procedures for a UE, immediately coordinate with all peer Network Functions, but delay coordination with the UE:

  - A procedure for moving UE contexts between AMF instances inspired by existing S1 HO procedure in EPC today but without immediate UE contact;

  - Do not assume existence stateful or stateless AMF instance, or separate database – different options can be possible and should be left for implementation;

- Maintain a pooling concept over several AMF instances to support redundancy between instances, especially if instances located at different physical locations:

  - Use a temporary UE identifier to allow RAN to find the serving AMF within an AMF pool;

  - Storage of UE context, including replication within AMF pool e.g. for geo redundancy can be left for vendor implementation either in separate DB, separate NF or distributed among AMF instances;

- When the UE next time connects to the CN, re-direct between AMFs (within AMF pool) if UE contacting “non-serving” AMF based on current temporary identifier;

- Re-use of re-routing mechanism from DECOR/Nw slice selection to re-route from one AMF instance to another when the UE connects with a “non-serving” AMF in the CN (in the AMF pool);

Based on these principles, a possible solution could be described as follows:

X.x.1 Change of AMF instance in between NAS procedures

For a UE, its serving AMF within an AMF pool may at any time be changed to another serving AMF within the same pool. The AMF Change procedure includes immediate signaling to all affected network functions except for the UE itself. An outline of the procedure is illustrated in the following call flow.

- AMF Change procedure example

In Step 1, the procedure is initiated, and the initial coordination between the Old and New AMFs take place. There may be multiple triggers for an AMF change including load (re-)balancing, planned maintenance or a failure of an AMF instance. The coordination may or may not include a UE context transfer from the old to the new AMF, depending on how UE contexts are stored and managed. This is considered specific to implementation within the AMF pool.

In case the AMF change is initiated because of a failure of the Old AMF, the assumption is that the UE context is stored and/or replicated in such a way that it is not lost due to the failure (see further chapter X.x.1.2)

In Step 2, the New AMF allocates a new temporary identity for the UE. However, as indicated in the picture, no signaling to the UE occurs at this point.

X.x.1.1 Executing the AMF change for a UE in Idle mode

When, at a later stage (in Figure 2), the UE becomes active, e.g., due to paging following downlink data, the previously allocated temporary identity is provided to the UE, and the UE is re-directed to the new serving AMF as illustrated in the following diagram. In step 6 there are different options for how to find a UE in other AMF instances. One options can be to broadcast to other AMF instances asking for the UE location, but other options are possible as well. Details of how to find the serving AMF and which of the options to select is FFS.

- DL data/Paging procedure incl. temporary Id change

X.x.1.2 Executing the AMF change at AMF Instance failure

If the AMF instance handling a specific UE has failed and can not be contacted, the base station handling the UE communication shall perform a new selection of an AMF instance. This AMF instance may or may not have access to the UE context. In case it does not have access to the UE context it must check among the other instances which AMF instance that have access to the UE context (step 3 in Figure 3). Once a new AMF instance is found, the message is re-directed and the procedure continues.

- Executing the AMF change at AMF Instance failure

X.x.1.3 Executing the AMF change for a UE in Connected mode

For a UE already active and in Connected mode, newly allocated temporary identity is provided to the UE at next contact with the Core network, as when the UE being active in connected mode from a Core Network point of view, the UE may still be RRC Inactive Connected and therefore the temporary Id reallocation not happen at once but also here be delayed until the UE next time communicates with the CN.

Details on how to inform the RAN about the change of AMF instance while still delaying the temporary Id re-allocation to the UE is FFS, but the actual re-allocation towards the terminal have similarities from step 5 of the procedure in Figure 3

Conclusion

From the agreements in TR 23.799 v 14.0.0 we have derived the following requirements:

- During a NAS or NG2 procedure a UE shall be served by the same AMF instance

- From one NAS or NG2 procedure to another it shall be possible to have a different AMF instance that serves a UE

- It shall be possible to change the serving AMF at any time without activating a UE that is inactive and without any additional UE interaction

We also identify a few principles that a solution should follow, and of these we would like to highlight the following:

- A single AMF instance itself can scale up or down within the instance without RAN coordination – This to have a reasonable scaling for the 5G RAN entities without excessive changes for the individual 5G RAN instance on the number of AMF instances to connect to

- The network architecture must not make assumptions about the data center infrastructure and the functionality for scaling between data centers - The network architecture should however include robust mechanisms to handle failures of complete data centers, e.g. due to earthquakes, terrorist attacks etc.

Based on these requirements and principles we have shown a possible solution meeting all these requirements and that also meets the agreements of KI #19.

# Proposal

Add the following to the TS 23.501.

*************** Start of changes *********************

### 6.1.x	Principles for change of AMF

To allow for a more flexible 5G CN architecture and to avoid issues caused by the persistent associations to a specific AMF instance, the system shall be specified accordingly.

The following principles shall apply for scalability and robustness of an AMF:

-	A single AMF instance itself can scale up or down within the instance without coordination with its peers, e.g. a single AMF instance can scale up or down within the instance without RAN coordination. This allows for scaling in 5G entities without excessive changes for the individual 5G RAN instance on the number of AMF instances to connect to.

-	The network architecture should include robust mechanisms to handle failures of complete AMF instances, e.g. due to earthquakes, other major unplanned events, etc.

On NG2 interface, to enable sufficient flexibility in the association between a UE and its serving AMF instance, the following principles shall apply for signalling involving the UE and AMF:

-	During a NAS or NG2 procedure a UE shall be served by the same AMF instance;

-	From one NAS or NG2 procedure to another it shall be possible to have a different AMF instance serve the UE;

-	It shall be possible to assign a different AMF instance for serving the UE without a need for immediate signalling with the UE.

*************** End of changes *********************