SA WG2 Meeting #119	S2-1701053

13 - 17 February 2017, Dubrovnik, Croatia	(revision of S2-17xxxx)

Source:	CATT

Title:	TS 23.501: Registration area management

Document for:	Discussion / Approval

Agenda Item:	6.5.2

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to add registration area management into TS 23.501.

- Discussion

In order to track a UE, the 5GS shall allocate a registration area to the UE during a initial registration or registration update procedure. During the TR phase, we have agreed that the TA concept defined in the EPS can still be used, we therefore propose to clarify how to implement registration area management by using TA concept.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts into TS 23.501.

* * * * Start of 1st Change * * * *

5.3.2	Registration Management

5.3.2.x	Registration area management

Registration Area management comprises the functions to allocate and reallocate a Tracking Area Identity list to the UE. The "Tracking Area List concept" is same as that in EPS. The allocation of Tracking Area Identity list may take UE mobility pattern into account. All tracking areas in a Tracking Area List to which a UE is registered are served by the same serving AMF.

Editor’s note: Registration area management for Non-3GPP access case is FFS.

If a UE is within an “allowed area” specified in clause 5.3.4.1.1, the registration area allocated to the UE is within or equivalent to the "allowed area".

* * * * End of Changes * * * *