SA WG2 Meeting #120	S2-171884

26 - 31 March 2017, Busan, Korea	(revision of S2-17xxxx)

Source:	China Mobile, Huawei

Title:	Clarification for standardized QoS characteristics

Document for:	Approval

Agenda Item:	6.5.5

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: Proposes a clarification of the standardized QoS characteristics.

# 1	Discussion

The EPS related standardized QCI characteristics and their corresponding example services are specified in TS 23.203.

Given that all the services in EPS will have to be inherited in 5G network, it is proposed that QoS characteristics mapping towards Standardized 5QI should keep consistent with EPS Standardized QCI characteristics in order to make feasible the interworking handling between EPS and 5GS.

# 2	 Proposal

It is proposed to adopt the following in TS 23.501.

* * * * Begin First Change * * * *

5.7.4	Standardized 5QI to QoS characteristics mapping

The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.5-1.

Table 5.7.5-1: Standardized 5QI to QoS characteristics mapping

| 5QI value | Resource Type | Priority Level | Packet Delay Budget | Packet Error Rate | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 2 | 100 ms | 10-2 | Conversational Voice |
| 2 | GBR | 4 | 150 ms | 10-3 | Conversational Video (Live Streaming) |
| 3 |  | 3 | 50 ms | 10-3 | Real Time Gaming, V2X messages |
| 4 |  | 5 | 300 ms | 10-6 | Non-Conversational Video (Buffered Streaming) |
| 65 |  | 0.7 | 75 ms | 10-2 | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 |  | 2 | 100 ms | 10-2 | Non-Mission-Critical user plane Push To Talk voice |
| 75 |  | 2.5 | 50 ms | 10-2 | V2X messages |
| 5 |  | 1 | 100 ms | 10-6 | IMS Signalling |
| 6 |  | 6 | 300 ms | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 7 | 100 ms | 10-3 | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 8 | 300 ms | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file |
| 9 | Non-GBR | 9 |  |  | sharing, progressive video, etc.) |
| 69 |  | 0.5 | 60 ms | 10-6 | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 |  | 5.5 | 200 ms | 10-6 | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 |  | 6.5 | 50 ms | 10-2 | V2X messages |

* * * * End First Change * * * *