SA WG2 Meeting #120	S2-172075

27-31 March 2017, Busan

Source:	Huawei, HiSilicon

Title:	TS 23.502: Update SMS over NAS procedures

Document for:	Approval

Agenda Item:	6.5.8

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution updates the description of SMS over NAS procedures in TS 23.502.

# 1	Introduction

The contribution proposes to update the following changes,

- Clarifying SMSF address is not part of the subscription. The UDM stores the SMSF address in case the SMSF registers to the UDM for UE.

- Adds the description of NF discovery service from NRF to discover SMSF.

# 2	Proposal

It is proposed to update procedures for SMS over NAS in TS 23.502.

***** BEGIN 1st CHANGE ***** All new context

#### 4.13.3.1	Registration procedures for SMS over NAS

Figure 4.13.3.1-1: Registration procedure for SMS over NAS

1. 	UE and network follow the procedure as defined in clause 4.2.2.2 for initial registration in 5GS. To enable SMS over NAS transporting, UE includes an "SMS supported" indication in Registration Request indicating the UE's capability for SMS over NAS transport.

2. If the “SMS supported” indication is included in the Registration Request, the AMF discovers and selects a SMSF to serve the UE. The AMF requests "Nudm_Register UE serving NF" service from UDM and stores SMSF’s address received from UDM in case the SMSF registered to the UDM for the UE before. If SMSF’s address does not exist in the UDM, the AMF invokes the NF discovery service (SMS NF type) from NRF to discover the SMSF address as described in TS 23.502 clause 5.2.7. For roaming scenario, AMF selects a SMSF in VPLMN.

3a.	AMF includes the "SMS supported" indication to the UE in the Registration Accept message if AMF has selected an SMSF; or

3b.	AMF includes the "SMS supported" indication to the UE in the Registration Accept message only after step 6 in which the AMF has received a positive indication from the selected SMSF.

4.	AMF invokes SMS activation service from the SMSF. The request includes AMF address and SUPI. AMF uses the SMSF address from step 2.

5.	SMSF responds back to AMF with an accept message. AMF stores the SMSF address received as part of the UE context.

5.	SMSF retrieves the SMS related subscription with update location procedure with the UDM via "Nudm_Register UE serving NF" service. SMSF also creates an UE context to store the SMS subscription information and the AMF address that is serving this UE.

6.	SMSF responds back to AMF with an accept message. AMF stores the SMSF address received as part of the UE context.

* * * * End of Changes * * * *