# SA WG2 Meeting #120	S2-172459

# 27 - 31 February 2017, Busan, Korea

Title:	DRAFT LS on Stage 2 completion on introduction of WLAN as alternative technology for ProSe Direct Discovery

Response to:

Release:	Rel-15

Work Item:	ProSe_WLAN_DD_Stage2

Source:	SA2

To:	CT1, CT4, SA5

Cc:

Contact person:	Sašo Stojanovski

(saso stojanovski intel com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-172458, S2-172460

# 1	Overall description

SA2 would like to inform that CT1, CT4 and SA5 that SA2 has completed the Stage 2 work on the work item on inclusion of WLAN as an alternative technology for ProSe Direct Discovery (ProSe_WLAN_DD_Stage2; SP-170057).

The Stage 2 functionality is introduced in TS 23.303 with two CRs: a WLAN-agnostic CR (S2-172460) and a Wi-Fi NAN (Neighbor Awareness Networking) specific CR (S2-172458).

SA2 would like to bring to the attention of CT1, CT4 and SA5 the following items on which work is required to complete the normative work on WLAN-based ProSe Direct Discovery:

To CT1:

-	Addition of a new parameter PC_tech in [PC3] Discovery Request message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) that the UE wishes to use for specific ProSe Application ID (open discovery) or specific Application ID (restricted discovery).

-	Addition of a new parameter PC_tech in [PC3] Discovery Response message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) that the UE is authorized to use for specific ProSe Application Code, ProSe Query Code, ProSe Response Code, Discovery Filter, Discovery Query Filter or Discovery Response Filter.

-	In all the messages listed previously, for Rel-15 the PC5_tech parameter should be able to indicate the E-UTRA and/or Wi-Fi NAN (Neighbor Awareness Networking) technologies, but should be encoded in extendable manner to allow for other PC5 radio technology alternatives to be added in future.

-	The [PC3] Match Report message needs to indicate when the match for a ProSe Application Code, ProSe Restricted Code or ProSe Response Code was done using WLAN-based ProSe Direct Discovery to enable charging in the network.

-	Provision of service authorization information that is specific to WLAN-based ProSe Direct Discovery as described in clause 4.5.1.1.1 (S2-172460).

To CT4:

-	Addition of a new parameter PC_tech in [PC6/PC7] Announce Authorisation message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) associated with a specific ProSe Application Code, ProSe Restricted Code or ProSe Response Code.

-	Addition of a new parameter PC_tech in [PC6/PC7] Monitor Request message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) associated with a specific ProSe Application ID Name provided in the message or to a valid ProSe Restricted Code stored in the ProSe Function.

-	Addition of a new parameter PC_tech in [PC6/PC7] Monitor Response message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) associated with a specific ProSe Application Code or ProSe Restricted Code.

-	Addition of a new parameter PC_tech in [PC6/PC7] Discovery Request message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) associated with a specific pair of ProSe Query Code and ProSe Response Code stored in the ProSe Function.

-	Addition of a new parameter PC_tech in [PC6/PC7] Discovery Response message indicating the PC5 radio technology (E-UTRA and/or specific WLAN technology) associated with a specific pair of ProSe Query Code and ProSe Response Code.

-	In all the messages listed previously, for Rel-15 the PC5_tech parameter should be able to indicate the E-UTRA and/or Wi-Fi NAN (Neighbor Awareness Networking) technologies, but should be encoded in extendable manner to allow for other PC5 radio technology alternatives to be added in future.

To SA5:

-	The [PC3] Match Report message needs to indicate when the match for a ProSe Application Code, ProSe Restricted Code or ProSe Response Code was done using WLAN-based ProSe Direct Discovery to enable charging in the network, as defined in the service requirement in TS 22.115 (CR0084).

# 2	Actions

To CT1, CT4, SA5:

ACTION: SA2 kindly asks CT1, CT4 and SA5 to take the information provided above into account during their work on WLAN-based ProSe Direct Discovery.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting #121	15-19 May 2017	Hangzhou, CN