SA WG2 Meeting #120	S2-172874

27 - 31 March 2017, Busan, South Korea	(e-mail revision 6 of S2-172843)

Source:	LG Electronics, Nokia, Alcatel-Lucent Shanghai Bell, CATT, HTC, Huawei, HiSilicon

Title:	23.502 §4.3.4: PDU session release procedure

Document for:	Approval

Agenda Item:	6.5.3 Session management and continuity functions and flows

Work Item / Release:	5G_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes PDU Session Release procedure.

# Discussion

# Proposal

Add the following to the TS 23.502.

* * * * Start of 1st Change (All New Text)* * * *

#### 4.3.4.1	General

The PDU session release procedure is used to release all the resources associated with a PDU session, including:

- The IP address/Prefixes allocated for an IP-based PDU session; this may include the release of multiple Prefixes in case of Multi-homing (as defined in TS 23.501 [2])

- Any UPF resource (including N3/N9 termination) that was used by the PDU session..

The SMF takes care to notify any entity associated with PDU session: PCF, DN (e.g. when DN authorization has taken place at PDU session establishment), etc. of a PDU session release.

#### 4.3.4.2	UE or network requested PDU session release for Non-Roaming and Roaming with Local Breakout

Figure 4.3.4.2-1 captures both the UE requested PDU session release procedure and the network requested PDU session release procedure. The procedure allows the UE to request the release of one PDU session. The procedure also allows the SMF or PCF to initiate the release of a PDU session. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use

Editor's note:	Alignment of message naming with the procedures for PDU session establishment is needed.

Editor's note:	The names of N11 Message are FFS.

Figure 4.3.4.2-1: UE or network requested PDU session release for non-roaming and roaming with local breakout

1.	The procedure is triggered by one of the following options:

1a.	The UE initiates the UE requested PDU session release procedure by the transmission of a SM PDU Session Release Request (PDU Session ID) message. This message is relayed to the SMF corresponding to the PDU Session ID via N2 and the AMF.

NOTE X:	Depending on the access type, when the UE is in CM-IDLE state, the UE can trigger a Service Request procedure before being able to release the PDU session.

1b.	The PCF may initiate the PDU-CAN Session modification procedure to request the release of the PDU session.

1c.	The SMF may decide to release a PDU session e.g. based on a request from the DN (cancelling the UE authorization to access to the DN) or based on a request from the UDM (subscription change) or from the OCS. The release procedure also may be triggered based on locally configured policy (e.g. the release procedure may be related with the UPF relocation for SSC mode 2 / mode 3).

If the SMF receives one of the triggers in step 1a ~ 1c, the SMF starts PDU session release procedure.

2.	The SMF releases the IP address / Prefix(es) that were allocated to the PDU session and releases the corresponding User Plane resources:

2a.	The SMF sends an N4 Session Release Request (N4 Session ID) message to the UPF. The UPF shall drop any remaining packets of the PDU session and release all tunnel resource and contexts associated with the N4 Session.

2b.	The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response (N4 Session ID) message to the SMF.

NOTE X:	If there are multiple UPFs associated with the PDU session, the Session Release Request procedure (steps 2a and 2b) is done for each UPF.

3.	If Dynamic PCC applied to this session the SMF initiates the PDU-CAN session termination procedure.

If it is the last PDU session it is handling for the UE the SMF releases the association with the UDM.

4.	The SMF sends an N11 Request message to the AMF (N2 SM Resource Release request, N1 SM Information (PDU Session Release Command))

The SMF creates an N1 SM Information including PDU Session Release Command message (PDU Session ID, Cause). The Cause may indicate a trigger to establish a new PDU session with the same characteristics (e.g. when procedures related with SSC mode 2 are invoked).

NOTE X:	SSC mode 2 is defined in TS 23.501[2] clause 5.6.9.

If the UP connection of the PDU session is active, the SMF shall also create an N2 SM request to release the (R)AN resources associated with the PDU session (N2 Resource Release request (PDU Session ID)).

The SMF sends an N11 message to the AMF (N2 SM Resource Release request, N1 SM Container (PDU Session Release Command),”N1 SM delivery can be skipped” Indication) message to the AMF. The “N1 SM delivery can be skipped” Indication tells the AMF whether it may skip sending the N1 SM container to the UE (e.g. when the UE is in CM-IDLE mode).

Editor’s Note: It is FFS whether this takes the form a Namf_Message Transfer (SMF): The SMF requests Namf_Message Transfer service of the AMF defined in Clause 5.2.2.4 to sends N1 SM and N2 SM messages to the UE and (R)AN.

NOTE X:	If the procedure is triggered to relocate PDU session anchor of a PDU session with SSC mode 2, the SMF is assumed not to include the “N1 SM delivery can be skipped” Indication.

Editor's note: It is FFS whether a Release Indication to explicitly delete the association of PDU Session ID and serving SMF ID within the AMF is added to the SMF request. This would allow skipping step 9 when the UE is in CM-IDLE state and ”N1 SM delivery can be skipped” is included in the N11 message.

Editor's note: It is FFS whether ”N1 SM delivery can be skipped” Indication is required.

5.	If the UE is in CM-IDLE state and ”N1 SM delivery can be skipped” Indication is included in the N11 message, the AMF acknowledges the step 4 and steps 5 to 7 are skipped.

NOTE X: 	The UE and the 5G Core will get synchronized about the status (released) of the PDU session at the next Service Request or Registration procedure.

Otherwise, if the UE is in CM-IDLE state and ”N1 SM delivery can be skipped” is not indicated, the AMF initiates the network triggered Service Request procedure to transmit the N1 SM Information to the UE..

If the UE is in CM-CONNECTED state, then the AMF transfers the message received from the SMF in step 4 (N2 SM Resource Release request, N1 SM Information) message to the (R)AN.

6.	When the (R)AN has received an N2 SM request to release the AN resources associated with the PDU session it issues AN specific signalling exchange(s) with the UE to release the corresponding AN resources.

In case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE releasing the RAN resources related to the PDU session.

During the this procedure, the (R)AN sends any NAS message (N1 SM PDU Session Release Command ) received from the AMF in step 5.

The UE acknowledges the PDU Session Release Command by sending a PDU Session Release Ack message via N1 SM signalling sent over the (R)AN.

7.	[Conditional] If the (R)AN had received a N2 SM request to release the AN resources, the (R)AN acknowledges the N2 SM Resource Release Request by sending an N2 SM Resource Release Ack (N1 SM Information (PDU Session Release Ack)) Message to the AMF.

Otherwise, the (R)AN just forwards the N1 SM Information (PDU Session Release Ack) from the UE to the AMF.

8.	The AMF sends N11 Response message (N1 SM Information (PDU Session Release Ack)) to the SMF.

9.	The SMF notifies the AMF that the PDU session is released. The AMF and SMF shall remove all contexts (including the PDU session ID) associated with the PDU session.

Editor’s Note: It is FFS whether this takes the form of a Nsmf_ PDU Session Event Report Notification (AMF): The SMF sends (N11 Message) PDU Session Event Report Notification as defined in Clause 5.2.8.1 to the AMF; the event trigger is PDU session release.

NOTE X:	The order with which SMF releases the resources is implementation dependant.

#### 4.3.4.3	UE or network requested PDU session release for Home-routed Roaming

This procedure is used in case of home-routed roaming scenarios.

Figure 4.3.4.3-1: UE or network requested PDU session release for home-routed roaming

1.	The procedure is triggered by one of the following options:

1a.	As in step 1a of clause 4.3.4.2 with the addition that the V-SMF sends a N Release PDU Session Request (Subscriber Permanent Identity, PDU Session ID, NAS message from the UE) message to the H-SMF. The V-SMF may request to release PDU session to the H-SMF without UE request.

1b.	This step is the same as step 1b in clause 4.3.4.2.

1c.	This step is the same as step 1c in clause 4.3.4.2.

If the SMF receives one of the triggers in step 1a ~ 1c, the H-SMF starts PDU session release procedure.

2-3.	These steps are the same as steps 2-3 in clause 4.3.4.2. The SMF is the SMF in HPLMN.

4.	The H-SMF sends Release PDU Session Command (Subscriber Permanent Identity, PDU Session ID, NAS message) message to the V-SMF.

5.	The V-SMF releases the corresponding User Plane resources. This includes the same procedure in step 2 but controlled from the SMF in VPLMN.

6-11.	These steps are the same as steps 4-9 in clause 4.3.4.2.

12.	The V-SMF sends Release PDU Session Confirm (Subscriber Permanent Identity, PDU Session ID) message to the H-SMF. The H-SMF shall remove all contexts associated with the PDU session.

* * * * End of Changes * * * *