SA WG2 Meeting #121	S2-173450

15-19 May, Hang Zhou, China	(was S2-170778)

Source:	Nokia; Alcatel-Lucent Shanghai Bell

Title:	On providing Allowed NSSAI to the RAN in Registration Procedure

Document for:	Approval

Agenda Item:	6.5.x

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: this paper updates the registration procedure so that the RAN becomes aware of the Accepted NSSAI so the RAN can know whether the UE is authorized or not to use the RAN part of the slice.

# Discussion

A UE may use a registration procedure to attach to the 5G Core Network, perform a TAU or change the registration status so the UE may e.g. change the Network Slices it is using.

In all these cases the UE is provided by the AMF an accepted NSSAI that identifies the Slices the UE can use in the Registration Area in the Registration accept message. It is assumed the slices are supported in the 5G RAN cells inside the registration area.

The 5G RAN may apply specific CP behaviour for a Network Slice the UE is authorized to use. However, the Only way the5G RAN can know about whether this Slice is authorized for the UE is if the 5G CN provides the S-NSSAI of the slices the UE is authorized to use to the N2 message carrying the Registration accept message. This mirrors some text already agreed in TS 23.501

# Proposal

It is proposed that the Registration Procedure includes providing the Accepted NSSAI to the RAN in N2 message carrying the Registration Accept Message. The following changes are proposed to clauses 4.2.2.2.2 and 4.2.2.2.3 of TS 23.502.

###First CHANGE###

#### 4.2.2.2	Registration

Editor's note:	Procedure equivalent to Attach procedure and TAU (for all type of triggers) procedure. Procedure includes aspects required to support network slicing, and policy control. Procedure may include e.g. authentication/authorisation, RRM, Capability handling, DRX aspects. Network sharing is considered as part of the procedure description.

##### 4.2.2.2.1	General

A UE needs to register with the network to get authorised to receive services, to enable mobility tracking and to enable reachability. The Registration procedure is used when the UE needs to perform initial registration to the 5G system, mobility registration update upon changing to a new Tracking area (TA) outside the UE's registration area in idle mode, when the UE performs a periodic registration update (due to a predefined time period of inactivity), and additionally when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure.

The General Registration call flow in clause 4.2.2.2.2 applies on all these registration procedures, but the periodic registration need not include all parameters that are used in other registration cases.

Editor's note:	The procedure description based on the NF services will be documented once the applicable components are identified.

Editor's note:	Further details due to network slicing is FFS.

Editor's note:	Aspects related to dual registration in 3GPP and non-3GPP access is FFS.

Editor's note:	The identities needs to be aligned with the agreed identities.

Editor's note:	Enhancements of the normative wording (i.e. adding shall/should/may) is FFS.

During the initial registration the Permanent Equipment Identifier is obtained from the UE. The AMF operator may check the PEI with an EIR. The AMF passes the PEI (IMEISV) to the UDM, to the SMF and the PCF.

##### 4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, Subscriber Permanent Identifier or Temporary User ID, Security parameters, NSSAI, UE 5GCN Capability, PDU session status and MICO mode preference)).

Editor's note:	The required AN parameters is FFS.

In case of 5G-RAN, the AN parameters include e.g. SUPI or the Temporary User ID, the Selected Network and NSSAI.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in non-registered state), a "mobility registration update " (i.e. the UE is in registered state and initiates a Registration procedure due to mobility) or a "periodic registration update" (i.e. the UE is in registered state and initiates a Registration procedure due to the periodic update timer expiry, see clause 4.2.2.2.1). If included, the Temporary User ID indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the PLMN of the 3GPP access, the UE shall not provide the UE Temporary ID allocated by the AMF during the registration procedure over the non-3GPP access.

The Security parameters are used for Authentication and integrity protection. NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]. The PDU session status indicates the available (previously established) PDU sessions in the UE.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	The possibility to piggyback a PDU session Establishment request at the same time as sending a Registration request is FFS.

Editor's note:	Changes required to support "stickiness" is FFS.

Editor's note:	It is FFS if Active flag/bitmap and HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the Temporary User ID does not indicate a valid AMF the (R)AN, based on (R)AT and NSSAI, if available, selects an AMF.

The (R)AN selects an AMF as described in TS 23.501 [2], clause X.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF selected according to local policies. If the selected AMF cannot serve the UE, the selected AMF selects an appropriate AMF for the UE. The relocation between the default AMF and selected AMF is described in clause 4.2.2.2.3, where the initial AMF refers to the default AMF, and the target AMF refers to the selected AMF.

3.	(R)AN to new AMF: N2 message (N2 parameters, Registration Request (Registration type, Subscriber Permanent Identifier or Temporary User ID, Security parameters, NSSAI and MICO mode preference)).

When 5G-RAN is used, the N2 parameters include the Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

If the Registration type indicated by the UE is periodic registration update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Information Request (complete Registration Request).

If the UE's Temporary User ID was included in the Registration Request and the serving AMF has changed since last registration, the new AMF may send Information Request to old AMF including the complete Registration Request IE to request the UE's SUPI and MM Context.

5.	[Conditional] old AMF to new AMF: Information Response (SUPI, MM Context, SMF information).

Old AMF responds with Information Response to new AMF including the UE's SUPI and MM Context.

If old AMF holds information about active PDU Sessions, the old AMF includes SMF information including SMF identities and PDU session identities.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to invoke an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause X.

9.	The AUSF shall initiate authentication of the UE and NAS security functions.

The authentication and security are performed as described in clause X.

Editor's note:	It is FFS how UDM is selected in this step.

Editor's note:	It is FFS whether the AUSF or the AMF initiates the authentication.

10.	[Conditional] new AMF to old AMF: Information Acknowledged ().

If the AMF has changed the new AMF acknowledge the transfer of UE MM context.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF sends a reject indication to the old AMF. The old AMF continues as if the Information Request was never received.

11.	[Conditional] new AMF to UE: Identity Request ().

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check.

The PEI check is performed as described in clause 4.7.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects an UDM as described in TS 23.501 [2], clause X.

14.	If the AMF has changed since the last registration, or if there is no valid subscription context for the UE in the AMF, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, the new AMF initiates the Update Location procedure. This will include that UDM initiates Cancel Location to old AMF, if any. The old AMF removes the MM context and notifies all possibly associated SMF(s), and the new AMF creates an MM context for the UE after getting the AMF related subscription data from the UDM.

The Update Location procedure is performed as described in clause X.

If network slicing is used, the AMF obtains the Allowed NSSAI based on the Requested NSSAI, UE subscription, and local policies. If the AMF is not suitable for supporting the Allowed NSSAI, the Registration Request is rerouted described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

Editor's note:	It is FFS how to update the figure with the "UDM initiates Cancel Location to old AMF".

15.	Conditionally the new AMF, based on the SUPI, selects a PCF.

The AMF selects a PCF as described in TS 23.501 [2], clause X.

Editor's note:	It is FFS whether the Permanent User ID is used for PCF selection.

Editor's note:	The conditions when the AMF selects a PCF is FFS.

16.	[Optional] new AMF to PCF: UE Context Establishment Request ().

The AMF request the PCF to apply operator policies for the UE.

17.	PCF to new AMF: UE Context Establishment Acknowledged ().

The PCF acknowledges the UE Context Establishment Request message.

18.	[Conditional] new AMF to SMF: N11 Request ().

If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE.

The AMF verifies PDU session status from the UE with the available SMF information. In case the AMF has changed the available SMF information has been received from the old AMF. The new AMF requests the SMF to release any network resources related to PDU sessions that are not active in the UE.

Editor's note:	It is FFS whether AMF perform the PDU session status logic or each SMF does it.

19.	SMF to new AMF: N11 Response ().

The SMF may decide to trigger e.g. UPF relocation as described in clause x.

If the Registration type indicated by the UE is periodic registration update, then steps 20 and 21 may be omitted.

Editor's note:	It is FFS how SMFs subscribes to AMF services.

Editor's note:	SMF relocation in relation to a Registration procedure is FFS.

20.	[Conditional] old AMF to PCF: UE Context Termination Request ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF.

21.	PCF to old AMF: UE Context Termination Acknowledged ().

22.	New AMF to UE: Registration Accept (Temporary User ID, Registration area. Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode).

The AMF includes in the N2 message the Allowed NSSAI to inform the 5G RAN of the S-NSSAIs of the Network slices the UE is authorized to use.

The AMF sends a Registration Accept message to the UE indicating that the registration has been accepted. Temporary User ID is included if the AMF allocates a new Temporary User ID. Mobility restrictions is included in case mobility restrictions applies for the UE. The AMF indicates the PDU session status to the UE. The UE removes any internal resources related to PDU sessions that are not marked active in the received PDU session status. If the PDU session status information was in the Registration Request, the AMF shall indicate the PDU session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

23.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new Temporary User ID was assigned.

Editor's note:	Details of N2 interaction is FFS.

##### 4.2.2.2.3	Registration with AMF relocation

When an AMF receives a Registration request, the AMF may need to reroute the Registration request to another AMF, e.g. due to network slicing is used and the initial AMF is not the appropriate AMF to serve the UE. The Registration with AMF relocation procedure, described in figure 4.2.2.2.3-1, is used to reroute the NAS message of the UE to the target AMF during a registration procedure.

Figure 4.2.2.2.3-1: NAS message redirection procedure

The initial AMF and the target AMF register its capability at the NRF.

1.	Step 1 and 2 of figure 4.2.2.2.2-1 has occurred, and the (R)AN sends the Registration request message within an Initial UE message to the initial AMF.

2.	If the AMF needs the SUPI and/or UE's subscription information to decide whether to reroute the Registration request or if the Registration request was not sent integrity protected or integrity protection is indicated as failed, then AMF performs step 4 to 14 of figure 4.2.2.2.2-1.

3.	[Conditional] AMF to NRF: NF discovery request (NF type, NSSAI).

The initial AMF decides to reroute the NAS message to another AMF. If the initial AMF does not locally store the target AMF address, the initial AMF sends an NF discovery request to the NRF to find a proper target AMF which has required NF capabilities to serve the UE. The NF type is set to AMF. If network slicing is used, NSSAI is included in the NF discovery request.

4.	NRF to AMF: NF discovery response (list of AMFs, NF capabilities).

The NRF replies with a set of potential target AMFs and their capabilities. Based on the information about registered NFs and required capabilities, a target AMF is selected by the initial AMF.

5.	If the initial AMF, based on local policy and subscription information, determines to forward the NAS message to the target AMF directly, the initial AMF sends a Reroute NAS message to the target AMF. The Reroute NAS message includes the information enabling (R)AN to identify the N2 terminating point and the NAS message carried at step 1, and optionally the UE's SUPI and MM Context. If network slicing is used and the initial AMF determines  the Allowed NSSAI as described at step 14 of figure 4.2.2.2.2-1, the Allowed NSSAI is included in the NAS signalling message and also in the N2 signalling message in step 5a to update the RAN on the new Allowed NSSAI. The target AMF then updates the (R)AN with a new updated N2 termination point for the UE (step 5b), the (R)AN acknowledge the updated N2 termination point (step 5c). Step 5 is skipped.

NOTE:	Steps 5b and 5c can occur separately or as part of the first subsequently required N2 interaction.

6.	If the initial AMF, based on local policy and subscription information, determines to forward the NAS message to the target AMF via RAN, the initial AMF sends a Reroute NAS message to the RAN (6a). The Reroute NAS message includes the information about the target AMF and the Registration request  message carried at step 1, and optionally the UE's SUPI and MM Context. If network slicing is used and the initial AMF determines  the Allowed NSSAI as described at step 14 of figure 4.2.2.2.2-1, the Allowed NSSAI is included in the Reroute NAS message and in the N2 signalling message used to carry it. The RAN sends the Initial UE message to the target AMF (6b).

7.	After receiving the Registration request message transmitted at step 5a or step 6b, the target AMF continues with the Registration procedure from step 6 of figure 4.2.2.2.2-1 (with the target AMF corresponding to the new AMF).

Editor's note:	The case when the target AMF cannot be determined and provided to the initial AMF, and the default AMF needs to be selected in step 3 is FFS.

Editor's note:	Whether both option (A) and option (B) are acceptable from a RAN perspective needs to be evaluated by RAN WGs.

###End of CHANGES###