SA WG2 Meeting #121	S2-173635

27-31 March 2017, Busan, Korea	rev based on 3537

Source:	Ericsson

Title:	TS 23.502: UDM services definition

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes UDM services and service operations

# Introduction

Based on service definition criteria included in S2-173537, this document proposes definition of UDM services.

# Discussion

Service definition criteria described in S2-173537 is relevant to achieve a definition of services that fulfil SBA expectations on independent LCM (developing/updating, upgrading, scaling…).

This document defines UDM services following this criteria, i.e. a service is defined for each different and independent functionality/capability/business logic, as follows:

- Authentication Management

- Access and Mobility Management

- Session Management

- SMS Management

In this way, it is possible to perform an individual and independent service LCM. For example:

- Scaling: “Access and Mobility Mng” service may require double traffic (amount of service requests) (for AMF or any future NF that requires Access and Mobility) than for “Session Mng” service (for SMF or any future NF that requires Session Mng), then different scaling patterns are possible to be applied.

- Developing (Updating): some changes may be required for “SMS Mng” business logic, then the SW piece (Service) will require to be updated. Even though required changes may be limited to one/some micro-services (in Cloud Platform), it implies that the SW piece needs updating, re-compilation, basic testing… all development SW tasks are required for the whole Service. Therefore, it is advisable to do not mix different functionalities in the same service, in order to allow independent updating of each functionality.

- Upgrading: a SW piece (Service) (or some of its micro-services) may need upgrading in a real and up and running Cloud Platform. It is recommended to avoid impacting consumers that are not really benefited by upgraded functionality. This could be minimized if each service only provides a specific functionality. There are techniques like rolling upgrade that could be used to minimize consumers’ service interruption while upgrading, but the usage could be optimized if each service clearly provides a limited and specific functionality.

- Deployment: it may be required to just deploy e.g. “SMS Mng”, but unless this functionality is defined in an independent service, independent development will not be possible.

Therefore, the only way to achieve an independent developing/updating, upgrading, scaling, deployment... (in general independent LCM), is to define a different service for each specific and independent business logic.

# Proposal

UDM Services definition in clause 5.2.3 is updated.

* * * * Start Change * * *.*

### 5.2.3	UDM Services

Editor's note:	The identified NF service(s) in this clause are to be evaluated. 5.2.3.1 "Register UE serving NF" service.

Editor's note:	How to define a service based procedure for "Purge of subscriber data from AMF" is to be determined.

#### 5.2.3.1	General

Table 5.2.3-1: NF Services provided by UDM

| NF service | Description | NF service operations | Consumer |  |  | Ref. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication Management | UDM provides Authentication information to AUSF used for authentication of user access to the 5GS. For AKA based user authentication, this includes an AKA authentication Vector. This service also includes recovery of authentication synchronization failures. | Nudm_Authentication_Information_Request | AUSF |  |  |  |  |
| Access and Mobility Management | UDM authorizes UE Access to 5GS based on access subscription profile. UDM supports UE mobility procedures within 5GS including keeping registration of serving AMF at any time and notifying old serving nodes (AMF or MMEs in case of mobility from EPC) that its registration in UDM is no longer valid when required (e.g. mobility events, subscription administration reasons). UDM delegates authorization of further access changes and mobility events to requesting NF by providing updated access and mobility subscription profile information to AMF. Additionally, AMF registers UE context information in UDM. | Nudm_Access&Mobility_Mgmt_Registration | AMF |  |
|  |  | Nudm_Access&Mobility_Mgmt_Deregistration |  |  |
|  |  | Nudm_Access&Mobility_Mgmt_Cancel_Registration_Notification |  |  |
|  |  | Nudm_Access&Mobility_Mgmt_Profile_Update_Notification |  |  |
|  |  | Nudm_Access&Mobility_Mgmt_UE_Context_Update |  |  |
| Session Management | UDM provides updated session management subscription profile information of the session managed by the requesting SMF (i.e. DNN characteristics of subscribed DNNs) during the time the requesting SMF is managing sessions for the UE. | Nudm_Session_Mgmt_Registration |  | SMF |  |  |  |
|  |  | Nudm_Session_Mgmt_Deregistration |  |  |  |  |  |
|  |  | Nudm_Session_Mgmt_Profile_Update_Notification |  |  |  |  |  |
| SMS Management | UDM provides updated SMS management subscription profile information to requesting SMSF during the time the requesting SMSF is managing SMS for the UE. UDM keeps registration of the SMSF serving each UE at any time and notifies old SMSF (or MME in case of mobility from EPC) that its registration in UDM for the purpose of providing SMS services to the UE is no longer valid. | Nudm_SMS_Mgmt_Registration |  | SMSF |  |  |  |
|  |  | Nudm_SMS_Mgmt_Deregistration |  |  |  |  |  |
|  |  | Nudm_SMS_Mgmt_Cancel_Registration_Notification |  |  |  |  |  |
|  |  | Nudm_SMS_Mgmt_Profile_Update_Notification |  |  |  |  |  |

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

Next change

#### 5.2.3.x	Authentication Management Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_Authentication Information_Request, | For AKA based user authentication, UDM provides an AKA authentication Vector for authenticating UE access to 5GS. | SUPI, Requester NF ID Optionally, RAND (used in a previous unsuccessful authentication attempt) in case of synchronization failure recovery, | AKA AV |

Editor's note:	The list of input and output parameters may have to be completed.

#### 5.2.3.x+1	Access and Mobility Management Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_Access&Mobility_Mgmt_Registration | UDM authorizes the requester AMF to serve the UE, registers requester AMF as UE's serving AMF in UDM and provides the access and mobility authorization profile information to the requester AMF. The requesting AMF is registered in UDM for subsequent notification of changes in the access and mobility profile information as well as for notification of UDM initiated AMF deregistration (due to e.g. mobility events, subscription administration reasons). Additionally, AMF registers in UDM UE context information. | SUPI, Requester AMF ID, Access information (e.g. Access/RAT Type). Optionally, UE context information (e.g. PEI, UE capabilities). | Result indication Access and Mobility profile information |
| Nudm_Access&Mobility_Mgmt_Deregistration | UDM Deregister UE's serving AMF. | SUPI, Requester AMF ID | Result Indication. |
| Nudm_Access&Mobility_Mgmt_Cancel_Registration_Notification | UDM notifies the Requester AMF, which has previously registered in UDM, that its registration in UDM is no longer valid. |  | SUPI, Serving AMF ID Deregistration reason. |
| Nudm_Access&Mobility_Mgmt_Profile_Update_Notification | UDM updates access and mobility profile information in serving AMF registered in UDM. |  | Updated Access and Mobility profile information |
| Nudm_Access&Mobility_Mgmt_UE_Context_Update | UDM stores additional and/or updated UE Context information provided by the UE serving AMF. | SUPI, Requester AMF ID UE Context Information | Result Indication. |

Editor's note:	The list of input and output parameters may have to be completed.

#### 5.2.3.x+2	Session Management Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_Session_Mgmt_Registration | UDM provides the session profile information corresponding to the DNN to which the UE is establishing a PDU session to via the requester SMF. The requesting SMF is registered in UDM for subsequent notification of changes in the session profile information of the requested DNN. | SUPI, Requester SMF ID, Requested DNN | Result indication Session (DNN) profile information |
| Nudm_Session_Mgmt_Deregistration | UDM Deregister UE's serving SMF. | SUPI, Requester SMF ID | Result Indication. |
| Nudm_Session_Mgmt_Profile_Update_Notification | UDM updates session (DNN) profile information in serving SMF(s) registered in UDM. Mind that there may be multiple SMFs with active PDU sessions to the same DNN for a given user. |  | Updated Session (DNN) profile information |

Editor's note:	The list of input and output parameters may have to be completed.

#### 5.2.3.x+3	SMS Management Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_SMS_Mgmt_Registration | UDM provides SMS management subscription profile information to the requester SMSF. The requesting SMSF is registered in UDM for subsequent notification of changes in the SMS profile information as well as for notification of UDM initiated SMSF deregistration (due to e.g. registration of another SMSF, subscription administration reasons). | SUPI, Requester SMSF ID | Result indication SMS profile information |
| Nudm_SMS_Mgmt_Deregistration | UDM Deregister UE's serving SMSF. | SUPI, Requester SMSF ID | Result Indication. |
| Nudm_SMS_Mgmt_Cancel_Registration_Notification | UDM notifies the Requester SMSF, which has previously registered in UDM, that its registration in UDM is no longer valid. |  | SUPI, Serving SMSF ID Deregistration reason. |
| Nudm_SMS_Mgmt_Profile_Update_Notification | UDM updates SMS profile information in serving SMSF registered in UDM. |  | Updated SMS profile information |

Editor's note:	The list of input and output parameters may have to be completed.

End changes